TSG-RAN Working Group 4 (Radio) meeting #104-bis-E	R4-2215833

Electronic Meeting, 10th – 19th October 2022

Source:	Ericsson

Title:	On further aspects related to FR2-2 transmitter conformance testing

Agenda item:	4.3.4.2

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#104-E) aspects related to conformance testing within FR2-2 was discussed extensively. The outcome, including open issues, aspects to further study and guidance for this meeting is captured in two way-forward contributions [1, 2].

In this contribution some additional details related to transmitter conformance testing is presented.

# Discussion

In current specification we can note that the maximum system uncertainty used for OTA output power dynamics test requirement derivation for FR1, and FR2-1 are equal, as shown in Table 2-1.

Table 2-1: OTA total power dynamic range

| Frequency range | Maximum OTA Test System uncertainty (dB) | Test Tolerance (dB) |
| --- | --- | --- |
| FR1 | ±0.4 | 0.4 |
| FR2-1 | ±0.4 | 0.4 |

From a technical and practical perspective and in relation to current specification it would be reasonable to assume the same value for FR2-2.

Proposal 1: For FR2-2 set OTA TT for OTA total power dynamic range to +/-0.4 dB.

According to TS 38.141-2, Annex C.1-2 the test requirement level for OTA output power dynamics is determined as:

Test requirement level = Total power dynamic range - TT

For FR2-2 is would be reasonable to assume the same value for TT.

Based on RF core requirement definition if TS 38.104, the RF conformance test requirement levels for FR2-2 have been calculated for supported SCS and carrier bandwidths in Table 2-2.

Table 2-2: Minimum requirement for total power dynamic range in FR2-2

| SCS | OTA total power dynamic range (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| 120 | 17.7 | 23.8 | N/A | N/A | N/A |
| 480 | N/A | 17.7 | 20.8 | 23.8 | N/A |
| 960 | N/A | 14.7 | 17.7 | 20.8 | 21.5 |

Proposal 2: For FR2-2 define minimum requirement for total power dynamic range as described in Table 2-2.

# Conclusion

In this contribution we present some aspects related to the derivation of test requirements for OTA total dynamic range. It is reasonable to follow the same approach as used for FR1 and FR2-1.

Based on the information in this contribution we propose following:

Proposal 1: For FR2-2 set OTA TT for OTA total power dynamic range to +/-0.4 dB.

Proposal 2: For FR2-2 define minimum requirement for total power dynamic range as described in Table 2-2.

Based on above proposals specification text for FR2-2 have been created in a draft CR to TS 38.141-2, clause 6 in a companion contribution [4] submitted to this meeting.

# References

[1] 	R4-2214374, “WF on FR2-2 BS conformance testing”, Nokia

[2]	R4-2214373, “WF on FR2-2 BS test environments”, Ericsson

[3]	R4-2212465, “On general aspects relevant for conformance testing up to 71 GHz”, Ericsson

[4]	R4-2215828, “Draft CR to TS 38.141-2: Addition of FR2-2 transmitter support in clause 6”, Ericsson