TSG-RAN Working Group 4 (Radio) meeting #104-bis-E	R4-2215835

Electronic Meeting, 10th – 19th October 2022

Source:	Ericsson

Title:	On initial results for SBFD adjacent channel co-existence evaluation

Agenda item:	6.17.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#104-E) coexistence simulation assumptions including models and parameters values relevant for Sub-Band Full Duplex (SBFD) adjacent channel coexistence evaluation was discussed extensively. The outcome from the discussion is captured in a way-forward contribution in [1]. It can be noticed that multiple options exist for traffic model, UE distribution and network grids. Also, some aspects related to the simulation assumptions is not settled.

In this contribution we present an overview of simulation assumptions together with initial simulation results related to SFBD adjacent channel coexistence between two networks.

# Discussion

In this study RAN4 is tasked to evaluate adjacent channel coexistence between legacy static TDD (STDD) network and an SBFD network. The planned coexistence evaluation considers the impact on a victim static STDD network when a SBFD network is deployed in the same geographical area and vice versa.

The SBFD duplex aggressor network is configured at adjacent channel with respect to the STDD victim network. As guidance for the co-existence simulation campaign a way-forward with modelling aspects and parameter settings was created in [1]. Even though many options for parameter settings are listed and some values not determined, initial simulation results are presented to stimulate further discussion on some general aspects at this meeting.

  - Simulation assumptions

In the way-forward contribution from last meeting simulation assumptions, options and open issues are listed. In Table 2.1-1 we have captured input to resolve open issues and reduce number of options giving a complete set of simulation assumptions.

Table 2.1-1: Simulation assumptions

| Modelling aspects | Description |
| --- | --- |
| Pathloss model | Use UMa LOS model described in Section 2.5. |
| Network configuration | Use 0%, 10%, 100% network grid shift as described in Section 2.6. |
| Indoor/Outdoor UE ratio | Use input from TR 38.828 together with following assumptions: For FR1 Macro-to-Macro uses 80% indoor and 20% outdoor. For FR2-1 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor. |
| UE distribution | Drop UE uniformly and drop UE in clusters as described in Section 2.7. |
| BS antenna model | Use the extended array antenna model in TR 38.803, subclause 5.2.3.2.4 with additions for SBFD described in Section 2.3, Table 2.3-1. |
| BS antenna parameters | For FR1 use parameters in TR 38.803, Table 5.2.3.2.4-3 with additions for SBFD described in Section 2.4, Table 2.4-1. For FR2-1 use parameters relevant for FR2-1 AAS BS presented to RAN4 and ITU-R [6, 7] with additions for SBFD in Section 2.4, Table 2.4-2. |
| BS power capability | For total conducted power see Section 2.4, Table 2.4-3. |
| BS antenna mechanical down-tilt angles | For FR1 see Section 2.4, Table 2.4-1. For FR2-1 see Section 2.4, Table 2.4-2. |
| Uplink power control model | For downlink scenario, no power control scheme is applied. For uplink scenario, TPC model specified in TR 36.942, sub-clause 9.1 is applied with following parameters. CLx-ile=-SNRtarget+Pmax-kT-FBS-10log10(BW) γ=1 SNRtarget=15 dB for FR1 and FR2 |

Proposal 1: We propose to capture simulation assumptions listed in Table 2.1-1 used for the co-existence evaluation in the SBFD technical report TR 38.858 and clearly indicate extensions to previously used models.

  - SBFD BS antenna geometry

To make SBFD to work, isolation between transmitter and receiver is essential. Splitting the antenna in a transmitter antenna and a receiver antenna separated by an isolator can improve isolation. The array configuration in terms of array geometry can also be selected to improve isolation, e.g., by considering narrow element patterns using vertical sub-array structure, TX-to-RX isolation can be significantly increased. Since sub-arrays also is used for base station antennas to optimize coverage it would be natural to assume vertical sub-arrays for SBFD BS.

In Figure 2.2-1, an antenna geometry discussed for SBFD is visualized. The transmitter and receiver antenna are separated in the vertical domain. The isolation in between is improved by an isolator placed between the transmitter and receiver antennas. Two main cases have been discussed:

- Constant gain, which would relate to a SBFD capable base station keeping the antenna gain for transmitter and receiver equal to the traditional TDD base station. The physical size will be more than twice as large as the reference TDD system, due to the antenna separation to achieve isolation between transmitter and receiver.

- Reduced gain, which would relate to a SBFD capable base station where the physical antenna area is divided in half to account for isolation between transmitter and receiver. This would lead to 3 dB less antenna gain compared to the TDD reference system and 3 dB less power capability.

In the figure, parameters required to model the antenna array for SBFD are also visualised.

# Figure 2.2-1: SBFD BS antenna configuration

In conjunction with antenna separation in the vertical domain, the isolation can be improved by using vertical sub-arrays. Each individual vertical sub-array will have a narrow radiation pattern which will improve the overall TX-to-RX isolation.

Observation 1: Scaling down the antenna area will impact both the antenna gain and power capability.

  - Extended array antenna model for far-field

Originally a single element array antenna model was developed in TR 37.840 for AAS BS. This model has been used for many simulation campaigns in RAN1 and RAN4. However, recently this model was questioned in ITU-R WP 5D since the model does not reflect real base station implementations. The technical background for the sub-array model extension is captured in [2]. An LS [3] was prepared in RAN4 to establish a common view on how base stations operating within FR1 and deployed in networks are implemented. Evaluating base stations deployed in networks, it is clear that base stations are using array antennas with sub-array structures to optimize performance. The reason for having sub-arrays is to optimize the base station implementation where UEs typically are spread in the horizontal domain, rather than in the vertical domain for most scenarios. Hence beamforming resources are focused on the horizontal domain. To be able to model vertical sub-array structures properly an extension to the original array antenna model was proposed. The array antenna model extension is described in TR 38.803, subclause 5.2.3.2.4.

The model supports single column vertical sub-array structures. The extended antenna model is capable of modelling array antennas with sub-array structures for FR1 as well as for FR2 using appropriate parameters. In addition to the single element model defined originally in TR 37.840 an intermediate stage is added to model the sub-array pattern. The sub-arrays pattern is then used instead of the element patten. The extended antenna model is described by the equations given in Table 2.3-1.

Table 2.3-1: Extended model supporting vertical sub-arrays

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

The sub-array stage models vertical sub-arrays from the original element pattern. The sub-array pattern is used together with the array factor to produce the composite radiation pattern.

For the purpose of SBFD an additional parameter is introduced for the vertical edge-to-edge separation between transmitter and receiver antenna. All parameters required by the model and relations between parameters is described in Table 2.3-2.

Table 2.3-2: Extended parameter definitions

| Category | Parameter | Symbol | Unit | Note |
| --- | --- | --- | --- | --- |
| Element | Horizontal half power beam width | 3dB | Degrees | Beam widths relates to available element area |
|  | Vertical half power beam width | 3dB | Degrees |  |
|  | Front-to-back ratio | Am | dB |  |
|  | Side lobe suppression | SLAv | dB |  |
|  | Element gain | GE,max | dBi | The model gain is applied per element; hence the antenna loss (L) needs to be considered |
| Sub-array | Number of element rows in sub-array | Msub | Integer |  |
|  | Element vertical separation within sub-array | dv,sub | m | Depends on dv, M and Msub |
|  | Sub-array pre-set down-tilt | subtilt | Degrees |  |
| Array | Number of columns | N | Integer |  |
|  | Number of rows | M | Integer |  |
|  | Horizontal separation | dh | m |  |
|  | Vertical separation | dv | m |  |
|  | Electrical scan angle | escan | Degrees | Parameters to set beam direction |
|  | Electrical down-tilt angle | etilt | Degrees |  |
| Antenna | TX/RX panel vertical separation | dant | m | Antenna edge-to-edge separation |
|  | Conducted power fed to each branch | Ptx | dBm |  |
|  | Number of supported polarizations | P | Integer | +45/-45 degrees or Vertical/Horizonal |
|  | Antenna loss | L | dB | Includes all losses in the antenna |
| Deployment | Mechanical down-tilt | mech | Degrees | Depends on mast height and ISD |

Observe that if Msub is set equal to 1 and dv,sub equal to dv and qsubtilt equal to 0 degrees, the extended array antenna model collapses into the original antenna model.

Observation 2: Antenna parameters more up to date for base station implantations deployed in commercial networks have been provided to ITU-R. These parameters support sub-array structures within the frequency range 1710 to 4990 MHz.

  - Antenna parameter values for SBFD

Defining parameters for a SBFD capable base station following aspects need careful considerations:

- For a SBFD base station high isolation between transmitter and receiver is essential for minimizing the self-interference, and so reducing the impact on uplink performance. A solution previously discussed is to divide the antenna in two panels separated by an isolator. By separating the antennas in the vertical domain allows to shape the element pattens in the vertical domain to maximize the composite TX-to-RX antenna isolation. The element pattern can be shaped by using vertical sub-arrays.

- The array antenna models the gain in the far-field region, while the TX-to-RX isolation is a near-field effect. This means that to be able to draw relevant and correct conclusions the same antenna configuration needs to be considered both for far-field aspects (e.g., antenna gain model required by performance and co-existence modelling) and near-field aspects (e.g., modelling required to evaluate relevant and feasible TX-to-RX isolation characteristics).

- At previous meeting information related to antenna structures using sub-arrays to improve TX-to-RX isolation have been presented. In [5] it can be seen that TX-to-RX isolation can be improved using sub-arrays.

- RAN4 has provided ITU-R WP 5D with additional information related to modelling of sub-array structures relevant for base stations operating within the frequency range 1710 to 4990 MHz. With the parameters base stations operating in deployed networks in many regions can be modelled.

RAN4 has communicated relevant sub-array parameters, for the frequency range 1710 to 4990 MHz, to ITU-R and CEPT. These parameters can be used for urban macro and dense urban macro for the proxy frequency of 4000 MHz assumed for SBFD co-existence simulations. Other deployment scenarios rely on previous parameter assumptions based on single element structures.

Observation 3: RAN4 has agreed on sub-array parameters relevant for the frequency range 1710 to 4990 MHz. Instead of defining a new set of parameters unique for SBFD, already defined parameters will fit the purpose to model sub-arrays for SBFD base stations for 4 GHz proxy frequency.

Considering deployment scenarios considered for SBFD, parameter sets have been created for urban macro, dense urban macro, micro and indoor. In Table 2.4-1 parameter sets for urban macro, dense urban macro, dense urban micro and indoor in presented for SBFD simulations for FR1.

Table 2.4-1: Antenna configuration relevant for FR1 (4000 MHz)

| Parameter | Deployment scenario |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urban macro “Constant gain” | Dense urban macro “Reduced gain” | Dense urban micro | Indoor |
| Horizontal half power beam width | 90 o | 90 o | 90 o | 90 o |
| Vertical half power beam width | 65 o | 65 o | 65 o | 90 o |
| Front-to-back ratio | 30 dB | 30 dB | 30 dB | 30 dB |
| Side lobe suppression | 30 dB | 30 dB | 30 dB | 30 dB |
| Element gain | 6.4 dBi | 6.4 dBi | 6.4 dBi | 5.5 dBi |
| Number of element rows in sub-array | 3 | 3 | 3 | 1 |
| Element vertical separation within sub-array | 0.7l mm | 0.7l mm | 0.7l mm | 0.5l mm |
| Sub-array pre-set down-tilt | 3.0 o | 3.0 o | 3.0 o | 0.0 o |
| Number of columns | 8 | 8 | 8 | 4 |
| Number of rows | 4 | 2 | 2 | 4 |
| Horizontal separation | 0.5l mm | 0.5l mm | 0.5l mm | 0.5l mm |
| Vertical separation | 2.1l mm | 2.1l mm | 2.1l mm | 0.5l mm |
| Array loss | 2.0 dB | 2.0 dB | 2.0 dB | 2.0 dB |
| Conducted power fed to each branch | 35.0 dBm | 35.0 dBm | 32 dBm | 24 dBm |
| Number of supported polarizations | 2 | 2 | 2 | 2 |
| TX/RX antenna vertical separation | 4.0l mm | 4.0l mm | 4.0l mm | 2.0l mm |
| Mechanical down-tilt | 6.0 o | 6.0 o | 0.0 o | 90.0 o |

For FR2-1 RAN4 currently assumes single element structures in TR 38.803. However, sub-array antenna geometries are used for many of the deployed base stations in networks in several regions. At last, RAN4 meeting relevant sub-array parameters for FR2-1 AAS BS was presented in [6]. The same information has also been provided to ITU-R WP 5D in [7].

For SBFD the “constant gain” approach is used for urban macro, since the physical size of an antenna twice as large fits the base station enclosure due to shorter wavelength. For dense urban macro and dense urban micro, the panel is divided as “reduced gain” approach suggests. For indoor the antenna is further scaled down. The parameter sets relevant for FR2-1 is described in Table 2.4-2.

Table 2.4-2: Antenna configuration relevant for FR2-1 (30000 MHz)

| Parameter | Deployment scenario |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urban macro | Dense urban macro | Dense urban micro | Indoor |
| Horizontal half power beam width | 90 o | 90 o | 90 o | 90 o |
| Vertical half power beam width | 90 o | 90 o | 90 o | 90 o |
| Front-to-back ratio | 30 dB | 30 dB | 30 dB | 30 dB |
| Side lobe suppression | 30 dB | 30 dB | 30 dB | 30 dB |
| Element gain | 5.5 dBi | 5.5 dBi | 5.5 dBi | 5.5 dBi |
| Number of element rows in sub-array | 2 | 2 | 2 | 2 |
| Element vertical separation within sub-array | 0.6l mm | 0.6l mm | 0.6l mm | 0.6l mm |
| Sub-array pre-set down-tilt | 0.0 o | 0.0 o | 0.0 o | 0.0 o |
| Number of columns | 24 | 24 | 24 | 8 |
| Number of rows | 8 | 4 | 4 | 4 |
| Horizontal separation | 0.5l mm | 0.5l mm | 0.5l mm | 0.5l mm |
| Vertical separation | 1.2l mm | 1.2l mm | 1.2l mm | 1.2l mm |
| Array loss | 2.0 dB | 2.0 dB | 2.0 dB | 2.0 dB |
| Conducted power fed to each branch | 7.0 dBm | 7.0 dBm | 7.0 dBm | 7.0 dBm |
| Number of supported polarizations | 2 | 2 | 2 | 2 |
| TX/RX antenna vertical separation | 4.0l mm | 4.0l mm | 4.0l mm | 100 mm |
| Mechanical down-tilt | 15.0 o | 10.0 o | 0.0 o | 90.0 o |

Based on the array antenna model parameters some fundamental antenna characteristics such as total antenna length and antenna width, TRP and peak EIRP can be derived.

The antenna physical width can be determined as:

want=Ndh		(Eq. 2.4-1)

and the total length of the SBFD configuration can be determined as:

lant=2Mdv+dant		(Eq. 2.4-2)

The power capability in terms of total conducted power is calculated in logarithmical scale as:

Ptot=10log10PMN+Ptx		(Eq. 2.4-3)

where P, M, N, Ptx is described in Table 2.3-2.

For BS output power considerations, the total radiated power (TRP) is calculated in logarithmical scale as:

TRP=10log10PMN+Ptx-L		(Eq. 2.4-4)

where P, M, N, Ptx and L is described in Table 2.3-2.

For radiated power considerations peak Equivalent Isotropic Radiated Power (EIRP) is calculated in logarithmical scale calculated as:

EIRP=Ptx+GE,max+20log10MN+10log10Msub+10log10P 		(Eq. 2.4-5)

where P, M, N, Msub, Ptx and GE is described in Table 2.3-2.

Observe that here all powers refer to total power as the sum over supported polarizations.

Using Eq. 2.3-1 to Eq. 2.3-5, physical size and power capability can be calculated from model parameters. The result is listed for considered deployment scenarios and frequency ranges in Table 2.4-3.

Table 2.4-3: Physical size and power capability

| Frequency range | Deployment scenario | Antenna width (m) | Antenna length (m) | Total conducted power (dBm) | TRP (dBm) | EIRP (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | Urban macro | 0.3 | 1.56 | 53.1 | 51.1 | 79.3 |
|  | Dense urban macro | 0.3 | 0.93 | 50.1 | 48.1 | 73.3 |
|  | Dense urban micro | 0.3 | 0.93 | 47.0 | 45.0 | 70.0 |
|  | Indoor | 0.15 | 0.45 | 39.0 | 37.0 | 56.5 |
| FR2-1 | Urban macro | 0.12 | 0.23 | 32.8 | 30.8 | 64.1 |
|  | Dense urban macro | 0.12 | 0.14 | 29.8 | 27.8 | 58.1 |
|  | Dense urban micro | 0.12 | 0.14 | 29.8 | 27.8 | 58.1 |
|  | Indoor | 0.04 | 0.12 | 25.1 | 23.1 | 48.7 |

Observe that total conducted power is a fictive parameter for highly integrated AAS BS or millimetre wave base stations since access to conducted interfaces does not exist. For integrated AAS BS, the power capability is better described as TRP and EIRP. However, for modelling purposes total conducted power per transmitter branch is often used.

  - Propagation model

The pathloss is modelled as described in Table 2.5-1 with an extension to better reflect BS-to-BS propagation and aligned to RAN1 agreement. The source to the model is TR 38.828, Table 5.2.3.1-1.

Table 2.5-1: Extended pathloss model

| Model | Equations | Shadow fading std (dB) | Applicability range, antenna height (m) |
| --- | --- | --- | --- |
| UMa LOS | PL1=32.4+20log10d3D+20log10(fc) | σSF=4.0 | 10<d2D<dBP 1 |
|  | PL2=32.4+40log10d3D+20log10fc-10log10((dBP)2+(hBS-hUT)2) | σSF=4.0 | dBP<d2D<5000 1.5<hUT<22.5 hBS=25 |
| Note 1: d'BP=4.h'BS.h'UT.fc/c, where fc is the centre frequency in Hz, c = 3.0.108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. |  |  |  |

In previous RAN1 meeting, consensus was reached to modify the expression of Line of Sight (LOS) probability according to the following agreement:

For LOS probability of gNB-gNB channel,

- For Macro-gNB-to-Macro-gNB case

  - Option 3: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901.

    - X = 0.75

- For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901.

Proposal 2: To follow the agreement already reached by RAN1 with respect to the BS-to-BS LOS probability: If the 2D distance between two Macro BSs is less than or equal to the ISD (Inter-site distance), so the LOS probability to 75%; otherwise, reuse BS-to-UE LOS probability from TR 38.901.

  - Network grid shift

When the placement of the two networks is considered, it is reasonable to study co-existence for different considerations. The separation in terms of network grid shift is defined as a percentage value. To capture different scenarios where operators have separate site grids with large separation to where operator are sharing site infrastructure the following configurations are relevant:

- 100% would be relevant for the case where two networks using separate site infrastructure separated with maximum distance is considered. This is a very specialized situation in which two operators manage to co-ordinate and place their BS sites at the maximum possible distance form one another.

- 10% is representative of a situation in which the BSs are not co-located, but the operators cannot co-ordinate to the extent that their BSs are always at maximum distance from one another.

- 0% is co-location; the simulations will show the impact of activity in the other network on co-existence.

Since we are evaluating a new technology, it is good to consider not just the extreme of maximum separation but also some intermediate value. Therefore, we propose to study impact at 0%, 10% and 100% network grid shift.

  - Coexistence evaluation

In the simulation campaign for which results are presented in the following, 6 different cases for UE distribution and network grid are considered. Different combinations of network grid shift and UE placement have been considered, as described in Table 2.7-1.

Table 2.7-1: Considered deployment scenarios

| 0% grid shift, Uniform UE distribution | 10% grid shift, Uniform UE distribution | 100% grid shift, Uniform UE distribution |
| --- | --- | --- |
| 0% grid shift, Clustered UE distribution | 10% grid shift, Clustered UE distribution | 100% grid shift, Clustered UE distribution |

In Figure 2.7-1, an example of network configuration is visualised for uniform and clustered UE distribution and 10% network grid shift. Clusters are randomly placed in the network coverage area. Considered inter-site distance (ISD) is 500 m, and <BS total conducted power is 49 dBm, even though typical power capability is larger for macro-BS as indicated in Table 2.4-3.

# Figure 2.7-1: Example of network layout with uniform and clustered UE distribution and 10% network grid shift

The proposed coexistence evaluation procedure considers two networks, N1 and N2. N2 is always an STDD network, while N1 can change depending on the coexistence scenario we are interested in. N1 can be an: 1) STDD, 2) SBFD with same antenna gain, 3) SBFD with same antenna area. We focus on two coexistence scenarios:

- STDD is the victim network: performance of N2 is evaluated when coexisting with N1 in any of its possible configurations: 1) STDD, 2) SBFD with same antenna gain (constant gain, as per definition in Section 2.2), 3) SBFD with same antenna area (reduced gain, as per definition in Section 2.2). The baseline in this coexistence evaluation is the performance of N2 when it coexists with an STDD N1.

- SBFD is the victim network: the baseline in this coexistence evaluation is an isolated SBFD N1 network, which is not affected by ACI (Adjacent Channel Interference). We compare this performance to one where N1 SBFD coexists to a legacy N2 STDD network, so that it is affected by ACI. Both antenna configurations (same area and same gain) are considered.

The considered carrier configuration for the STDD carrier and SBFD carrier is visualised in Figure 2.7-2. Notice that for the case of SBFD we consider a carrier configuration, where the DL slots of STDD are substituted by SBFD slots, but the UL STDD slot is maintained also in SBFD network and is synchronized to the STDD network. We opt for this configuration since it allows to protect the UL of the legacy STDD operator, which would be otherwise affected by BS-to-BS interference from the SBFD operator. This configuration protects the performance of the STDD operator but comes at the cost of a loss in flexibility and of the need to maintain the two operators synchronized.

# Figure 2.7-2: Carrier configuration for SBFD and STDD carrier

For the purpose of comparison, simulation assumptions for system level performance in RAN1 are captured in annex in section 5.

  - Simulation results

The simulation campaign used parameters are described in previous sections, except for BS output power which is assumed to be 49 dBm according to [1].

The simulation results are organized in two subsections. In subsection 2.8.1 we focus on evaluating the impact on a victim legacy STDD network from another STDD or SBFD network; in subsection 2.8.2 we focus instead on evaluating the performance of a SBFD network when it is affected or not by ACI (Adjacent Channel Interference) from a legacy neighbour STDD operator.

The focus is on an urban macro deployment with three-sectorized BSs and FR1 (4 GHz). Concerning the antenna configuration, the urban macro case reported in Table 2.4-1 is considered. The offered traffic is defined so that the resource utilization of the links is high, because when in SBFD we shift resources to the UL, for an equivalent offered load, the utilization will be pushed to near 100% utilization values, assimilable to full buffer conditions. The ACIR (between adjacent channels of different operators, and adjacent sub-bands of the same SBFD operator), and isolation values are reported in the following Table 2.8-1.

Table 2.8-1 Intra and Inter network ACIR and isolation values

|  | Parameter | Value (dB) |
| --- | --- | --- |
| Intra-network isolation values | Antenna isolation for self-interference | 80 |
|  | Front-to-back ratio (3-sector) | 60 |
|  | Digital Cancellation | 15 |
|  | UE-UE SBFD operator | 28 |
|  | BS-BS SBFD operator | 42 |
| Inter-network isolation values | BS-UE (adjacent channel) | 33 |
|  | UE-BS (adjacent channel) | 30 |
|  | Co-site isolation | 45 |

Results are presented as mean user throughput and 5%-tile throughput, for DL and UL. Without loss of generality, the focus of the following discussion is on network grid shift 10%, which we consider as a realistic setup. Campaigns have been conducted also for network grid shift 0% and 100%, leading to similar observations and conclusions Based on that, to avoid redundancy, in the following only network grid shift 10% plots are presented and discussed.

    - STDD is the victim network

The results will be presented as bars in a common figure as described below:

- The baseline for legacy STDD networks, i.e., the coexistence between two STDD networks, is plotted in blue colour bar.

- The impact on victim STDD network from SBFD network with same antenna gain is plotted in orange colour bar.

- The impact on victim STDD network from SBFD network with same antenna area is plotted in green colour bar.

Mean user throughput results, for both DL and UL are plotted in Figure 2.8.1-1 for uniform UE distribution and in Figure 2.8.1-2 for clustered UE distribution.

# Figure 2.8.1-1: Mean user throughput for uniform UE distribution (STDD as the victim)

# Figure 2.8.1-2: Mean user throughput for clustered UE distribution (STDD as the victim)

From Figures 2.8.1-1 and 2.8.1-2 it can be observed that independently of the user’s distribution in the scenario, the impact of UL traffic transmitted over the SBFD UL sub-band is not significant. Specifically, the performance of the STDD victim N2 is not affected by the coexistence with another SBFD network. Analysing the behaviour of the different sources of interference that contribute to the SINR (Signal to Interference and Noise Ratio), the dominating interference source is the inter-cell co-channel. The resulting DL SINR and consequently the DL user throughput, may oscillate depending on the clustering (closer UEs generate higher inter-cell interference), the power of the received signal (which may be lower in case of clustering), the higher power spectral density of DL SBFD, (which distributes the same transmission power as STDD over a smaller number of resource blocks, compared to STDD), and finally the antenna configuration (the same antenna area configuration generates less DL inter-cell and adjacent channel interference). In this context, the UE-to-UE CLI is not determining the DL STDD coexistence performance, which may look like a promising result. However, getting deeper into the network data, if the UL SINR of the SBFD network over the different kinds of slots is analysed in detail, it can be observed that the UL SINR over the SBFD slots is so low (below –10 dB), that these UL sub-band resources cannot be used due to the internal BS-to-BS interference generated by the DL of SBFD and the BS-to-BS interference generated by the adjacent victim STDD operator. The detail of this SINR value is depicted in Figure 2.8.1-3.

# Figure 2.8.1-3: UL SINR in SBFD network for kind of slots.

# The UL of N2 victim STDD is unaffected by the operation of the neighbour SBFD operator, thanks to the selected carrier configuration, which maintains the last UL slot synchronized to that of the legacy STDD victim operator.

Similar behaviours are observed for the corresponding results of 5%-tile user throughput, as plotted in Figure 2.8.1-4 for DL, for the cases of uniform and clustered distribution of users. Notice that UL 5%-tile throughput is not shown because with 80% indoor users, and a high load level assimilable to full buffer traffic, the UL 5%tile throughput is zero.

# Figure 2.8.1-4: 5%-tile user throughput for uniform (left) UE distribution and clustered (right) UE distribution (STDD as the victim)

Observation 4: The DL and UL performance of a legacy STDD operator is not affected by a neighbour SBFD operator.

Observation 5: The UL is not affected since the considered carrier configuration is designed so that an UL only slot is maintained synchronized to the UL slot of STDD pattern. In absence of this configuration, UL performance should be further evaluated.

Observation 6: The DL is not affected because the UL sub-band of SBFD is so interfered when the traffic load is high, that it results unusable for transmission.

Proposal 3: Traffic at a certain level of utilization should be forced on the SBFD UL sub-band so that coexistence performance can be properly evaluated for the DL of the legacy STDD network.

One option could be to simulate an ideal SBFD that artificially does not generate internal CLI (Cross Link Interference) in the SBFD network. Another option would be to force the UEs to transmit anyway on the UL sub-band, even if they will not get any throughput.

    - SBFD is the victim network

We analyse the performance of N1 SBFD coexisting with a STDD operator.

In the following figures 5 cases are compared:

- The baseline for legacy STDD networks, i.e., the coexistence between two STDD networks, is plotted in blue colour bar. This bar is plotted as a reference for comparison.

- The performance of an isolated SBFD network deployment, considering a same gain antenna configuration, and not affected by ACI, is plotted in orange colour bar.

- The impact on victim SBFD network from an STDD neighbour network, with same gain antenna configuration, is plotted in green colour bar.

- The performance of an isolated SBFD network deployment, considering a same area antenna configuration, and not affected by ACI is plotted in light blue bar.

- The impact on victim SBFD network from an STDD neighbour network, with same area antenna configuration, is plotted in purple colour bar.

Mean user throughput results, for both DL and UL are plotted in Figure 2.8.2-1 for uniform UE distribution and in Figure 2.8.2-2 for clustered UE distribution, and 10% network grid shift.

# Figure 2.8.2-1: Mean user throughput for uniform UE distribution (SBFD as the victim)

Figure 2.8.2-2: Mean user throughput for uniform UE distribution (SBFD as the victim)

From Figures 2.8.2-1 and 2.8.2-2, it can be observed that the DL performance of SBFD is reduced, compared to STDD, due to the shift of resources to the UL sub-band, and to the increased probability of interference in DL of SBFD, caused by an increment in resource utilization, when DL resources are shifted to UL, and the offered traffic is the same as STDD. The DL of SBFD is affected by the DL STDD of the adjacent operator, as can be observed comparing the orange to the green bars. Finally, the reduction of the antenna area further decreases the DL performance of SBFD, as it can be observed when comparing light blue and purple bars.

Concerning the UL, as previously described in Figure 2.8.1-3, the UL sub-band of SBFD is highly interfered by the DL of SBFD and by the DL of the adjacent neighbour operator. This high level of interference brings the SINR to such a low level, that these resources become unusable, so that the UL performance of SBFD becomes comparable to that of a STDD transmitting UL traffic only through the last UL slot, which is designed to be synchronized across the two operators. When the antenna area has to be maintained, and consequently the antenna gain is halved, the performance of SBFD even down perform that of STDD.

It is worth noting that with the setup agreed in previous meeting, SBFD operating at high load already generates such a high BS-to-BS interference on the UL sub-band that its UL performance reaches already the lower bound represented by the UL of STDD. In this setup, it is not possible to evaluate the impact of an adjacent operator, since the SBFD internal interference is so high that the additional BS-to-BS interference generated by the DL of STDD cannot make the performance worse. This is why the orange and green bars, and the light blue and purple bars show the same level of performance. As a result of that, the coexistence performance of SBFD/STDD networks should be evaluated in conditions when the victim network performs properly (so, for example, at a low load operation point), in order to be able to evaluate the degradation of performance when coexisting with a neighbour operator.

Similar behaviours are observed for the corresponding results of 5%-tile user throughput, as plotted in Figure 2.8.2-3 for DL, in cases of uniform and clustered distribution of users. Notice that UL 5%-tile throughput is not shown because with 80% indoor users, and a high load level assimilable to full buffer traffic, the UL 5%tile throughput is zero for certain setups, e.g., same antenna area configuration, clustered scenario.

Figure 2.8.2-3: 5%-tile user throughput from uniform (left) UE distribution and clustered (right) UE distribution (SBFD as the victim)

Observation 7: An isolated SBFD network operated at high low generates such a high level of BS-to-BS interference that the UL traffic is mainly served by the UL only slot, so that the UL SBFD network performance becomes similar to that of an UL STDD network.

Observation 8: When we add ACI from an adjacent STDD operator, the UL performance of SBFD cannot be further reduced, so that it is not possible to evaluate coexistence performance.

Proposal 4: Coexistence performance should also be evaluated in a setup where SBFD internal interference is not too high. An option could be to evaluate coexistence performance when the victim SBFD is operating at low load, while the aggressor STDD at high load.

# Conclusion

In this contribution we have collected technical background information including aspects related to SBFD coexistence modelling and parameter assumptions. Since modelling and parameters deviate from previous work in RAN4 we need to capture technical background in the SBFD technical report (TR 38.858).

Initial co-existence simulation results for given parameter assumptions have been provided based on guidance in way-forward from last meeting. At this stage it can be noticed that:

- The DL and UL performance of a legacy STDD operator is not affected by a neighbour SBFD operator.

- The UL is not affected since the considered carrier configuration is designed so that an UL only slot is maintained synchronized to the UL slot of STDD pattern. In absence of this configuration, UL performance should be further evaluated.

- The DL is not affected because the UL sub-band of SBFD is so interfered when the traffic load is high, that it results unusable for transmission.

- An isolated SBFD network operated at high low generates such a high level of BS-to-BS interference that the UL traffic is mainly served by the UL only slot, so that the UL SBFD network performance becomes similar to that of an UL STDD network.

- When we add ACI from an adjacent STDD operator, the UL performance of SBFD cannot be further reduced, so that it is not possible to evaluate coexistence performance.

In addition to simulation assumptions and initial simulation results following proposals are presented for approval to further stimulate the work for next meeting:

Proposal 1: We propose to capture simulation assumptions listed in Table 2.1-1 used for the co-existence evaluation in the SBFD technical report TR 38.858 and clearly indicate extensions to previously used models.

Proposal 2: To follow the agreement already reached by RAN1 with respect to the BS-to-BS LOS probability: If the 2D distance between two Macro BSs is less than or equal to the ISD (Inter-site distance), so the LOS probability to 75%; otherwise, reuse BS-to-UE LOS probability from TR 38.901.

Proposal 3: Traffic at a certain level of utilization should be forced on the SBFD UL sub-band so that coexistence performance can be properly evaluated for the DL of the legacy STDD network.

Proposal 4: Coexistence performance should be evaluated in a setup where SBFD internal interference is not too high. An option could be to evaluate coexistence performance when the victim SBFD is operating at low load, while the aggressor STDD at high load.

# References

[1]	R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC

[2]	R4-2109872, “Draft LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Qualcomm

[3]	R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Qualcomm

[4] 	TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, v14.3.0, 3GPP

[5]	R4-2212620, “Analysis of issues related to self-interference and co-channel CLI”, Ericsson

[6]	R4-2212457, “On issues related antenna modelling and additional information relevant for FR2 base station”, Ericsson

[7]	Document 5D/1276-E, “BASE STATION PARAMETERS AND MITIGATION TECHNIQUES FOR MMWAVE FREQUENCY BANDS”, Telefon AB LM Ericsson

# Annex: Parameters used for system level evaluations

## Urban Macro Scenario

|  | Parameters | Scenario |
| --- | --- | --- |
| System parameters | Scenario | UMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | Carrier Frequency | 4 GHz |
|  | Duplex Type | Static TDD (DDDDU and DUDDU), SBFD (XXXXU) |
|  | Base Static TDD pattern | 80:20 DL:UL |
|  | SBFD pattern | 80:20 SBFD slot:UL |
|  | Channel bandwidth | 100 MHz for STDD 40:20:40 MHz (DUD) for SBFD |
|  | Available resource blocks | 273 for STDD 106:51:106 (DUD) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot SBFD->UL: 2OS in the SBFD slot |
|  | Sub-Carrier spacing | 30 kHz |
|  | Number of active UEs | 4000 |
|  | Channel model | gNB-UE: UMa gNB-gNB: UMa with 75% LoS if 2D<ISD UE-UE: UMi |
|  | UE to BS min 2D distance | 35 m |
|  | Gridshift | 0%, 10%,100% |
| BS | (Mg, Ng, M, N, P), where M, N indicate sub-array number | (1,1,4,8,2) (same antenna gain) (1,1,4,4,2) (same antenna area) |
|  | Sub-array configuration | 3x1 |
|  | Max gNB Tx Power (per polarization) | 49 dBm (same antenna gain) 46 dBm (same antenna area) |
|  | (dv, dh) | (0.7λ, 0.5λ) |
|  | Antenna element gain | 6.4 dBi |
|  | Antenna element and sub-array model | R4-2109872, Table 1-2: Macro urban |
|  | Subarray electrical downtilt | 3 deg |
|  | Mechanical downtilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
|  | BS height | 25 m |
|  | Panel HW assumptions | Same antenna gain, same antenna area |
| UE | UE antenna | 1TX 2RX |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 |
|  | SNR target | 15 dB |
|  | Noise figure | 9 dB |
|  | Max modulation | 64 QAM |
|  | UE distribution outdoor:indoor | 20:80 |
|  | Buildings | One 120x50 m building per cell placed randomly |