3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215851

Electronic Meeting, 10-19 October 2022

Source:	Ericsson, Telstra

Title:	TP for TR 38.718-11-11: DC_3-7-n26

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-12-11 to include DC_3A-7A_n26A, DC_3A-7C_n26A, DC_3C-7A_n26A and DC_3C-7C_n26A configurations  as defined in WID [1].

# ---Start of changes---

## 5.X	DC_3-7_n26

### 5.X.1	Operating bands for EN-DC

Table 5.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3-7_n26 | CA_3-7 | n26 | Yes |

### 5.X.2	Configuration for DC

Table 5.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7A_n26A DC_3C-7C_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A | CA_3A-7A CA_3A-7C CA_3C-7A CA_3C-7C | n26A |

### 5.X.3		∆TIB and ∆RIB values

For DC_3-7_n26, the TIB,c and RIB,c values are reused from DC_3-7_n5 and are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n26 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n26 | 0.3 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n26 | 3 | 0 |
|  | 7 | 0 |
|  | n26 | 0 |

### 5.X.4	REFSENS requirements

Reusing the coexistence study results from EN-DC combination DC_3-7_n5, MSD values should be defined as below:

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7A_n26A DC_3C-7C_n26A | 3 | 1780 | 10 | 50 | 1875 | N/A | FDD | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 |  | IMD21 |
|  | n26 | 845 | 5 | 25 | 890 | N/A |  | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-222505, Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei