3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2215862

# Electronic Meeting, October 10- October 19, 2022

Agenda item:	6.20.2

Source: 	vivo

Title: 	Discussion on the improvement on FR2 SCell/SCG setup/resume

Document for:	Discussion

- Introduction

In the RAN4 #104-meeting, companies provided general analysis on the impact of measurement acquisition/ reporting on FR2 SCell/SCG setup/resume delay and the feasibility of the improved measurement. For the issues proposed in the last meeting, we will provide our views in this paper.

- Discussion

The following issues were captured in the WF on Further NR Mobility Enhancements [1].

- RAN4 are to discuss the feasibility of the improved measurement for FR2 SCell/SCG setup - Clarification on time point of “when UE has initiated access” (Issue 2-1-1) - Potential direction for further study: measurement enhancement when UE is about to enter connected mode (Issue 2-1-2) - Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode (Issue 2-1-3) - Applicable scenarios (Issue 2-1-4) - Assumption for feasibility study: RF chain status when performing enhanced measurement (Issue 2-2-1) - Assumption for feasibility study: number of frequency layers (Issue 2-2-2) - Assumption for feasibility study: Reduced number of samples (Issue 2-2-3) - Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping (Issue 2-2-4) - Assumption for feasibility study: Configuration assumption (Issue 2-2-5) - Whether RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay (Issue 2-3-1) - Impact on RACH due to measurement during RRC connection setup/resume (Issue 2-3-2) - Feasibility of improvement in FR2 SCell/SCG setup delay (Issue 2-3-3)

2.1 Clarification and potential direction

- Clarification on time point of “when UE has initiated access” (Issue 2-1-1) - Option 1 (Ericsson, Nokia): RAN4 to clarify that the “when UE has initiated access” is the point in time when RRC procedures in clauses 5.3.2.3, 5.3.3.2 or 5.3.13.2 (38.331) are initiated. - Paging - Initiation (UE receives SIB1 and apply default MAC cell group configuration) - RRC connection resume - Option 2 (CMCC, Apple, QC, xiaomi, HW, vivo, MTK): Discuss the starting point for improved measurement directly. - Potential direction for further study: measurement enhancement when UE is about to enter connected mode (Issue 2-1-2) - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume. - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest. - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest. - Option 2 (Apple, Ericsson, Nokia): UE perform enhanced measurement during RRC connection setup/resume. - Option 2a: perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume) - Option 2b (MTK, vivo, xiaomi, HW): after receiving paging for MT originating call - Option 2c (MTK, vivo, xiaomi, HW): after first RACH preamble transmission, i.e. Msg1

Actually, issue 2-1-1 and issue 2-1-2 seem to discuss the similar question: When will UE start to perform improved measurement. And issue 2-1-1 focuses more on whether this clarification is needed. In our understanding, the necessity of the clarification in Option 1(issue 2-1-1) could be further discussed after the starting point for enhanced measurement is determined. For the Option 1 in issue 2-1-2, the available time that could be used for measurement is more limited than Option 2. We prefer to use Option 2b and Option 2b which respectively corresponds to MO and MT call to further evaluate the feasibility of enhanced measurement.

Proposal 1: Whether it needs the clarification on the time point of “when UE has initiated access” could be further discussed after the starting point for enhanced measurement is determined.

Proposal 2: For MT originating call, UE perform enhanced measurement after receiving paging. And for MO call, UE perform enhanced measurement after first RACH preamble transmission, i.e. Msg1.

- Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode (Issue 2-1-3) - Option 1 (Apple, xiaomi, MTK, HW, vivo): further enhancement on Rel-16 EMR (measurement during green part) is out of scope - Option 2 (Ericsson, Nokia, LGE): further enhancement on Rel-16 EMR (measurement during green part) is in the scope Note: Option 1 is not precluding using the measurement results obtained during EMR for measurement during RRC connection procedure. “Enhancement on R16 EMR (measurement during green part)” refers to that the enhanced measurement is still performed in idle/inactive mode and before UE initiating access.

According to the description in the WID below, we could clearly know that the time point that enhanced measurement will be initiated is when UE requests RRC connection setup/resume. Therefore, for the temporal dimension, the further enhancement on Rel-16 EMR (measurement during green part) is out of scope and R18 only focus on the new measurement (measurement during orange part).

The description from WID

- The following sequence of events should be assumed.

    - The UE initiates and performs improved measurements when it requests RRC connection setup/resume.

Furthermore, so far Option 1 does not preclude the possibility of UE utilizing the measurement result/prior information obtained during EMR to the new measurement during RRC connection. And in our understanding, only based on this assumption could the short and accuracy measurement be performed.

Proposal 3: Further enhancement on Rel-16 EMR (measurement during green part) is out of scope while the possibility of UE utilizing the early measurement result/prior information for the enhanced measurement does not precluded so far.

- Applicable scenarios (Issue 2-1-4) Agreement: - The enhanced measurement (if feasible) at least applies to scenarios that the EMR measurement results are unavailable or invalid. Way forward - Revised Option 2 (QC, vivo, LGE, Nokia): Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and valid - e.g. On the cells that have been detected/measured in early measurement

Some companies questioned the necessity on performing enhanced measurement under the scenario in revised Option 2. When EMR measurement results are ‘available’ and ‘valid’, network could directly use these EMR results for CA/DC configuration and it is unnecessary to perform enhanced measurement. For this point, maybe the question is, companies have different understanding for ‘available’ and ‘valid’. Therefore, we think one thing needs to be aligned firstly is about the definition of ‘available/unavailable’ and ‘valid/invalid’.

In the last meeting, some companies pointed out that ‘available’ means that there have results stored in the memory after UE finished early measurement. We agree with this interpretation. Further, it does not mean that the measurement is not yet out-of-date and can be used directly for CA/DC configuration. And for ‘valid’, there have two kinds of interpretations, one means ‘All prior information including the beam information and cell information are available to use for enhanced measurement’ and another means ‘Due to the possible long delay between the ending point of EMR and the starting point of the enhanced measurement, the measurement results are out-of-date whereas the beam information is still helpful’. Considering the limited time can be used for enhanced measurement, we think the latter definition of ‘valid’ is more applicable and conductive for us to further study short and accuracy measurement during RRC connection setup/resume. Furthermore, we still see that difficulty to give a further accurate definition of ‘valid’. Considering this, we think maybe we can borrow some ideas from the existing definition of cell known conditions: ‘The validity of the early measurement result can be assessed by determining whether the measurement was taken within the last [X] seconds.’ Under this definition, the enhanced measurement is applicable to the scenario that EMR measurement results are ‘available’ but ‘invalid’. And it will focus on the cells that have been detected/measured in early measurement so that UE do not have to perform cell search.

Proposal 4: RAN4 should align the definition of ‘available’ and ‘valid’. ‘Available’ means that there have results stored in the memory after UE finished early measurement. And the ‘valid’ early measurement result can be assessed by determining whether the measurement was taken during the last [X] seconds before the enhanced measurements perform.

Proposal 5: According to the definition of ‘available’ and ‘valid’ in proposal 4, the applicable scenario can be modified as:

- Revised Option 2: Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and invalid

  - e.g. On the cells that have been detected/measured in early measurement

2.2 Assumptions for feasibility study

- Assumption for feasibility study: RF chain status when performing enhanced measurement (Issue 2-2-1) - Option 1 (Apple, HW, QC): One active RF chain - Option 2 (CMCC, xiaomi, MTK, Ericsson, vivo, Nokia): Two active RF chains - Option 3 (Nokia): more than two active RF chains

Considering if one RF chain is used as the general case, performing enhanced measurement during the RRC connection setup/resume may have potential impact on the RACH procedure. From this perspective, we prefer to use Option 2 as the baseline.

Proposal 6: Use two RF chain as the baseline when performing enhanced measurement.

- Assumption for feasibility study: number of frequency layers (Issue 2-2-2) - Reduce the number of EMR carriers to be measured for improved measurement. - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable.

Considering the limitation of setup time, reducing the number of frequency layers is a method worth considering while ensuring the measurement accuracy as much as possible. For how to select the frequency layers for enhanced measurement, we believe that only one frequency layer needs to be measured on each band, so that other frequency layers on the same band do not need to spend extra delay to measure. And the measurement delay can be reduced. Then how many frequency layers to measure will depend on the number of bands that can be supported.

Proposal 7: For how to select the frequency layers for enhanced measurement, we believe that only one frequency layer needs to be measured on each band and the number of frequency layers to measure will depend on the number of bands that can be supported.

- Assumption for feasibility study: Reduced number of samples (Issue 2-2-3) - Option 1: Yes - Option 2 (Apple, MTK, vivo, HW): No - Option 3 (CMCC, xiaomi, Ericsson, LGE, Nokia): FFS how to reduce number of samples without measurement accuracy degradation.

For the common case, the number of samples is the key point to guarantee accuracy. If we consider to reduce the number of samples, it will bring some negative impact for the performance robustness. Meanwhile, there doesn't seem to be any special scenario where it can reduce the number of samples without measurement accuracy degradation. Therefore, in our opinion, sacrificing accuracy for less measurement delay is not a priority.

Proposal 8: Considering reducing the number of samples comes at the expense of accuracy, it should not be a priority.

- Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping (Issue 2-2-4) - Option 1(Ericsson, Nokia): Reduce the scaling factor of Rx beam sweeping - Option 1a (CMCC): use Rx beam sweeping factor of R17 HST FR2 or R17 positioning - For FR2 HST, the value of scaling factor is 2 or 6 pending on the different deployment. - In Rel-17 positioning WI, the candidate Rx beam sweep numbers for reduced Rx beam sweeping factor (<8) UE capability are {1, 2, 4, 6}. - Option 1b (xiaomi, Ericsson, vivo, QC): FFS use prior information on the UE Rx beam to reduce the scaling factor of Rx beam sweeping - Option 2 (Apple, MTK, HW): Not to reduce the scaling factor of Rx beam sweeping - Option 3 (Apple, Ericsson): Further discussion

For the first bullet in Option 1, the reason why it can be considered to reduce the Rx beam sweeping factor of R17 HST FR2 is the special deployment of high-speed train. It is not applicable for the common scenarios related to enhanced measurement during RRC connection setup/resume procedure. Therefore, maybe we cannot borrow some ideas from it. And for Rel-17 positioning enhancement, we notice that in order to reduce the PRS measurement latency for FR2 positioning frequency layers, a UE capability is introduced for lower Rx beam sweeping factor (e.g., 1, 2, 4, 6). We think it can be a candidate way to reduce the scaling factor. And considering the very limited time for RRC connection setup/resume, the optional values of scaling factor need to be smaller, e.g.1 or 2.

For the Option 1b, it could be considered in the case which the early measurement result is ‘available’ but ‘invalid’. the previous beam information obtained in the early measurement can be regarded as the additional information from the network. It is useful for the scaling factor reduction.

Proposal 9: In order to reduce the scaling factor of Rx beam sweeping, both introducing UE capability for lower Rx beam sweeping factor like Rel-17 positioning and using the previous beam information obtained in the early measurement as the additional information can be the candidate options.

- Assumption for feasibility study: Configuration assumption (Issue 2-2-5) Agreement: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. Way forward: - Option 1 (vivo, QC): use SMTC when specifying the requirements - Option 2 (Ericsson): use SSB period when specifying the requirements - Option 3 (Apple, xiaomi, Ericsson, MTK, HW, Nokia): Further discussion

We support to use SMTC when specifying the requirements. And for Option 2, it needs to have further discussion on how does UE can know the actual SSB periodicity of non-serving cell.

Proposal 10: Use SMTC configuration when specifying the requirements on enhanced measurement.

2.3 Feasibility discussion

- Whether RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay (Issue 2-3-1) Agreement: RRC connection setup/resume delay is very short for improvement on FR2 Scell/SCG setup delay. Way forward - Option 1 (Ericsson, Nokia, vivo): Study short and accuracy measurement during RRC connection setup/resume. - Feasibility of improvement in FR2 SCell/SCG setup delay (Issue 2-3-2) Further discuss the feasibility of improvement in FR2 SCell/SCG setup delay.

Considering the very limited time for RRC connection setup/resume, the short and accuracy measurement is a considerable way to further study. In our opinion, the short and accurate measurement can only be performed on the cells that have already detected in the early measurement. In this scenario, UE perform the enhanced measurement to further check whether the quality of detected cell is still strong to be used for FR2 CA/DC configuration.

Proposal 11: The short and accurate measurement can only be performed on the cells that have already detected in the early measurement. In this scenario, UE perform the enhanced measurement to further check whether the quality of detected cell is still strong to be used for FR2 CA/DC configuration.

- Impact on RACH due to measurement during RRC connection setup/resume (Issue 2-3-2) Agreement: MSG2/MSG4 during RACH procedure may be lost due to measurement on FR2 intra-band inter-frequency during RRC connection setup/resume. Way forward: - Option 1 (Apple, MTK, HW): For inter-band inter-frequency measurement, Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning. - Option 2 (vivo, Nokia): FR2 intra-band CA is not the target scenario of enhanced measurement. - Option 3 (Ericsson): Study potential mitigation schemes to avoid/mitigate the impact on RACH due to measurement during RRC connection setup/resume.

For Option 2, according to the current requirement on SCell activation, for intra-band CA in FR2, we can know that the activation delay is TFirstSSB+ 5ms. It seems there have little gain to perform enhanced measurement in this scenario. Therefore, it is not necessary to discuss the impact on RACH due to measurement during RRC connection setup/resume for FR2 intra-band CA.

For Option 1, if there are more than one frequency to measure during RRC connection setup/resume, RF retuning may impact the reception of Msg2 and Msg4. And the degree of this impact will deepen with the increase of the number of frequency layers. Therefore, in order to mitigate the impact, one related thing is to keep the number of frequency layers for enhanced measurement within a small range. As we propose in Issue 2-2-2, we can consider that only one frequency layer is measured on each band. In this way, the number of frequency layers for enhanced measurement will be reduced which further reduces the possibility of the impact due to RF retuning.

Proposal 12: A smaller number of frequency layers could be considered in the enhancement measurements in order to minimize the impact of RF tuning during RACH procedure.

- Conclusions

Proposal 1: Whether it needs the clarification on the time point of “when UE has initiated access” could be further discussed after the starting point for enhanced measurement is determined.

Proposal 2: For MT originating call, UE perform enhanced measurement after receiving paging. And for MO call, UE perform enhanced measurement after first RACH preamble transmission, i.e. Msg1.

Proposal 3: Further enhancement on Rel-16 EMR (measurement during green part) is out of scope while the possibility of UE utilizing the early measurement result/prior information for the enhanced measurement does not precluded so far.

Proposal 4: RAN4 should align the definition of ‘available’ and ‘valid’. ‘Available’ means that there have results stored in the memory after UE finished early measurement. And the ‘valid’ early measurement result can be assessed by determining whether the measurement was taken during the last [X] seconds before the enhanced measurements perform.

Proposal 5: According to the definition of ‘available’ and ‘valid’ in proposal 4, the applicable scenario can be modified as:

- Revised Option 2: Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and invalid

  - e.g. On the cells that have been detected/measured in early measurement

Proposal 6: Use two RF chain as the baseline when performing enhanced measurement.

Proposal 7: For how to select the frequency layers for enhanced measurement, we believe that only one frequency layer needs to be measured on each band and the number of frequency layers to measure will depend on the number of bands that can be supported.

Proposal 8: Considering reducing the number of samples comes at the expense of accuracy, it should not be a priority.

Proposal 9: In order to reduce the scaling factor of Rx beam sweeping, both introducing UE capability for lower Rx beam sweeping factor like Rel-17 positioning and using the previous beam information obtained in the early measurement as the additional information can be the candidate options.

Proposal 10: Use SMTC configuration when specifying the requirements on enhanced measurement.

Proposal 11: The short and accurate measurement can only be performed on the cells that have already detected in the early measurement. In this scenario, UE perform the enhanced measurement to further check whether the quality of detected cell is still strong to be used for FR2 CA/DC configuration.

Proposal 12: A smaller number of frequency layers could be considered in the enhancement measurements in order to minimize the impact of RF tuning during RACH procedure.

- Reference

[1] R4-2214348 WF on Further NR Mobility Enhancements, MediaTek Inc.