3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2215868

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.8.3.2

Source:	vivo

Title:	On L3 measurement for FR2 multi-Rx chain

Document for:	Discussion

- Introduction

In the RAN4#104-e meeting, there were initial discussions on the RRM impacts and general aspects of FR2 multi-Rx chain DL reception. The following agreements were achieved and captured in the WF [1].

<Agreement >: Issue 1-1-1-2: Number of searchers for cell detection and measurements - 2 searchers are assumed. Issue 1-1-3: Feasibility/necessity of enhancing requirements for L1 measurements - Following L1 measurements related requirements are considered in the multi-Rx chain WI - L1-RSRP measurement - BFD/CBD - RLM - L1-SINR - TCI state switching with dual TCI - Scheduling/measurement restrictions

There were also discussions on feasibility/necessity of enhancing requirements for L3 measurements for multi-Rx chain UE. However, no agreements could be achieved.

Issue 1-1-4: Feasibility/necessity of enhancing requirements for L3 measurements - Proposals - Option 1: Following L3 measurements related requirements are enhanced in the multi-Rx chain DL reception WI - RRM measurement requirements for RRC-IDLE state and for RRC_INACTIVE state. - Handover to FR2-1 unknown cell - FR2-1 unknown SCell activation - FR2-1 unknown PSCell addition and release - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG). - Option 1a: Following L3 measurements related requirements are enhanced in the multi-Rx chain DL reception WI - Handover to FR2-1 unknown cell - FR2-1 unknown SCell activation - FR2-1 unknown PSCell addition and release - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG). - Option 2a: Not to enhance L3 measurement requirement in R18 multi-panel WI.

In this contribution, we provide our views on feasibility/necessity of enhancing requirements for L3 measurements for FR2 multi-Rx chain DL reception.

- Discussion

According to the WID [1], L3 measurement delay enhancement can be considered for multi-Rx chain.

- L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements

The starting point can focus on the enhancements related to L1-RSRP measurement enhancements. L1-RSRP measurement delay can be reduced by multi-Rx chain simultaneous DL reception from different directions. In the last meeting, feasibility/necessity of enhancing L1-RSRP measurements are confirmed. Therefore, such enhancement could also help to reduce delay for L3 procedures. For example, L1-RSRP measurement is needed for FR2 unknown SCell activation. The overall FR2 SCell activation delay can be reduced accordingly, if L1-RSRP measurement delay is reduced with multi-Rx chain simultaneous DL reception. Of course, in which WI, eFeRRM or this WI, the enhancement is discussed is separated issue.

In many L3 procedures, e.g., handover to FR2 unknown target cell, PSCell addition of unknown FR2 target cell, PSCell change with unknown FR2 target cell and SCG activation with FR2 unknown PSCell, cell search is needed and Rx beam sweeping is also needed during cell search. In general, for UE with multi-Rx chain capability, it can search the SSBs of unknown target cell from two different directions simultaneously. Thus, the cell search delay can be reduced due to less Rx beam sweeping number and overall procedure delay can be reduced accordingly.

For L3 measurements, it was agreed 2 searchers are assumed for FR2 multi-Rx chain DL reception. For the above L3 procedures, the cell search of the FR2 unknown target cell during activation period is prioritized over any other L3 measurements. So, the searcher limitation would not be an issue. Multi-Rx chain UE is also able to receive SSBs from different directions. So, it is feasible to enhance cell search time during the above L3 procedures.

Observation 1: FR2 SCell activation delay for unknown SCell can be enhanced by reducing L1-RSRP measurement time and/or cell search time for multi-Rx chain UE.

Observation 2: Handover delay, PSCell addition/change delay and SCG activation delay for FR2 unknown target cell/PSCell can be enhanced by reducing cell search time for multi-Rx chain UE.

For L3 measurements in connected mode, it is different story. Due to searcher limitation, UE may not be able process the signals received from different directions at baseband depending on CA/DC configurations.

Observation 3: It needs further study if enhancement of L3 measurements in connected mode for multi-Rx chain simultaneous DL reception is feasible.

If RAN4 agrees to introduce L3 measurement delay for UE supporting multi-Rx chain simultaneous reception, a new capability should be introduced different from simultaneousReceptionDiffTypeD-r16 or other L1 measurement related UE capability. UE supporting L1 measurements with simultaneous DL reception does not necessarily mean L3 procedures/measurements that include L1 measurements can automatically be enhanced for multi-Rx chain UE.

Proposal 1: A new capability, which is different from simultaneousReceptionDiffTypeD-r16 or other L1 measurement related UE capability, should be introduced for L3 procedure/measurement delay reduction for UE supporting multi-Rx chain simultaneous reception, if L3 measurement enhancement is agreed to be specified.

- Summary

In this contribution, we provided views on feasibility/necessity of enhancing requirements for L3 measurements for FR2 multi-Rx chain DL reception. Following observations and proposal are present.

Observation 1: FR2 SCell activation delay for unknown SCell can be enhanced by reducing L1-RSRP measurement time and/or cell search time for multi-Rx chain UE.

Observation 2: Handover delay, PSCell addition/change delay and SCG activation delay for FR2 unknown target cell/PSCell can be enhanced by reducing cell search time for multi-Rx chain UE.

Observation 3: It needs further study if enhancement of L3 measurements in connected mode for multi-Rx chain simultaneous DL reception is feasible.

Proposal 1: A new capability, which is different from simultaneousReceptionDiffTypeD-r16 or other L1 measurement related UE capability, should be introduced for L3 procedure/measurement delay reduction for UE supporting multi-Rx chain simultaneous reception, if L3 measurement enhancement is agreed to be specified.

- References

- RP-221753	Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL

- R4-2214344	WF on FR2 multi-Rx RRM, vivo