3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2215870

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.8.3.4

Source:	vivo

Title:	On TCI state switch for FR2 multi-Rx chain

Document for:	Discussion

- Introduction

In the RAN4#104-e meeting, there were initial discussions on the RRM impacts and general aspects of FR2 multi-Rx chain DL reception. The following agreements were achieved and captured in the WF [1].

<Agreement >: Issue 1-1-1-2: Number of searchers for cell detection and measurements - 2 searchers are assumed. Issue 1-1-3: Feasibility/necessity of enhancing requirements for L1 measurements - Following L1 measurements related requirements are considered in the multi-Rx chain WI - L1-RSRP measurement - BFD/CBD - RLM - L1-SINR - TCI state switching with dual TCI - Scheduling/measurement restrictions

There were many other issues being identified and discussed. The open issues of TCI state switching are also captured in the WF [1]. We provided our views on TCI state switching requirements in [2] in the last meeting. In this contribution, we further provide our views on TCI state switching requirements for FR2 multi-Rx chain DL reception.

- Discussion

- 2.1 General aspects for TCI related

Issue 1-3-1-1: Independent TCI switching for dual TCIs for multi-Rx chain UE - Proposals - Option 1: Each TCI switching per RX chain is assumed to be independent in aspect of TCI switching delay. RAN4 to study if Rel-17 TCI switching delay requirements can be applicable as Rel-18 UE requirements with multi-RX chains. - Option 2: RAN4 to study if a UE with multiple RX chains tracks time and frequency per TCI when dual TCIs are activated per RX chain. - Option 3: RAN4 to discuss the requirements for any change to the set of active TCI states used for simultaneous reception, e.g., active TCI state switching within this set.

For multi-TRP operation to support 4-layer MIMO, two active TCI states should be configured. Since the two TCI states are activated by the same MAC CE, the starting time for TCI state switching is the same. When dual TCI states switching is triggered, UE should be able to track timing for each TCI state and perform TCI state switching independently. Depending on assumption of receiving time difference, UE may track timing of only one of the TCI states if receive timing difference is within CP. Otherwise, independent timing tracking is needed.

Proposal 1: UE with multi-Rx chain should track timing/frequency independently for each TCI state when dual TCI states are activated.

Issue 1-3-1-2: TCI framework for dual TCIs - Proposals - Option 1: Dual TCI state switching delay requirements shall base on Rel-15/16 TCI framework. - Option 2: To define requirements for TCI switching delay with dual TCI, both R15/R16 TCI framework and R17 TCI framework, i.e., unified TCI, are considered for UE supporting multi-Rx chain simultaneous reception.

The Rel-15/16 TCI framework should be considered in the WI when defining TCI related requirements for multi-Rx chain UE. For R17 TCI framework, as dual TCI states for multi-TRP is to be introduced in Rel-18 MIMO evo WI, requirements for dual TCI states switching enhancement based on multi-Rx chain DL reception may be discussed in Rel-18 MIMO evo WI. It would also be fine to discuss it in this WI from technical perspective. It is another issue that may need coordination between different WIs.

Proposal 2: Dual TCI state switching delay requirements shall be based on Rel-15/16 TCI framework in this WI.

There is another issue related to use cases for dual TCI state switching as in option 3.

- Define dual TCI state switching requirements for following cases: - PDCCH non-SFN: Two MAC CE with one for each TCI state - PDCCH SFN: single MAC CE for two TCI states - PDSCH single DCI: single DCI for two TCI states - PDSCH multiple DCI: Two DCI with one for each TCI state

The four cases are valid for multi-TRP operation and requirements for TCI state switching should be considered for these cases.

Proposal 3: When defining Dual TCI state switching delay requirements, following cases shall be considered.

- PDCCH non-SFN: Two MAC CE with one for each TCI state

- PDCCH SFN: single MAC CE for two TCI states

- PDSCH single DCI: single DCI for two TCI states

- PDSCH multiple DCI: Two DCI with one for each TCI state

- 2.2 Known/unknown condition

Legacy requirements for MAC CE based TCI state switch are specified under both known and unknown conditions. It is specified for known conditions only for DCI based TCI state switch. To define dual TCI states switch delay requirements for multi-Rx chain, known/unknown conditions should also be decided.

Issue 1-3-2: Known/unknown conditions for TCI state switching for multi-Rx chain - Proposals - P1: Study whether TCI state switching delay can be modified or not for multi Rx chain UE, e.g. how to evaluate known/unknown condition by using multi Rx chain, how to define TCI state change delay by the cases that dedicated Rx chain is used, partial Rx chains are used, or all Rx chains are used. - P1a: For MAC-CE based TCI state switch delay with unknown target TCI state, the reduction on switch delay requirements can be considered (i.e. consider the improvement of TL1-RSRP with multiple simultaneous reception) - P1b: For dual TCI state switching delay requirements, which scenarios of triggering, known/unknown TCI state and TCI association to different PCI could be considered and clarified. - P1c: Discuss the known conditions of dual TCI and discuss whether to define requirements for unknown dual TCI state switch.

In general, similar principle can be followed for dual TCI states switch. For MAC CE based dual TCI states switch, requirements should be specified for both known and unknown conditions. The known/unknown condition are specified for each TCI state. For example, if two TCI states switch is triggered by single MAC CE, there could be case that one TCI state is known and the other TCI state is unknown. For DCI based dual TCI states switch, the requirements should be specified for known conditions only.

In legacy requirements, known conditions for TCI state switch is defined as follows.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

For dual TCI states switch, the legacy known/unknown condition can be reused in general and it is for each TCI state of the dual TCI states.

Proposal 4: The legacy known/unknown condition can be reused for each TCI state of the dual TCI states.

Proposal 5: For MAC-CE based dual TCI states switch, requirements for both known and unknown conditions are specified.

Proposal 6: For DCI based dual TCI states switch, requirements are only specified for known condition, i.e., each of the dual TCI states are known.

- 2.3 TCI state switching delay requirements

Issue 1-3-3: Enhancement for TCI state switching delay requirements - Proposals - P1: For dual TCI state switch, the legacy TCI state switch delay requirement can be reused - P2: The new RRM requirements defined for simultaneous measurements and procedures on two chains need to apply, provided the corresponding active TCI states are configured and used for simultaneous reception during the measurement or evaluation period. - P3: The legacy TCI state switching delay requirement can apply for each RX beam.

For dual TCI states switch, it is expected two TCI states switch are performed independently. Thus, the dual TCI states switch delay requirements can be specified for each TCI state and legacy requirements can be reused. However, 4-layer MIMO can only be schedule after both TCI states switch are completed.

Proposal 7: For dual TCI states switch, delay requirements are specified for each TCI state and legacy TCI state switch delay requirements are reused.

In legacy TCI state switch delay requirements, L1-RSRP measurement are needed if target TCI state is unknown. Rx beam sweeping is needed for FR2 then. For UE with multi-Rx chain, beam sweeping factor can be reduced because the UE can receive from two directions simultaneously. Therefore, there is possibility to enhance legacy TCI state switch delay requirements by reducing L1-RSRP measurement time under unknown conditions.

Proposal 8: Legacy TCI states switch delay requirements are enhanced for UE with multi-Rx chain.

- Summary

In this contribution, we provided views on RRM core requirements for NR FR2 multi-Rx chain DL reception WI.

Proposal 1: UE with multi-Rx chain should track timing/frequency independently for each TCI state when dual TCI states are activated.

Proposal 2: Dual TCI state switching delay requirements shall be based on Rel-15/16 TCI framework in this WI.

Proposal 3: When defining Dual TCI state switching delay requirements, following cases shall be considered.

- PDCCH non-SFN: Two MAC CE with one for each TCI state

- PDCCH SFN: single MAC CE for two TCI states

- PDSCH single DCI: single DCI for two TCI states

- PDSCH multiple DCI: Two DCI with one for each TCI state

Proposal 4: The legacy known/unknown condition can be reused for each TCI state of the dual TCI states.

Proposal 5: For MAC-CE based dual TCI states switch, requirements for both known and unknown conditions are specified.

Proposal 6: For DCI based dual TCI states switch, requirements are only specified for known condition, i.e., each of the dual TCI states are known.

Proposal 7: For dual TCI states switch, delay requirements are specified for each TCI state and legacy TCI state switch delay requirements are reused.

Proposal 8: Legacy TCI states switch delay requirements are enhanced for UE with multi-Rx chain.

- References

- R4-2214344	WF on FR2 multi-Rx RRM, vivo

- R4-2213054	Discussion on RRM requirements for multi-chain DL reception, vivo