3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215881

# Electronic Meeting, Oct 10th  – 19th, 2022

Source:	ZTE Corporation

Title:	Discussion on Tx switching across 3 or 4 bands

Agenda Item:	6.19.2

Document for:	Discussion

# Introduction

In RAN4#104-e, a WF was agreed regarding on UL Tx switching across 3 or 4 bands with single TAG [1][2]. And one of the open issues is the exact value of Tx switching period in this scenario:

Exact value of Tx switching period • Further discuss the two options in the next meeting: – Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching. – Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

In our view, Rel-16/17 have already provided sufficient implementation flexibility by allowing different Tx switching period for 1T-2T and 2T-2T switching cases for the same pair of switching bands, and increasing the number of bands while actual switching still happens in one selected band pair does not require a different switching period.

# Discussion

In Rel-17, there are some fundamental enhancements for Tx switching on top of Rel-16:

- From 1T-2T to 2T-2T switching

- One of the two bands may include 2 contiguous CCs

Such fundamental enhancements have direct impacts on switching operation for a band pair, thus are sufficiently able to justify a different switching period for the same band pair, though the value set is the same.

Observation 1: Rel-17 Tx switching enhancements are sufficiently able to justify a different switching period for the same band pair.

Fig. 1, Tx switching in Rel-18

Further enhancement in Rel-18 is to increase the number of bands for the switching operation. As shown in Fig. 1, the actual switching is still band-pair-wise operation due to the limitation of two concurrent Tx chains, and the additional operation is just to select 2 bands out of 3 or 4 bands for the switching, and it has no direct impact on the actual switching, unless such implementation aspects are identified.

Observation 2: The additional operation of choosing 2 bands out of 3 or 4 bands for the switching in Rel-18 does not have a direct impact on the actual band-pair-wise switching operation, unless such implementation aspects are identified.

Therefore, we propose that RAN4 specify the same value of switching period for each band pair in Rel-18 as Rel-16/17.

Proposal: RAN4 to specify the same value of switching period for each band pair in Rel-18 Tx switching as Rel-16/17.

# Conclusion

In this paper, we have the following observations and proposal for UL Tx switching in Rel-18:

Observation 1: Rel-17 Tx switching enhancements are sufficiently able to justify a different switching period for the same band pair.

Observation 2: The additional operation of choosing 2 bands out of 3 or 4 bands for the switching in Rel-18 does not have a direct impact on the actual band-pair-wise switching operation, unless such implementation aspects are identified.

Proposal: RAN4 to specify the same value of switching period for each band pair in Rel-18 Tx switching as Rel-16/17.

# Reference

R4-2214463, WF on UL Tx switching across 3/4 bands with single TAG, China Telecom

R4-2214249, Email Discussion Summary for [104-e][138] NR_MC_enh, Moderator(China Telecom)