3GPP TSG-RAN WG4 Meeting # 104bis-e												R4-2215884

Electronic Meeting, 10th – 19th October 2022

Agenda item:			6.18.3

Source:	Ericsson

Title:	On accuracy improvement based on NR carrier phase measurements

Document for:	Discussion

# Introduction

RAN4#104e discussed RF issues related to carrier phase measurements for positioning and made the following agreement [1]:

Agreement:

- Wait for RAN1 conclusion or RAN1 LS to start RAN4 work on accuracy improvement study based on carrier phase measurements

# Discussion

Issues related to NR carrier phase measurement for positioning were also discussed in RAN1#110. The discussion was mainly focused on assumptions for evaluation of carrier phase measurements for positioning and following agreements were made:

Agreement
In the evaluation of NR carrier phase positioning, the following frequency errors can be considered, which are modeled independently for each UE and each TRP:

  - Initial Residual CFO (is the same for one measurement instances [or multiple phase measurement instances]):

    - Ideal: 0 (UE/TRP)

    - Practical: uniform distribution within

        - [-30, +30] Hz (FR1, UE), [-100, +100] Hz (FR1, UE),

        - [-120, +120] Hz (FR2, UE), [-400, +400] Hz (FR2, UE),

        - [-10, +10] Hz (for each TRP, FR1),

        - [-40, +40] Hz (for each TRP, FR2).

  - Oscillator-drift (is the same for one or multiple phase measurement instances for positioning fix):

    - Ideal: 0 (UE/TRP)

    - Practical: uniform distribution within [-0.1, 0.1] ppm (UE), [-0.02, +0.02] ppm (each TRP) within measurement duration

- Note: The Doppler frequency can be determined based on the UE speed in the evaluation assumption.

Agreement
In the evaluation of NR carrier phase positioning, the offset between the initial phase of the transmitter and the initial phase of the receiver can be modeled as a random variable uniformly distributed within [0, X].

- Possible values of X: 2pi

  - Other values FFS

Agreement
In the evaluation of NR carrier phase positioning, the antenna reference point (ARP) location error of a TRP can be modeled as follows:

  - Ideal: no ARP error

  - Practical: a zero-mean, truncated Gaussian distribution with zero mean and standard deviation of T=[1, 5] cm truncated to 2T in each of (x, y, z) direction

Agreement

In the evaluation of NR carrier phase positioning, the following the UE/TRP antenna phase center offset (PCO) model can be considered as the starting point:

dPCO =  a * dPhi + w

where

  - a is the scale factor, a=[0, 1, 3]

    - FFS: other values

  - dPhi is the direction difference (in degrees):

    - Example 1, dPhi is the difference between the true and the calculated (or measured) directions between a transmitter (UE/TRP) and a receiver (TRP/UE).

    - Example 2: dPhi is the direction difference between one UE to two TRPs, or between one TRP to two UEs.

  - w is 0 or a random variable uniformly distributed within [-2, +2], or [-5, +5], or [-X, +X] degrees

    - FFS: value of X or left up to companies

  - Note: the above model is valid only when absolute value of dPhi < Y degrees

    - FFS: value of Y or left up to companies

Agreement

For the evaluation of NR carrier phase positioning, UE position can be calculated by the use of the carrier phase measurements obtained at the M sequential time instances, where

  - Baseline:

    - M=1

  - Optional :

    - M=4

    - Other values of M

  - Companies should report their assumptions on UE mobility (e.g. speed)

Agreement

Further evaluate the following multipath mitigation methods for the carrier phase positioning, which include, but are not limited to, the following:

  - The methods of estimating the carrier phase of the first path

    - Note: Both time-domain and frequency-domain methods can be considered

  - LOS/NLOS/ Multi-path indication for the carrier phase measurements for improving the accuracy of the position calculation

    - Rel-17 LOS/NLOS indicator can be used as the starting point

  - measurements of the first path and additional paths

    - E.g. carrier phase measurements, timing measurements

  - other channel information, such as RSRP/RSRPP, CIR/CFR, etc.

Observation 1:  RAN1 agreed to study impact of different error sources on carrier phase measurement accuracy.

Observation 2: RAN1 agreed on assumptions for modeling of different error sources that have an impact on achievable positioning accuracy based on NR carrier phase measurement.

As noted in observation 1 and observation 2, RAN1 have agreed to study impact of different error sources on carrier phase measurement accuracy and have aligned their view on evaluation assumptions. Since RAN1 has not concluded its evaluation on the impact of different error sources on achievable accuracy based on NR carrier phase measurement, scope of RF requirement to be defined by RAN4 is not yet clear.

Observation 3: RAN1 has not concluded its evaluation on the impact of different error sources on achievable accuracy based on NR carrier phase measurement.

It shall therefore be in interest of RAN4 to wait for at least initial conclusion from RAN1 before evaluating RF requirement aspects of the NR carrier phase measurement.

Proposal 1: RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement.

# Summary

In this paper we presented our view on RF requirement aspect of accuracy improvement based on NR carrier phase measurement. Since evaluation of different error sources and their impact on NR carrier phase measurement is a work in progress in RAN1in our view RAN4 shall wait for outcome of RAN1 evaluation for RF requirement study. The discussions in previous chapter are summarized in the observations and proposal listed below.

Observation 1:  RAN1 agreed to study impact of different error sources on carrier phase measurement accuracy.

Observation 2: RAN1 agreed on assumptions for modeling of different error sources that have an impact on achievable positioning accuracy based on NR carrier phase measurement.

Observation 3: RAN1 has not concluded its evaluation on the impact of different error sources on achievable accuracy based on NR carrier phase measurement.

Proposal 1: RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement.

# References

- R4-2214462, “WF on expanded and improved NR positioning study”, Intel.

- R1-2207690, “FL Summary for improved accuracy based on NR carrier phase measurements”, CATT.

- R1-2207691,	 “FL Summary #2 for improved accuracy based on NR carrier phase measurements”, CATT.

- R1-2208206, “FL Summary #3 for improved accuracy based on NR carrier phase measurements”, CATT.