3GPP TSG-RAN WG4 Meeting#104-bis-e	R4-2215886

Online, 10th Oct – 19th Oct, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on HST-SFN Scheme A

Agenda item:	4.5.3.1.1

Document for:	Discussion

- Introduction

During RAN4#103-e, simulation assumptions were agreed to define requirements HST-SFN Scheme A [1]. According to this scheme, TRS from different TRPs are to be transmitted in a non-SFN manner, whereas DMRS and PDSCH would be transmitted in an SFN manner. In this paper, we share simulation results for this scheme.

- Simulation Results for HST-SFN Scheme A

Based on the agreed simulation assumptions [1], we provide simulation results as below for both FDD and TDD scenarios with 1667Hz and 870Hz Doppler, respectively.

| Parameters | Values |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Table 1: Simulation Assumptions for HST-SFN Scheme A

Table 2: Alignment Results for HST-SFN Scheme A

| Duplex Mode/SCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
|  | Alignment SNR (dB) |  | Alignment SNR (dB) |  |
|  | MCS13 | MCS17* | MCS13 | MCS17* |
| FDD 15KHz | 8.99 | NA | 6.42 | NA |
| TDD 30KHz | 9.75 | NA | 7.51 | NA |

*Peak throughput cannot be achieved with MCS17

Table 3: Impairment Results for HST-SFN Scheme A

*Peak throughput cannot be achieved with MCS17

| Duplex Mode/SCS | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
|  | Impairment SNR (dB) |  | Impairment SNR (dB) |  |
|  | MCS13 | MCS17* | MCS13 | MCS17* |
| FDD 15KHz | 10.99 | NA | 8.42 | NA |
| TDD 30KHz | 11.75 | NA | 9.51 | NA |

# 3. Conclusions

In this paper, we share our share our simulation results for HST-SFN Scheme A.

# 4. References

[1] R4-2210670, “WF on demodulation requirement for Enhancement on HST-SFN deployment,” 3GPP TSG RAN WG4 Meeting #103-e, Online, May 09 – May 20, 2022.