# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215898

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	RF requirements on CA band combination of n5-n28

# Source: 	ZTE Corporation

# Agenda item:	6.2.3.2

# Document for:	Approval

# 1	Introduction

The CA band combination of the 700MHz, 800MHz and 900MHz bands (i.e. LB bands) were initial discussed in last meeting. For n5-n28, some agreements were captured in the WF[1], where:

BCS information：

n5: 5, 10, 15, 20

n28: 5, 10, 15, 20, 25, 30

- UE RF architecture

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

UL configuration

The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

RF requirement

  - No MSD due to IMD,

  - MSD due to cross band isolation.

In this contributions, we give some discussion on the RF requirements on n5-n28 for smartphone to support the NR CA configurations of LB bands.

# 2	Discussion

In terms of WF, the BCS information for 2UL/2DL CA_n5-n28 is:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |

In general, the studies on the RF requirements should base on the RF architecture. For the antenna implementation, either 2 antenna or 3 antenna is approved in the WF, where the antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

In our understanding, the relationship between the RF filter implementation with antenna number is:

1. 2 antenna: one is for common n5&n28 main UL/DL, one is for common n5&n28 diversity DL): n-plexer should be used.

2. 3 antenna: one for n5 main UL/DL and one for n28 main UL/DL, and the left one is for n5&n28 diversity DL, or one is for common n5&n28 main UL/DL, the other two are for n5 and n8 diversity DL, respectively.

For both implementation, simultaneous Tx/Rx can be enabled, there are no frequency range overlapping for this combination, and we think it is similar with the other LB-LB combinations like n5-n12, CA_n5-n14 and CA_n20-n28 which have already been included in the specification.

In terms of the WF, there is no IMD issue for 2UL CA_n5-n28, so the only RF requirements are the 1UL/2DL specific requirements, i.e. delta Tib,c/Rib,c and cross band isolation requirements

Observation 1. No IMD MSD for 2UL/2DL CA_n5-n28, and the left requirements are delta Tib,c/Rib,c and cross band isolation requirements should be defined for 1UL/2DL CA_n5-n28.

However, the delta Tib,c/Rib,c and cross band isolation requirements have already defined in the specification, where for ΔTIB,c defined as:

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n28 | 0.5 | 0.5 |

For ΔRIB,c are, 0dB are defined.

It shall be noted that the delta Tib,c/Rib,c for CA_n5-n28 are from LTE CA 5-28 according to [2], where the delta Tib,c/Rib,c for LTE CA_5-28 are defined in [3].

In addition, for the cross band MSD, some measurements have already been done in [3] for CA_n5-n28, where the n28 frequency range restriction to 703-733 MHz for the UL and 758-788 MHz for the DL was removed.

Observation 3. delta Tib,c/Rib,c and cross band isolation requirements have already been defined for 1UL/2DL CA_n5-n28 in the TS38.101-1.

Although there were no RF architecture discussed/included in the TP, considering all of RF requirements for 1UL/2DL CA_n5-n28 have already been included in the TS38.101-1, so we think there may no further work for this band combination.

Observation 4. There may no further work for 2UL/2DL CA_n5-n28.

# 3 Conclusion

In this contribution, we give some discussions on the RF requirements for CA_n5-n8.

The conclusions are summarized below:

Observation 1. No IMD MSD for 2UL/2DL CA_n5-n28, and the left requirements are delta Tib,c/Rib,c and cross band isolation requirements should be defined for 1UL/2DL CA_n5-n28.

Observation 2. delta Tib,c/Rib,c and cross band isolation requirements have already been defined for 1UL/2DL CA_n5-n28 in the TS38.101-1.

Observation 3. There may no further work for 2UL/2DL CA_n5-n28.

# 4	Reference

[1] R4-2214445, WF on study on FS_NR_700800900, CATT

[2] R4-2119869, TP to TR 38.717-02-01 Addition of CA_n5-n28, Nokia

[3]R4-1807209, TP for TR 36.715-02-01:CA_2DL_5A-28A_1UL_BCS0, Huawei, HiSilicon

[3] R4-2206141, Measurements for CA_n5-n28 MSD, Skyworks Solutions, Inc.