# 3GPP TSG-RAN WG4 Meeting # 104bis-e                                                R4-2215901

# Electronic Meeting, Oct. 10-Oct. 19, 2022

Source:	ZTE, Ericsson

Title:	TP for TR 37.718-21-11: Including band combinations DC_3-41_n1

Agenda item:			5.4.2

Document for:	Approval

1. Introduction

A text proposal for TR 37.718-21-11 to add the following configurations:

| Band combination configuration | UpLink configuration |
| --- | --- |
| DC_3A-41A_n1A | DC_3A_n1A |
| DC_3A-41A_n1A | DC_41A_n1A |
| DC_3A-41C_n1A | DC_3A_n1A |
| DC_3A-41C_n1A | DC_41A_n1A |
| DC_3A-41C_n1A | DC_41C_n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A |
| DC_3A-3A-41A_n1A | DC_41A_n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A |
| DC_3A-3A-41C_n1A | DC_41A_n1A |
| DC_3A-3A-41C_n1A | DC_41C_n1A |

Noted that the fallbacks of DC_3A_n1A,  DC_41A_n1A, DC_41C_n1A and DC_3A-3A_n1A have already been defined in current TS38.101-3 spec.

2. Text Proposal

---Start of changes---

# 5.x	DC_3-41_n1

5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-41A_n1A | DC_3A_n1A DC_41A_n1A | CA_3A-41A | n1A |
| DC_3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A | CA_3A-41C | n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A DC_41A_n1A | CA_3A-3A-41A | n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A | CA_3A-3A-41C | n1A |

5.x.2	Co-existence studies

When uplink is DC_3A_n1A there is IMD5 interfering band 41 downlink.

# 5.x.3		∆TIB and ∆RIB values

For DC_3-41_n1, the TIB,c and RIB,c values are reused from DC_1_n3-n41 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3-41_n1 | 0.5 | 0.33 / 0.84 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |  |

Table 5.x.3-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |
| DC_3-41_n1 | - | 03 / 0.54 | - |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz |  |  |  |

### 5.x.4	REFSENS requirements

It is proposed to re-use the IMD5 MSD values from already specified configuration DC_1_n3-n41 which is similar to DC_3-41_n1.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-41A_n1A DC_3A-41C_n1A DC_3A-3A-41A_n1A DC_3A-3A-41C_n1A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |

---End of changes---

Reference

[1]		RP-221832, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon