3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215910

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.1

Source:	Ericsson

Title:	The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the remaining issues of the PDSCH demodulation requirements for FR2-2. Furthermore, in RAN4 meeting #104-e, WF [1] underlined the propagation environment to be considered to set the test cases.

Channel model tap delay resolution - Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW - [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger than 200MHz; - Adopt channel model according to the channel bandwidth as (detailed in the Appendix): SCS [kHz]; CBW [MHz]; Applicable channel models; Tap resolution [ns]; [Maximum Number of Taps]. 120; 100; TDLA30-650 TDLD30-200; 5; 12. 120 480 [960]; ≥400; TDLA10-650 TDLD10-200; 2; 16. Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA - Option 1: QPSK; - Option 2: Decide based on simulation results; - Option 3: Do not introduce requirements with 650Hz Doppler Frequency; Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz - Agreement: 100MHz for 120kHz and keep 400MHz FFS Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz - Agreement: 400MHz for 480kHz and keep 800MHz FFS; Issue 1-2-2d: RB Allocation Table for SCS 120kHz / CBW 100MHz - Option 1: MCS 4 and 13: 66 RBs; MCS 17: 33RBs - Option 2: MCS 4,13: 66 PRBs; MCS17: 30PRBs - Option 3: MCS 4, 13, 17, and 20: Configure 66 RBs; Issue 1-2-2e: RB Allocation Table for SCS 480kHz / CBW 400MHz - Option 1: MCS 4: 66 RBs; MCS 13: 16 RBs - Option 2: MCS 4, 9: 66 RBs; MCS13: 33RBs; MCS 17: 16 RBs; Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput - Agreement (GTW August 22, 2022): FFS whether introduce requirements for 30% of peak throughput Issue 1-2-5: UE Phase Noise Model for Simulation Alignment - Both PN Models in TR 38.808; - Companies clarified that PN modelling can be used to run simulations for the choice of simulation parameters, while simulation results collection for alignment is not expected to use PN models (and expected PN impact is added as a margin to the impairment results). Unless there are diverging views, do not discuss this issue further in the second round; Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment - Agreement: CPE compensation only is agreed. - Agreement: Do not consider ICI compensation;

# 2	Remaining issues for PDSCH requirements

Several issues need to be discussed in this meeting. Then, we will provide our proposals to address them.

## 2.1	SCS/CBW combinations and general issues

For FR2-2, it is better that we focus on the SCS 120 KHz (as a common SCS with FR2-1) and an additional SCS such as 480 KHz. Furthermore, and to align our efforts with RAN1, RAN5 and TE vendor requirements, we refer to the following Table

Table 7.2.3-4: Maximum DL testable SNR for different allocations

| SCS, kHz | Allocation size, PRB | Allowed noise increase, dB |  |
| --- | --- | --- | --- |
|  |  | 1 dB | 2 dB |
| 120 | 264 | [-0.6] | [3.8] |
|  | 132 | [4.1] | [7.3] |
|  | 66 | [7.7] | [10.6] |
|  | 33 | [11] | [13.7] |
|  | 16 | [14.3] | [16.9] |
| 480 | 264 | < -20 (NOTE 1) | [-9.6] |
|  | 132 | [-14.5] | [-0.6] |
|  | 66 | [-0.6] | [3.8] |
|  | 33 | [4.1] | [7.3] |
|  | 16 | [7.9] | [10.7] |
| 960 | 165 | < -20 (NOTE 1) | < -20 (NOTE 1) |
|  | 132 | < -20 (NOTE 1) | [-9.6] |
|  | 66 | [-14.5] | [-0.6] |
|  | 33 | [-0.6] | [3.8] |
|  | 16 | [4.2] | [7.5] |
| NOTE 1: Result does not converge |  |  |  |

It is worth mentioning that the reference SNRs in the Table above have been derived based on the following parameters, gathered in the following Table

Table 7.2.3-1: Demodulation test setup SNR calculation parameters for band n263

| Parameter | Comment |
| --- | --- |
| REFSENS | Using REFSENS agreed for band n263 -72.0 dBm/400 MHz |
| Multi-band relaxation | TBD |
| TE amplifier 1dB compression | 23 dBm |
| Backoff from P1dB | -13 dB (RAN4 assumption) -17.7 dB (RAN5 assumption) |
| Cable loss | 10.3 dB, assuming 1m length |
| Connector insertion loss | 0 |
| FS path loss | 66.7 dB, assuming 0.725m range length |
| TE DL absolute power setting uncertainty | +/-6 dB |
| Probe antenna gain | 12 dBi |
| Beam peak search procedure error | 0.5 dB |

Observation 1: Is it worth reminding that the endorsed Proposal 4 in [2] states that: “For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability”. This has led to the SNRs increase in FR2-1. We expect same behavior in FR2-2 by considering a fading backoff margin of 11.08 dB instead of 17.7 dB.

Observation 2: We consider the reference SNRs as indicated in Table 7.2.3-4, and we trust that the companies and TE vendors will spend the needed efforts to review the reference SNRs whose may allow for higher MCS for 120 KHz/100 MHz, 480 KHz/400 MHz.

According to Observation 1 and 2, we propose the following

Proposal 1: RAN4 defines the PDSCH demodulation requirements with:

- FR2-2 TDD: SCS = 120 kHz with CBW = 100 MHz (66 RBs) only.

- FR2-2 TDD: SCS = 480 kHz with CBW = 400 MHz (66 RBs) only.

It is worth mentioning here that we still deprioritize SCS 960 KHz for the following reasons detailed in the following observation

Observation 3: Using SCS 960 KHz, we increase max CBW by 25 % (from 1600 MHz to 2000 MHz), while the CP is halved (looses 50% of its duration).

In Proposal 2, we state some general aspects needed for PDSCH demodulation.

Proposal 2: RAN4 defines the UE demodulation requirements for 120 KHz/100 MHz with

- TDLD30-200 and TDLA30-650 channel models

- Modulation order: Up to 64QAM

Proposal 3: RAN4 defines the UE demodulation requirements for 480 KHz/400 MHz with

- TDLD10-200 channel model

- Modulation order: Up to 16 QAM

## 2.2	PDSCH performance under Phase noise effects

NR operations in 52.5 GHz – 71 GHz band are susceptible to phase noise (PN) effects that should be assessed and addressed. However, RAN4 was not able to agree on a single representative PN model during the study item and WF [1]. In TR 38.808, two PN models have been defined and referred to as Set 1 and Set 2.

Proposal 4: Define PDSCH requirements using both PN models in TR 38.808, where PN is considered at the UE only, while we set the design margin as 0 dB at the BS and 5 dB at the UE, when applying TR 38.808 Set 1.

In WF [1], an important question on the maximum achievable MCS has been raised. Therein, two conditions should be satisfied as stated in Proposal 5 below.

Proposal 5: Define maximum MCS for PDSCH requirements considering

- Use CPE compensation only

- Consider performance degradation due to PN less than 1dB

Consequently, and based on our simulation results [3], we provide our view in Proposal 6 and Proposal 7.

Proposal 6: Define PDSCH demodulation requirements for UE at 70% of the peak throughput for

- 120 KHz/100 MHz: Up to MCS 17 (66 RBs) and MCS 20 (16 RBs)

- 480 KHz/400 MHz: MCS 4 (66 RBs)

Proposal 7: Define PDSCH demodulation requirements for UE at 30% of the peak throughput for

- 120 KHz/100 MHz: MCS 13 (66 RBs)

- 480 KHz/400 MHz: MCS 13 (16 RBs)

## 2.3	PDSCH Test cases and simulation assumption

For the sake of compatibility with FR2-1, it is recommended to keep same TDD UL/DL pattern for PDSCH when using SCS 120 KHz in FR2-2. The following proposal draws the interaction between the parameters needed for tests.

Proposal 8: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |

Proposal 9: Referring to the Table in Proposal 8, consider PDSCH test cases at 70% of the peak throughput for 120 KHz and 480 KHz SCS as follow

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz (66) | 4 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |
| 1-2 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |
| 1-3 | 100MHz / 120kHz (66) | 17 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | TBD | New |
| 1-4 | 100 MHz/ 120 KHz (16) | 20 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | TBD | New |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 400MHz / 480kHz (66) | 4 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | TBD | New |
| Note 1: As an extension to FR2.120-1, we proposed a TDD UL-DL pattern for SCS 480 KHz. |  |  |  |  |  |  |  |  |

Observation 4: For SCS 480 KHz, we defined a TDD UL-DL pattern in [2], given by 14D2S4U, S1 = 12D:2G:0U, S2 = 0D:6G:8U.

In the sequel, our proposals for the test cases’ parameters to be used when considering PDSCH at 30% of the peak throughput.

Proposal 10: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLD (10 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |

Proposal 11: Referring to the Table in Proposal 8, consider PDSCH test cases at 30% of the peak throughput using the following test setup.

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 30% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-2 | 400MHz / 480kHz (16) | 13 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | TBD | New |

# 5	Specification structure

Table 1	Specification structure for UE Demodulation performance requirements in FR2-2 (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.2 | PDSCH demodulation requirements | Update Table 7.2-1 |
| 7.2.1 | 1RX requirements |  |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.[4] | Minimum requirements for UEs in FR2-2 | New section |
| 7.2A | PDSCH demodulation requirements for CA |  |
| 7.2A.1 | 1RX requirements |  |
| 7.2A.2 | 2RX requirements | Updates on FR2-2 |
| ……. | …… |  |
| A.3.2 | Reference measurement channels for PDSCH performance requirements |  |
| A.3.2.2 | TDD | Updates on FR2-2 |

# 6	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: Is it worth reminding that the endorsed Proposal 4 in [2] states that: “For 64QAM scenarios (both Demod and CSI), consider fading backoff margin of 11.08 dB (replacing the current working assumption of 17.71 dB) corresponding to the 1e-3 faded signal clipping probability”. This has led to the SNRs increase in FR2-1. We expect same behavior in FR2-2 by considering a fading backoff margin of 11.08 dB instead of 17.7 dB.

Observation 2: We consider the reference SNRs as indicated in Table 7.2.3-4, and we trust that the companies and TE vendors will spend the needed efforts to review the reference SNRs whose may allow for higher MCS for 120 KHz/100 MHz, 480 KHz/400 MHz.

Proposal 1: RAN4 defines the PDSCH demodulation requirements with:

- FR2-2 TDD: SCS = 120 kHz with CBW = 100 MHz (66 RBs) only.

- FR2-2 TDD: SCS = 480 kHz with CBW = 400 MHz (66 RBs) only.

Proposal 2: RAN4 defines the UE demodulation requirements for 120 KHz/100 MHz with

- TDLD30-200 and TDLA30-650 channel models

- Modulation order: Up to 64QAM

Proposal 3: RAN4 defines the UE demodulation requirements for 480 KHz/400 MHz with

- TDLD10-200 channel model

- Modulation order: Up to 16 QAM

Proposal 4: Define PDSCH requirements using both PN models in TR 38.808, where PN is considered at the UE only, while we set the design margin as 0 dB at the BS and 5 dB at the UE, when applying TR 38.808 Set 1.

Proposal 5: Define maximum MCS for PDSCH requirements considering

- Use CPE compensation only

- Consider performance degradation due to PN less than 1dB

Proposal 6: Define PDSCH demodulation requirements for UE at 70% of the peak throughput for

- 120 KHz/100 MHz: Up to MCS 17 (66 RBs) and MCS 20 (16 RBs)

- 480 KHz/400 MHz: MCS 4 (66 RBs)

Proposal 7: Define PDSCH demodulation requirements for UE at 30% of the peak throughput for

- 120 KHz/100 MHz: MCS 13 (66 RBs)

- 480 KHz/400 MHz: MCS 13 (16 RBs)

Proposal 8: Define PDSCH demodulation requirements for UE at 70% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |

Proposal 9: Referring to the Table in Proposal 8, consider PDSCH test cases at 70% of the peak throughput for 120 KHz and 480 KHz SCS as follow

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz (66) | 4 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |
| 1-2 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |
| 1-3 | 100MHz / 120kHz (66) | 17 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | TBD | New |
| 1-4 | 100 MHz/ 120 KHz (16) | 20 Rank 1 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | CPE | TBD | New |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 70% Peak Throughput

| Test case | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | 400MHz / 480kHz (66) | 4 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | TBD | New |
| Note 1: As an extension to FR2.120-1, we proposed a TDD UL-DL pattern for SCS 480 KHz. |  |  |  |  |  |  |  |  |

Observation 4: For SCS 480 KHz, we defined a TDD UL-DL pattern in [2], given by 14D2S4U, S1 = 12D:2G:0U, S2 = 0D:6G:8U.

Proposal 10: Define PDSCH demodulation requirements for UE at 30% of the peak throughput using the following test setup.

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLD (10 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |

Proposal 11: Referring to the Table in Proposal 8, consider PDSCH test cases at 30% of the peak throughput using the following test setup.

FR2-2 TDD, SCS 120 KHz/100 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 30% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-1 | 100MHz / 120kHz (66) | 13 Rank 1 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | CPE | TBD | New |

FR2-2 TDD, SCS 480 KHz/400 MHz, Metric: 30% Peak Throughput

| Test number | CBW / SCS (PRB) | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | PN Compen- Sation | SNR (dB) @ 70% of peak Throughput | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-2 | 400MHz / 480kHz (16) | 13 Rank 1 | FR2.120-1 | TDLD10-200 | 2x2 ULA Low | CPE | TBD | New |

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- R5-221628, “Discussion on fading crest factor”, Qualcomm.

- R4-2215911, “Simulation results for PDSCH demodulation in 52.6 – 71 GHz band”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH Mapping Type A

### FR2-1

Please refer to 38.101-4 Tables A.3.2.2.5

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| SCS |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 for DL slots 9 for special slots |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

## A.2	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |