3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215911

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.1

Source:	Ericsson

Title:	Simulation results for PDSCH demodulation in 52.6 – 71 GHz band.

Document for:	Information

# 1	Introduction

In this paper and based on the progress summarized in WF [1], we present new simulation results on the PDSCH performance in the frequency range 52.6 GHz to 71 GHz. We carried out our simulations using the agreed channel models and parameters.

Channel model tap delay resolution - Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW - [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger than 200MHz; - Adopt channel model according to the channel bandwidth as (more details in the Appendix): SCS [kHz]; CBW [MHz]; Applicable channel models; Tap resolution [ns]; [Maximum Number of Taps]. 120; 100; TDLA30-650 TDLD30-200; 5; 12. 120 480 [960]; ≥400; TDLA10-650 TDLD10-200; 2; 16. Issue 1-1-3: MCS to use with 650Hz Doppler Frequency for TDLA - Option 1: QPSK; - Option 2: Decide based on simulation results; - Option 3: Do not introduce requirements with 650Hz Doppler Frequency; Issue 1-1-5: Max CBW for UE Demodulation Requirements definition for SCS 120kHz - Agreement: 100MHz for 120kHz and keep 400MHz FFS Issue 1-1-6: Max CBW for UE Demodulation Requirements definition for SCS 480kHz - Agreement: 400MHz for 480kHz and keep 800MHz FFS; Issue 1-2-2d: RB Allocation Table for SCS 120kHz / CBW 100MHz - Option 1: MCS 4 and 13: 66 RBs; MCS 17: 33RBs - Option 2: MCS 4,13: 66 PRBs; MCS17: 30PRBs - Option 3: MCS 4, 13, 17, and 20: Configure 66 RBs; Issue 1-2-2e: RB Allocation Table for SCS 480kHz / CBW 400MHz - Option 1: MCS 4: 66 RBs; MCS 13: 16 RBs - Option 2: MCS 4, 9: 66 RBs; MCS13: 33RBs; MCS 17: 16 RBs; Issue 1-2-4: Whether RAN4 should introduce requirements for 30% of peak throughput - Agreement (GTW August 22, 2022): FFS whether introduce requirements for 30% of peak throughput Issue 1-2-5: UE Phase Noise Model for Simulation Alignment - Both PN Models in TR 38.808; - Companies clarified that PN modelling can be used to run simulations for the choice of simulation parameters, while simulation results collection for alignment is not expected to use PN models (and expected PN impact is added as a margin to the impairment results). Unless there are diverging views, do not discuss this issue further in the second round; Issue 1-2-6: UE Phase Noise compensation Model for Simulation Alignment - Agreement: CPE compensation only is agreed. - Agreement: Do not consider ICI compensation;

# 2	PDSCH Simulation Results

## 2.1 Simulation setup

As per WF [1], we carried out simulations using the agreed propagation environments. Furthermore, RAN4 was not able to agree on a single representative phase noise (PN) model during the study item. Therefore, and for the sake of fairness, we performed our simulations considering both PN models in TR 38.808 [2], referring to as Set 1 and Set 2, where Set 1 has been proposed in [3] and Set 2 also known as TR 38.803 Example 2.

We will use as general parameters.

- Numerologies:

- SCS = 120 KHz, CBW = 100 MHz (66 PRBs, 33 PRBs, and 16 PRBs).

- SCS = 480 KHz, CBW = 400 MHz (66 PRB and 16 PRB).

- MCS 4, 13, 17, and 20

- Max Doppler frequency/DMRS

- UE speed at 3 Km/h (f_D = 200 Hz), DMRS 1+1 ((2,11) symbol indexes)

- UE speed at 10 Km/h (f_D = 650 Hz), DMRS 1+1 ((2,11) symbol indexes)

- Two PN models in TR 83.808 [2]:

    - Set 1 in TR 38.808: PN model in [3], referred to as R4_2010176

The design margin (DM) should be set to 0 dB for BS while to be set to 5 dB for UE

    - Set 2 in TR 38.808: PN model TR38.803 Ex2, Set 2 in TR 38.808

- Carrier frequency: 70 GHz

- Antenna configuration: 2x2 ULA Low

- Rank 1

- Rel-15 PTRS configuration, K = 2, L = 1

- Phase noise at the UE receiver only

- Benchmark: No PN and No compensation

- CPE compensation only

We summarize the parameters above in the following table

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread (DS)) and TDLD (10 ns DS) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | TR38.803 example 2 BS PN profile and Set 1 [2] |
| UE PN Model | TR38.803 example 2 UE PN profile and Set 1 [2] |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol indexes |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 4, 13, 17, and 20. Note: It is assumed that NohPRB = 0 for MCS calculations. |

## 2.2 Simulation results

In this section, we will present the normalized PDSCH throughput as function of the signal to noise ratio (SNR). As per RAN4 PDSCH metric, we will refer to SNR1 and SNR2 as the corresponding SNR value to 70% (or 30 %) of the peak throughput value when using PN models Set 1 and Set 2, respectively.

Furthermore, it is worth mentioning that considering the PRB number allocation, we only apply CPE compensation. In the sequel, we present the simulation setup tables that we have used.

Test cases:

- CBW (MHz)/SCS (KHz) = 100/120, TDLA30-200/650 and TDLD30-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 70 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 1-1 | 70 | 100/120 (66) | 4 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low | TBD | TBD |
| Test 1-2 | 70 | 100/ 120 (66) | 13 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-3 | 70 | 100/ 120 (66) | 17 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-4 | 70 | 100/ 120 (33) | 17 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-5 | 70 | 100/ 120 (33) | 20 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low |  |  |
| Test 1-6 | 70 | 100/ 120 (16) | 20 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low |  |  |

- CBW (MHz)/SCS (KHz) = 400/480, TDLA10-200 and TDLD10-200, Metric: 70% Peak Throughput

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 70 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| Test 2-1 | 70 | 400/480 (66) | 4 | 1 | TDLA10-200 TDLD10-200 | CPE | 2x2 ULA Low | TBD | TBD |
| Test 2-2 | 70 | 400/480 (16) | 13 | 1 | TDLA10-200 TDLD10-200 | CPE | 2x2 ULA Low |  |  |

- Metric 30% Peak Throughput, HARQ processes: 16

| Test case | fc (GHz) | CBW/SCS (PRB) | MCS | Rank | Propagation condition | PN compensation | Antenna config | SNR @ 30 Peak Throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | SNR1 (Set 1) | SNR2 (Set 2) |
| est 3-1 | 70 | 100/120 (66) | 13 | 1 | TDLA30-200 TDLA30-650 TDLD30-200 | CPE | 2x2 ULA Low | TBD | TBD |
| Test 3-2 | 70 | 400/ 480 (16) | 13 | 1 | TDLA10-200 TDLD10-200 | CPE | 2x2 ULA Low |  |  |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCSs but limited to 1 MIMO layer (Rank 1). | 1-1, 1-2, 1-3, 1-4, 1-5, 1-6 2-1, 2-2, 2-3, |
| 30% of peak Throughput | Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 3-1, 3-2 |

## 2.2.1 SCS/CBW = 120 KHz/100 MHz results

Considering our benchmark, when no PN applied at both sides (ideal case), the achieved SNRs at 70% of the peak throughput have been referred to as SNR0 and gathered along with SNR1 and SNR2 in Tables 1.1 and 1.2. Furthermore, in the sequel, we will refer to D1 = SNR1 – SNR0 and D2 = SNR2 – SNR0 as the SNR gap (dB) between the PN CPE compensation output and the NO PN case.

For 120 KHz SCS and 100 MHz CBW, Table 1.1 gathers the results where 66 PRBs have been allocated and MCS (4, 13 and 17) have been considered, while Table 1.2 considers moderate and high MCS (17 and 20) with partial PRB allocation. It is worth that the max DL testable SNR level is at [7.7] dB.

Table 1.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for MCS (4, 13, and 17) and using 66 PRBs.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-1 | 4 (66) | TDLA30-200 | -3.5 | -3.3 | 0.2 | -3.2 | 0.3 |
|  |  | TDLA30-650 | -3.1 | -2.9 | 0.2 | -2.9 | 0.2 |
|  |  | TDLD30-200 | -7.2 | -7.0 | 0.2 | -6.9 | 0.3 |
| Test 1-2 | 13 (66) | TDLA30-200 | 4.4 | 4.8 | 0.4 | 5.0 | 0.6 |
|  |  | TDLA30-650 | 5.0 | 5.3 | 0.3 | 5.5 | 0.5 |
|  |  | TDLD30-200 | 0.6 | 1.0 | 0.4 | 1.1 | 0.5 |
| Test 1-3 | 17 (66) | TDLA30-200 | 7.7 | 8.3 | 0.6 | 8.8 | 1.1 |
|  |  | TDLA30-650 | 8.3 | 8.9 | 0.6 | 9.3 | 1.0 |
|  |  | TDLD30-200 | 4.1 | 4.6 | 0.5 | 5.0 | 0.9 |

Observation 1: Referring to Table 1.1, and considering SCS/CBW = 120 KHz/100 MHz with full PRB allocation for MCS (4 , 13 and 17) and CPE for PN compensation, we note that

- MCS 4 and MCS 13 are testable with TDLA30-650

- MCS17 is testable with TDLD30-200

Table 1.2 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for moderate and high MCS (17 and 20) with partial PRB allocation.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 1-4 | 17 (33) | TDLA30-200 | 7.8 | 8.4 | - | 8.8 | - |
|  |  | TDLA30-650 | 8.4 | 9.0 | - | 9.4 | - |
|  |  | TDLD30-200 | 4.1 | 4.7 | 0.6 | 5.1 | 1.0 |
| Test 1-5 | 20 (33) | TDLA30-200 | 10.1 | 11.1 | - | 11.8 | - |
|  |  | TDLA30-650 | 10.7 | 11.7 | - | 12.5 | - |
|  |  | TDLD30-200 | 6.4 | 7.3 | 0.9 | 8.0 | 1.6 |
| Test 1-6 | 20 (16) | TDLA30-200 | 9.9 | 10.9 | - | 11.6 | - |
|  |  | TDLA30-650 | 10.4 | 11.4 | - | 12.1 | - |
|  |  | TDLD30-200 | 6.2 | 7.2 | 1.0 | 7.8 | 1.6 |

Observation 2: Referring to Table 1.2, and considering SCS/CBW = 120 KHz/100 MHz with partial PRB allocation for MCS (17 and 20) and CPE for PN compensation, we note that

- MCS20 is testable with TDLD30-200 when considering 16 PRBs out of 66.

## 2.2.2 SCS/CBW = 480 KHz/400 MHz results

Following same arguments as in Section 2.2.1, the results have been gathered in Tables 2.1 where both the full and partial PBR allocation have been considered. It is worth mentioning here the max DL testable SNR is at [-0.6] dB.

Table 2.1 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for low MCS (4 and 13).

|  | CBW (MHz) / SCS (KHz) = 400/480 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 70% Max Throughput | SNR1 (dB) @ 70% Max Throughput | D1 (dB) | SNR2 (dB) @ 70% Max Throughput | D2 (dB) |
| Test 2-1 | 4 (66) | TDLA10-200 | -3.1 | -2.9 | 0.2 | -2.9 | 0.2 |
|  |  | TDLD10-200 | -7.3 | -7.1 | 0.2 | -7.1 | 0.2 |
| Test 2-2 | 13 (16) | TDLA10-200 | 5.1 | 5.6 | - | 5.6 | - |
|  |  | TDLD10-200 | 0.4 | 0.9 | - | 0.9 | - |

Observation 3: Referring to Table 2.1, and considering SCS/CBW = 480 KHz/400 MHz with full and partial PRB allocation for MCS  4 and 13, respectively, with CPE for PN compensation, we note that

- Only MCS 4 with full PRB allocation is testable with TDLA/D10-200

Observation 4: As a general conclusion, the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation are

- 120 KHz/100 MHz (66 PRBs): MCS 17 under TDLD30-200

- 480 KHz/400 MHz (66 PRBs): MCS 4 under TDLD30-200

## 2.2.3 Reference SNRs at 30% of the peak Throughput

In this section, we evaluate the reference SNRs at 30% of the peak throughput for both 120 KHz and 480 KHz SCS considering a full PRB allocation (66) or a partial PRB allocation (16), respectively, for MCS 13. The results have been gathered in Tables 3.1 and 3.2 where CPE compensation only was used.

Table 3.1 - Reference SNR values whether PN has been considered or not for SCS 120 KHz/ CBW 100 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 100/120 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 3-1 | 13 (66) | TDLA30-200 | -1.3 | -1.1 | 0.2 | -1.1 | 0.2 |
|  |  | TDLA30-650 | -1.5 | -1.3 | 0.2 | -1.2 | 0.3 |
|  |  | TDLD30-200 | -5.0 | -4.8 | 0.2 | -4.7 | 0.3 |

Table 3.2 - Reference SNR values whether PN has been considered or not for SCS 480 KHz/ CBW 400 MHz for MCS 13.

|  | CBW (MHz) / SCS (KHz) = 400/480 |  |  | PN @ RX (UE) only – CPE compensation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | MCS (PRB) | Propagation condition | NO PN SNR0 (dB) @ 30% Max Throughput | SNR1 (dB) @ 30% Max Throughput | D1 (dB) | SNR2 (dB) @ 30% Max Throughput | D2 (dB) |
| Test 3-2 | 13 (16) | TDLA10-200 | -0.2 | 5.6 | - | 5.6 | - |
|  |  | TDLD10-200 | -5.0 | -4.6 | 0.4 | -4.6 | 0.4 |

Observation 5: Referring to Tables 3.1 and 3.2, when considering performance metric at 30% of the peak throughput, for 120 KHz/100 MHz and 480 KHz/400 MHz using MCS 13, it has been noted that

- 120 KHz/100 MHz (66 PRBs): MCS 13 can be tested with TDLA30-650

- 480 kHz/400 MHz (16 PRBs): MCS 13 can be tested with TDLD10-200

# 3	Summary

In this paper, we provided some initial simulation results that should help in the definition of the PDSCH requirements. In the following, some useful observations that can be further discussed.

Observation 1: Referring to Table 1.1, and considering SCS/CBW = 120 KHz/100 MHz with full PRB allocation for MCS (4 , 13 and 17) and CPE for PN compensation, we note that

- MCS 4 and MCS 13 are testable with TDLA30-650

- MCS17 is testable with TDLD30-200

Observation 2: Referring to Table 1.2, and considering SCS/CBW = 120 KHz/100 MHz with partial PRB allocation for MCS (17 and 20) and CPE for PN compensation, we note that

- MCS20 is testable with TDLD30-200 when considering 16 PRBs out of 66.

Observation 3: Referring to Table 2.1, and considering SCS/CBW = 480 KHz/400 MHz with full and partial PRB allocation for MCS  4 and 13, respectively, with CPE for PN compensation, we note that

- Only MCS 4 with full PRB allocation is testable with TDLA10-200

Observation 4: As a general conclusion, the max MCS that satisfy the following criteria: Performance degradation due to PN is less than 1 dB with CPE compensation are

- 120 KHz/100 MHz (66 PRBs): MCS 17 under TDLD30-200

- 480 KHz/400 MHz (66 PRBs): MCS 4 under TDLD30-200

Observation 5: Referring to Tables 3.1 and 3.2, when considering performance metric at 30% of the peak throughput, for 120 KHz/100 MHz and 480 KHz/400 MHz using MCS 13, it has been noted that

- 120 KHz/100 MHz (66 PRBs): MCS 13 can be tested with TDLA30-650

- 480 kHz/400 MHz (16 PRBs): MCS 13 can be tested with TDLD10-200

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- 3GPP TR 38.808 v17.0.0, “Study on supporting NR from 52.6 GHz to 71 GHz”, (Release 17).

- R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson.

# Appendix	Simulation assumption

## A.1	Propagation environments as per WF [1]

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |