3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215914

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.3

Source:	Ericsson

Title:	SDR requirements in 52.6 – 71 GHz band

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the SDR requirements for FR2-2. It is worth reminding that SDR has been kept FFS where some related aspects have been gathered in the WF [1].

Issue 3-1-1: Scope of the FR2-2 UE SDR Requirements: - Agreement (GTW August 22, 2022): FFS whether SDR test cases will be specified for FR2-2 pending further checking on the test feasibility on supporting SNR range Issue 3-1-2: Maximum MCS (Scaling Factor = 1) for FR2-2 in the SDR tables - Proposals: - Option 1: MCS 24 for Rank1 and rank2; - Option 2: MCS 27 for Rank 1; Issue 3-1-3: Minimum MCS for FR2-2 in the SDR tables - Proposals: - Option 1: MCS 0 for Rank1 and rank2; - Option 2: Depending on maximum Modulation Format: Maximum Modulation Format; Minimum MCS. 6; 14. 4; 10. 2; 4. Issue 3-1-4: Rank to use for FR2-2 SDR Requirements - Option 1: Rank 1 and Rank 2; - Option 2: Rank 1 only; Issue 3-1-5: Whether to define separate MCS to SNR mapping table for different CBW - Agreement: define different MCS to SNR mapping table for different CBW; Issue 3-1-7: For FR1+FR2-2 CA scenarios, whether to transmit ACK/NACK separately on each CC - Option 1: Yes; - Option 2: No;

# 2	SDR Requirements

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs such as rank 1/2 and QPSK/16QAM/64QAM. RAN4 only need to evaluate the applicability rule for FR2-2 where rank 1 is supported as shown in Table 3.6-1.

Table 3.6-1	MCS indexes for indicated UE capabilities applicable for FR2-2.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 2Rx UE | 1 | 6 | 0.75 | 22 |
|  | 1 | 6 | 0.4 | 14 |
|  | 1 | 4 | 1 | 16 |
|  | 1 | 4 | 0.8 | 16 |
|  | 1 | 4 | 0.75 | 16 |
|  | 1 | 4 | 0.4 | 10 |
|  | 1 | 2 | 1 | 9 |
|  | 1 | 2 | 0.8 | 9 |
|  | 1 | 2 | 0.75 | 9 |
|  | 1 | 2 | 0.4 | 4 |

Proposal 1: Based on the simulation results in [], do not define SDR requirements in FR2-2

# 3	Summary

In this paper we tried to contribute with the following proposal for further discussions:

Proposal 1: Based on the simulation results in [2], do not define SDR requirements in FR2-2.

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

- R4-2215915, “Simulation results for SDR requirements in 52.6 – 71 GHz band”, Ericsson.