3GPP TSG-RAN WG4 Meeting#104-bis-e	R4-2215950

# Electronic meeting, 10 – 19 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | 15.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Carrier resource grid mapping to channel raster and use of UE-specific bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are different interpretations and understanding of the parameter carrierBandwidth in SIB1 in the field and whether the UE specific (regular) channel bandwidths configured by the IE ServingCellConfig have to be located on the channel raster (in practice the latter is no issue for FR2 since channel rasters are SCS based). The CA channel spacing with UE-specific channel bandwidths is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the carrier resource grid with size as indicated by carrierBandwidth in SIB1 shall be mapped to the channel raster. The CA channel spacing with UE-specific channel bandwidths is specified. Clause 5.3.1: the specification of the channel bandwidth is aligned with 38.104 not to suggest that there are multiple carriers per cell. Clause 5.3.4: the requirements apply for the channel bandwidth applied by the UE (initial access) or by BS configuration. Clause 5.4.2.1: clarification of the RF channel position and relation to the carrier resource grid. Clause 5.4.2.2: clarification that the carrier resource grid with size as indicated by carrierBandwidth in signaling of common parameters (including SIB1) shall be mapped to the channel raster for at least one numerology. The defintions of nPRB and k are corrected. Clause 5.4A.1: the CA channel spacing for CA configurations with UE-specific channel bandwidths (configured by the BS) is specified thus defining contiguous and non-contiguous CA configurations with UE-specific bandwidths. The existing specification of nominal CA spacing is reused consistent with legacy CA operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear definitions of configurations lead to interoperability problems and UE malfunction in the field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.3.4, 5.4.2.1, 5.4.2.2, 5.4A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes w r t the corresponding RAN4#104 submission marked ‘Ericsson2’ |  |  |  |  |  |  |  |  |

< start of changes >

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier or parts thereof in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple bandwidth parts to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

< text omitted >

### 5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth [13] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3 for the corresponding UE channel bandwidth applied.

### 5.3.5	Channel bandwidth per operating band

< text omitted >

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of the carrier resource grids of the RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequency is designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [2016667...3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel position maps to a resource element on the carrier resource grid. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in Clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on channel raster and the frequency location of the subcarrier of the corresponding resource element of the carrier resource grid of the RF channel is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the carrier resource grid of the RF channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRB mod 2 = 0 | NRB mod 2 = 1 |
| --- | --- | --- |
| Resource element index k | 0 | 6 |
| Physical resource block number nPRB |  |  |

nRB is the PRB index, k is the resource element index within the PRB, and NRB is the carrier bandwidth given by carrierBandwidth indicated in downlinkConfigCommon or uplinkConfigCommon for the relevant transmission direction and numerology. The value range of the said carrierBandwidth parameter is (1..275) in number of PRBs as specified in [13].

#### 5.4.2.3	Channel raster entries for each operating band

< text omitted >

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with 60kHz channel raster:

with

n = µ0 – 2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz, o is the largest  value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1, and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said  value, with  as defined in TS 38.211 [9].

The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band contiguous carrier aggregation with UE specific channel bandwidth and location configurations in adjacent component carriers, the frequency separation between the center frequencies of the UE specific channel bandwidths shall be less than or equal to the nominal CA channel spacing specified above with the UE specific channel bandwidth replacing BWChannel(j). The center frequency of a UE specific channel bandwidth for each numerology and transmission direction is the frequency location of the subcarrier of the resource element determined according to Table 5.4.2.2-1 but with NRB the carrier bandwidth given by carrierBandwidth configured in ServingCellConfig. In case there is no UE specific channel bandwidth and location configurations in the adjacent carrier the RF reference frequency of the adjacent channel is used for determining the frequency separation.

For intra-band non-contiguous carrier aggregation, the channel spacing between two NR component carriers in different sub-blocks or frequency separation between UE specific channel bandwidth and location configurations in component carriers shall be larger than the nominal channel spacing defined in this clause.

## 5.5	Configurations

< end of changes >