3GPP TSG-RAN WG4 Meeting #104-bis 	R4- 2215959

Electronic Meeting, October 10 – 19, 2022

Source:		Ericsson

Title:			Proposal for optimization of EMC test configurations

Agenda item:		6.16.2

Document for:	Discussion

# Introduction

Current testing of EMC requirements as specified in TS 37.113 [1] for MSR BS is increasingly complex and considers many test configurations, due to various possible RAT combinations.

In this contribution we want to present an initial proposal on how to achieve the optimization of the test configurations.

# Discussion

## 3GPP standards situation

## According to TS 37.113 [1], an MSR BS Base station is “characterized by the ability of its receiver and transmitter to process two or more carriers in common active RF components simultaneously in a declared RF bandwidth, where at least one carrier is of a different RAT than the other carrier(s).”

Observation 1: One characteristic of MSR BS is the ability of receiver and transmitter to process two or more carriers in common active RF components simultaneously.

Standards such as TS 37.113 [1] defines the MSR BS test configurations that shall be used for demonstrating EMC conformance. The optimization of EMC test configurations regarding MSR BS will be discussed in the following section.

## Optimization of EMC Test Configurations for MSR BS

In table 1 we present a summary of the different Capability Sets associated to the test configurations in TS 37.141 [2]. The table also shows a summary of the RAT combinations currently used for EMC testing.

In view of the observations presented in R4-2215958 [3], we consider optimization in the EMC testing can be achieved by reducing the number of RAT combinations defined in some Capability Sets. As a result of this reduction, it will be possible also to reduce the number of Test Configurations considered for EMC testing in an MSR environment. Considering the proposal in [3], the EMC test optimization is summarized in table 2.

The optimization consists of two steps. The first step is to make optimization based on the existing RATs in one Capability Set, e.g., CS12, which contains GSM, WCDMA, and NB IoT SA. According to the proposal in [3], we can use NB IoT SA to cover GSM. Therefore, the RATs in CS12 can be reduced to WCDMA and NB IoT SA, which corresponding to CS10. Furthermore, the second step in table 2: if the MSR BS supports LTE (and/or NR, NB IoT SA in other cases), we can use LTE to cover WCDMA. In this case, the RATs under consideration can be further optimized to LTE and NB IoT, which corresponding to CS11. In EMC test cases of both emission and immunity, the test configuration is TC11. If the MSR BS does not support LTE, CS10 (TC10) is used. The rest 18 Capability Sets can be reduced by analogy to CS1, CS2, CS4, CS5, CS8, CS10, CS11, CS16, and CS17 (step 1), and furthermore to CS2, CS8, CS11, and CS17 (step 2).

It is worth to mention that these CSs (as well as the corresponding TCs) are already defined in TS 37.141 [2]. Therefore, our goal is to reach optimization by using existing TCs and not creating a new TC. In addition, our proposal covers at least one wideband and one narrowband active RF chain.

Table 1. Summary of Capability sets and Supported RATs combinations for MSR BS

| Capability Set | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | CS7 | CS8 | CS9 | CS10 | CS11 | CS12 | CS13 | CS14 | CS15 | CS17 | CS16 | CS18 | CS19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR Configuration | WCDMA | LTE+IoT (opt) | WCDMA + LTE+IoT (opt) | GSM + WCDMA | GSM + LTE+IoT (opt) | GSM + WCDMA + LTE GSM + WCDMA GSM + LTE WCDMA + LTE | GSM + WCDMA GSM + LTE+ IoT (opt) LTE+ WCDMA + IoT (opt) | IoT SA | GSM + IoT SA | WCDMA + IoT SA | LTE + IoT SA | GSM + WCDMA + IoT SA GSM + IoT SA WCDMA + IoT SA GSM+WCDMA | GSM + LTE + IoT SA GSM + IoT SA LTE + IoT SA GSM+LTE | WCDMA + LTE + IoT SA WCDMA+ IoT SA LTE + IoT SA WCDMA + LTE | GSM + WCDMA +IoT GSM + LTE + IoT WCDMA + LTE + IoT SA GSM + IoT SA WCDMA + IoT SA LTE + IoT SA GSM + WCDMA GSM + LTE WCDMA + LTE | LTE+ NR+ IoT(opt) LTE + IoT SA+ IoT(opt) NR + IoT SA + IoT(opt) LTE+ NR+ IoT SA+IoT(opt) | LTE+ NR+ IoT (opt) | GSM + LTE+ IoT (opt) GSM + NR+ IoT (opt) LTE + NR + IoT (opt) GSM + LTE+ NR+ IoT (opt) | WCDMA + LTE+ IoT (opt) WCDMA + NR+ IoT (opt) LTE + NR + IoT (opt) WCDMA + LTE+ NR+ IoT (opt) |
| Step 1 optimization | WCDMA | LTE | LTE | GSM + WCDMA | LTE + IoT-IN/NG(If support) | GSM + LTE | LTE + IoT-IN/NG(If support) | IoT-SA | IoT-SA | WCDMA + IoT-SA | LTE + IoT-SA | WCDMA + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE+ NR+ IoT-SA | LTE+ NR+ IoT-IG/NG(If support) | LTE+ NR+ IoT-IG/NG(If support) | LTE+ NR+ IoT-IG/NG(If support) |
|  | CS1 | CS2 | CS2 | CS4 | CS2 | CS5 | CS2 | CS8 | CS8 | CS10 | CS11 | CS10 | CS11 | CS11 | CS11 | CS17 | CS16 | CS16 | CS16 |
| Step 2 optimization | LTE | LTE | LTE | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | IoT-SA | IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE+ NR+ IoT-SA | LTE+ NR+ IoT-SA | LTE+ NR+ IoT-SA | LTE+ NR+ IoT-SA |
|  | CS2 | CS2 | CS2 | CS11 | CS11 | CS11 | CS11 | CS8 | CS8 | CS11 | CS11 | CS11 | CS11 | CS11 | CS11 | CS17 | CS17 | CS17 | CS17 |

Table 2. Proposal for EMC Test Configuration Optimization

Note: For CS5, CS7, and CS18, in the first step, optimization is done based on that IoT-IN/NG is supported. If IoT-IN/NG is not supported, GSM should be kept, i.e., the original CS should be kept as the result of step 1 optimization.

# Conclusion

In this contribution a proposal on EMC test reduction is presented. Based on our observations, we propose:

Proposal 1: To agree with using the following capability sets: CS1, CS2, CS4, CS5, CS8, CS10, CS11, CS16, and CS17 defined in TS 37.141 [2] for EMC testing of MSR BS, as presented in this contribution. Furthermore, the capability sets can be narrow down to CS2, CS8, CS11, and CS17 if the MSR BS supports NR, LTE and NB IoT SA.

# References

3GPP TS 37.113. “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)”

3GPP TS 37.141. “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing”

R4-2215958. “Discussion of BS EMC Enhancement for NR and LTE”