3GPP TSG-RAN WG4 Meeting #104-bis-e                                                      R4-2215975

Electronic Meeting, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 36.718-02-01 to add CA_7C-20A-32A

Agenda item:	7.1.3.2

Document for:	Approval

# Background

This contribution provides a text proposal for LTE CA_7C-20A-32A with UL configurations 7C and 7A-20A as requested in RAN4#104-e.

# Reference

# Text Proposal

##### ---Start of changes---

### 5.4.x	CA_7C-20A-32A

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 32 |  | N/A |  | 1452 MHz | – | 1496 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_7C-20A-32A | CA_7C CA_7A-20A | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 5.3.x.2	Co-existence studies

Coexistence for CA_7-20 and CA_7 has already been specified in TS 36101.

#### 5.3.x.3	∆TIB and ∆RIB values

Relaxation values for CA_7-20-32 have already been specified in TS 36101.

#### 5.3.x.4	REFSENS Requirements

No additional REFSENS relaxation required compared to CA_7A-20A-32A.

##### ---End of changes---