3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215976

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Discussion on NTN demod general

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.1

Document for:	Discussion

Background

During RAN#104-e meeting, WF [1] for NTN demodulation requirements – general and PDSCH was approved. In this contribution, we share our views about general issues for NTN demodulation requirements.

Discussion

Channel model for NTN-TDLC (LOS)

- Agreement: - Assuming elevation angle: 30 degree to decide K_offset and K-factor - DS = [3.5ns], Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots]

In last meeting, there is still open issue that the delay spread and the K_offset for the channel model for NTN-TDLC (LOS) need companies for further checking. The values are proposed and agreed as baseline in last meeting as the compromise way. Generally, we are OK to confirm to use that values, but we think maybe it is better to use an integer value the delay spread, e.g. 4ns. However, considering the simulation effort and the fact that the deadline of this WI is coming, we don’t have strong view about that.

Confirm to use DS = 3.5ns and K_offset = 8 slots for the channel model NTN-TDLC (LOS).

Based on clause 7.7.6 of TS 38.901, the new NTN-TDL-C channel model with the LOS K_factor changed is calculated as following:

- Calculate Kmodel=P1,modelLOS-10log10n=1N10Pn,model10 = 9.99960 dB

- Calculate Pn,model=Pn,model-Kdesired+Kmodel, the scaled power for each tap is shown as the following table, with the first tap power is normalized to 0dB:

| Tap # | Delay | Scaled Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6024 | LOS path |
|  | 0 | -8.8768 | Rayleigh |
| 2 | 14.8124 | -21.6318 | Rayleigh |

- Calculate the RMS delay spread after the K-factor adjustment DSRMS = 1.21917 ns

- Divide the delays by that value to obtain DS = 1, the final channel model with delay spread re-normalized is shown as following table:

| Tap # | Normalized delay | Scaled Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6024 | LOS path |
|  | 0 | -8.8768 | Rayleigh |
| 2 | 12.1496 | -21.6318 | Rayleigh |

The new NTN-TDL-C channel model with the changed LOS K_factor 8.05 is shown as following:

| Tap # | Normalized delay | Scaled Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6024 | LOS path |
|  | 0 | -8.8768 | Rayleigh |
| 2 | 12.1496 | -21.6318 | Rayleigh |

For the further quantization based on the procedure of the legacy TDL channel model simplification, the normalized delay should be quantization to the delay resolution 5ns. From our understanding, this procedure is to facilitate testing from the TE point of view. Considering that we only have two or three taps for the NTN-TDL channel model to be used, to make the channel model more accurate, we propose to reduce the delay resolution to 2ns for NTN TDL channel model simplification. We think it is feasible about the TE implementation complexity.

Reduce the delay resolution to 2 ns for NTN TDL channel model simplification.

Proposals

In this contribution, we discuss on general issues for NTN demodulation requirements. Our observations and proposals are:

- Confirm to use DS = 3.5ns and K_offset = 8 slots for the channel model NTN-TDLC (LOS).

- The new NTN-TDL-C channel model with the changed LOS K_factor 8.05 is shown as following:

| Tap # | Normalized delay | Scaled Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6024 | LOS path |
|  | 0 | -8.8768 | Rayleigh |
| 2 | 12.1496 | -21.6318 | Rayleigh |

Reduce the delay resolution to 2 ns for NTN TDL channel model simplification.

Reference

R4-2214386, WF for NTN demodulation requirements - general and PDSCH, RAN4#104-e, Qualcomm Incorporated