3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215982

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Simulation results on satellite NTN demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.2.1

Document for:	Information

Background

During RAN4#104-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our updated simulation results about the NTN PUSCH demodulation requirements.

Simulation

PUSCH

Here we provide the updated simulation results for PUSCH for alignment, as shown in Table 2.1-2, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for NTN normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 group hopping and sequence hopping are disabled for transform precoding enabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low, NTN-TDLC[3.5]-200 Low (K_factor = 8.05) |
| Test metric |  | SNR@70%maximum throughput |

Table 2.1-2 Ideal simulation results for PUSCH (Transform precoding disabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA |  | NTN-TDLC |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 1.21 | 1.22 | 2.39 | 2.44 |
| 2 | 15 / 5 | 1x2 | -2.89 | -2.89 | -2.44 | -2.44 |
| 3 | 30 / 10 | 1x1 | 0.89 | 0.91 | 2.27 | 2.27 |
| 4 | 30 / 10 | 1x2 | -3.06 | -3.06 | -2.68 | -2.66 |

Table 2.1-2 Ideal simulation results for PUSCH (Transform precoding enabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA |  | NTN-TDLC |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 1.81 | 1.81 | 2.40 | 2.44 |
| 2 | 15 / 5 | 1x2 | -2.64 | -2.62 | -2.46 | -2.44 |
| 3 | 30 / 10 | 1x1 | 1.73 | 1.73 | 2.30 | 2.32 |
| 4 | 30 / 10 | 1x2 | -2.77 | -2.76 | -2.66 | -2.66 |

UL timing adjustment

Here we provide the updated simulation results for UL timing adjustment for alignment, as shown in Table 2.2-3, based on simulation assumption as shown in Table 2.2-1 and Table 2.2-2.

Table 2.2-1 Simulation assumption for UL timing adjustment (part 1)

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | CSRS = 5, BSRS =0, for 20 RB |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| NOTE 1. The transmission of SRS is optional. The transmission comb is configured as KTC = 2. The SRS periodic is configured as TSRS = 10 for 15kHz SCS and 20 for 30kHz SCS respectively. |  |  |

Table 2.2-1 Simulation assumption for UL timing adjustment (part 2)

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 Low |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 0.04 S-1 for 15kHz SCS and 0.08 S-1 for 30kHz SCS |

Table 2.2-3 Ideal simulation results for UL timing adjustment

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 2.56 | 2.58 |
| 2 | 15 / 5 | 1x2 | -1.88 | -1.86 |
| 3 | 30 / 10 | 1x1 | 1.66 | 1.92 |
| 4 | 30 / 10 | 1x2 | -2.29 | -2.10 |

PUSCH repetition type A

Here we provide the updated simulation results for PUSCH repetition type A for alignment, as shown in Table 2.3-2, based on simulation assumption as shown in Table 2.3-1.

Table 2.3-1 Simulation assumption for PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low |
| Test metric |  | SNR@70%maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 2.3-2 Ideal simulation results for PUSCH repetition type A

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | -3.73 | -3.76 |
| 2 | 15 / 5 | 1x2 | -7.56 | -7.59 |
| 3 | 30 / 10 | 1x1 | -4.72 | -4.71 |
| 4 | 30 / 10 | 1x2 | -8.25 | -8.23 |

Conclusion

In this contribution, we provide our updated simulation results on NTN PUCCH demodulation requirements.

Reference

R4-2214387, WF on NTN SAN demodulation requirements, RAN4#104-e, Huawei, HiSilicon