3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215988

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Simulation results on satellite NTN demod PRACH

Source: 	Huawei, HiSilicon

Agenda item:	4.2.7.2.3

Document for:	Information

Background

During RAN4#104-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our updated simulation results about the NTN PRACH demodulation requirements.

Simulation

PRACH

Here we provide the updated simulation results for PRACH for alignment, as shown in Table 2.1-3 and Table 2.1-4, based on simulation assumption as shown in Table 2.1-1 and Table 2.1-2.

Table 2.1-1 Simulation assumption for PRACH (Part1)

| Parameter | Value |
| --- | --- |
| Preamble format | 0/B4/C2/2 |
| SCS | 1.25kHz for long preamble format 15kHz and 30kHz SCS for short preamble format (LRA=139) |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | AWGN, NTN-TDLA100-200 Low |
| Time error tolerance | 0.522μ+Tdelay where Tdelay is the largest delay of the propagation channel |
| Frequency offset | 0Hz for AWGN, 200Hz for fading channel |
| Test metric | false alarm probability<0.1%, detection probability>99% |

Table 2.1-2 Simulation assumption for PRACH (Part2)

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| 2 | 1.25 | 13 | 22 | 32 |
| B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Table 2.1-3 Ideal simulation results for PRACH with long format

| Case Number | Antenna configuration | Propagation conditions | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0 | 2 |
| 1 | 1x1 | AWGN | -14.22 | -19.10 |
| 2 | 1x1 | NTN-TDLA100-200 | -0.06 | -12.42 |
| 3 | 1x2 | AWGN | / | -21.54 |
| 4 | 1x2 | NTN-TDLA100-200 | -8.73 | -17.72 |

Table 2.1-4 Ideal simulation results for PRACH with short format

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | Propagation conditions | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | B4 | C2 |
| 1 | 15 | 5 | 1x1 | AWGN | -13.49 | -10.07 |
| 2 | 15 | 5 | 1x1 | NTN-TDLA100-200 | -4.86 | 0.18 |
| 3 | 15 | 5 | 1x2 | AWGN | / | / |
| 4 | 15 | 5 | 1x2 | NTN-TDLA100-200 | -10.25 | -6.34 |
| 5 | 30 | 10 | 1x1 | AWGN | -14.01 | -10.29 |
| 6 | 30 | 10 | 1x1 | NTN-TDLA100-200 | -5.52 | -1.83 |
| 7 | 30 | 10 | 1x2 | AWGN | / | / |
| 8 | 30 | 10 | 1x2 | NTN-TDLA100-200 | -11.04 | -7.56 |

Conclusion

In this contribution, we provide our updated simulation results on NTN PRACH demodulation requirements.

Reference

R4-2214387, WF on NTN SAN demodulation requirements, RAN4#104-e, Huawei, HiSilicon