3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215995

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Discussion on BS coverage enhancement demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	4.4.2.1

Document for:	Discussion

Background

During RAN4#104-e meeting, PUSCH demodulation performance of Rel-17 NR coverage enhancement was discussed and the fruitful agreements are noted in [1]. In this contribution, we share our views about the BS coverage enhancement PUSCH demodulation requirements.

Discussion

Simulation alignment

Currently the simulation results are still not aligned well, especially for the PUSCH TBoMS performance requirements. If finally the simulation results are still not aligned in this meeting, the rule defined in Rel-15 R4-1904713 should be reused, i.e. if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available.

If finally the simulation results are still not aligned in this meeting, the rule defined in Rel-15 R4-1904713 should be reused, i.e. if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available.

Number of HARQ process

In last meeting, there is open issue that whether the number of HARQ should be specified for PUSCH JCE performance requirements. This issue is proposed since there is large difference between different number of HARQ process for the distance of the initial transmission and the re-transmission therefore the performance may be diverse. Here we share our simulation results as following Table 2.2-1 for different number of HARQ process with the simulation assumption specified in the simulation result collection [2].

Table 2.2-1 Simulation results for PUSCH JCE for different number of HARQ process

| Duplex Mode | Antenna Configuration | DMRS | HARQ process number |  |  | Diff |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2 | 4 | 8 |  |
| TDD | 1T2R | DMRS 1+0 | -6.29 | -5.95 | -5.66 | 0.63 |
|  |  | DMRS 1+1 | -6.6 | -6.24 | -5.97 | 0.63 |
|  | 1T4R | DMRS 1+0 | -9.5 | -9.29 | -9.02 | 0.48 |
|  |  | DMRS 1+1 | -9.97 | -9.72 | -9.44 | 0.53 |
|  | 1T8R | DMRS 1+0 | -12.33 | -12.19 | -11.94 | 0.39 |
|  |  | DMRS 1+1 | -12.93 | -12.79 | -12.54 | 0.39 |

There is maximum 0.63dB difference that can be negligible between different number of HARQ process.

From our understanding, the number of HARQ process is up to BS implementation. With such negligible performance difference for different number of HARQ process, we don’t think it necessity to explicitly specify the number of HARQ process for PUSCH JCE performance requirements.

Do not explicitly specify the number of HARQ process for PUSCH JCE performance requirements.

Proposals

In this contribution, we discuss on BS PUSCH demodulation requirements for coverage enhancement. Our observations and proposals are:

- There is maximum 0.63dB difference that can be negligible between different number of HARQ process.

- If finally the simulation results are still not aligned in this meeting, the rule defined in Rel-15 R4-1904713 should be reused, i.e. if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available.

Do not explicitly specify the number of HARQ process for PUSCH JCE performance requirements.

Reference

R4-2214315, Email discussion summary for [104-e][326] NR_cov_enh_Demod, RAN4#104-e, Moderator (China Telecom)

R4-2211780, Summary of simulation results for PUSCH coverage enhancements, RAN4#104-e, China Telecom