3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216003

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Simulation results on UE FeMIMO demod HST-SFN

Source: 	Huawei, HiSilicon

Agenda item:	4.5.3.1.1

Document for:	Information

Background

During RAN4#104-e meeting, WF [1] on UE demodulation and CSI requirements for FeMIMO was approved. In this contribution, we provide our updated simulation results about the FeMIMO HST-SFN deployment demodulation requirements.

Simulation

PDSCH requirement for SFN scheme A with Single Carrier

Here we provide the updated simulation results for PDSCH requirement for SFN scheme A with Single Carrier for alignment, as shown in Table 2.1-2 and Figure 2.1-1, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PDSCH requirement for SFN scheme A with Single Carrier

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2, 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot patterns |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Table 2.1-2 Simulation results for PDSCH requirement for SFN scheme A with Single Carrier

| Case Number | Duplex | Maximum Doppler (Hz) | Antenna configuration | MCS | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 870 | 2x2 | 13 | 10.74 |
| 2 | FDD | 870 | 2x2 | 17 | 14.86 |
| 3 | FDD | 870 | 2x4 | 13 | 8.14 |
| 4 | FDD | 870 | 2x4 | 17 | 12.35 |
| 5 | TDD | 1667 | 2x2 | 13 | 10.83 |
| 6 | TDD | 1667 | 2x4 | 17 | 14.80 |
| 7 | TDD | 1667 | 2x4 | 13 | 8.27 |
| 8 | TDD | 1667 | 2x4 | 17 | 12.32 |

Figure 2.1-1 Simulation results for PDSCH requirement for SFN scheme A with Single Carrier

PDSCH requirement for SFN scheme B with Single Carrier

Here we provide the initial simulation results for PDSCH requirement for SFN scheme B with Single Carrier for alignment, as shown in Table 2.2-2 and Figure 2.2-1, based on simulation assumption as shown in Table 2.2-1.

Table 2.2-1 Simulation assumption for PDSCH requirement for SFN scheme B with Single Carrier

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2, 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern |  | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot patterns |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |

Table 2.2-2 Simulation results for PDSCH requirement for SFN scheme B with Single Carrier

| Case Number | Duplex | Maximum Doppler (Hz) | Antenna configuration | MCS | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | FDD | 870 | 2x2 | 17 | 13.22 |
| 2 | FDD | 870 | 2x4 | 17 | 10.42 |
| 3 | TDD | 1667 | 2x2 | 17 | 13.07 |
| 4 | TDD | 1667 | 2x4 | 17 | 10.30 |

Figure 2.2-1 Simulation results for PDSCH requirement for SFN scheme B with Single Carrier

Conclusion

In this contribution, we provide our updated simulation results on FeMIMO HST-SFN demodulation requirements.

Reference

R4-2214397, WF on UE demodulation and CSI requirements for FeMIMO, RAN4#104-e, Samsung