3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216007

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Discussion and simulation results on UE FeMIMO CSI mTRP

Source: 	Huawei, HiSilicon

Agenda item:	4.5.3.2

Document for:	Discussion

Background

During RAN4#104-e meeting, WF [1] on UE demodulation and CSI requirements for FeMIMO was approved.

- Sub-topic 3-1: Test Setup of CSI reporting requirement for m-TRP transmission - Issue 3-1-1: Number of CSI-RS port Agreement: - 8 ports per TRP - Issue 3-1-2: Beamforming Model Agreement: - Consider beam-steering model into the multi-TRP PMI reporting test set-up - Use the step sizes specified in Table B.2.3.2.3A-1 with independent start value for each TRP. - Issue 3-1-3: Gamma value Agreement: - [1.6] for 8 ports as baseline. The gamma value can be updated based on simulation results in next meeting.

In this contribution, we share our views about the UE FeMIMO mTRP CSI requirements.

Discussion

Test Setup of CSI reporting requirement for m-TRP transmission

Gamma value

The only remained issue is the gamma value for multi-TRP CSI reporting requirements. Based on our simulation results in our accompanying contribution [2], the gamma value is larger than 2 for all configurations. Also considering the impairment factors due to UE implementation, we are OK to confirm to use the gamma value 1.6 for 8 ports.

Confirm to use the gamma value 1.6 for 8 ports.

Proposals

In this contribution, we discuss on UE CSI requirements for FeMIMO mTRP. Our observations and proposals are:

- Confirm to use the gamma value 1.6 for 8 ports.

Reference

R4-2214397, WF on UE demodulation and CSI requirements for FeMIMO, RAN4#104-e, Samsung

R4-2216007, Simulation results on UE FeMIMO CSI mTRP, RAN4#104bis-e, Huawei, HiSilicon