3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216067

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence simulation assumption for ATG scenario

Agenda Item:	6.13.2

Document for:	Approval

Introduction

In RAN4#104 meeting, a way forward [1] on the ATG co-existence study was approved with some consensus and agreements. However, we still have some open issues. In this paper, we’d like to provide our views on network layout, antenna model and so on.

Discussion on antenna model of ATG BS

Firstly, we’d like to clarify the difference between TN scenario and ATG scenario. Since we have agreed that the UE altitude of commercial aircraft flight can be from lower boundary (3~7km) to upper boundary 10km and ATG network need to coverage 100km, that means the upper boundary of vertical coverage range need to reach 85º~ 89º. Generally, the BS vertical coverage range is about 25º, so the lower boundary of vertical coverage range can be 60º~ 64º. (Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as in ITU-R M.2101.) Thus, the corresponding mechanical up-tilt can be 17.5º~ 13.5º. For maximum coverage angle in the horizontal plane, +/- 60º can be reused for ATG BS.

Proposal 1: For maximum coverage angle in the horizontal plane, +/- 60º can be reused for ATG BS.

Proposal 2: In order to reuse the existing requirements and industry eco-system, 25º maximum coverage angle range in the vertical plane can be reused for ATG BS.

Proposal 3: For ATG BS mechanical up-tilt angle, we can further choose one value from 17.5º~ 13.5º range considering the lower boundary of commercial aircraft altitude and the cell range.

Figure 1 Illustration between the coverage and mechanical up-tilt angle

Discussion on the network layout

Referring to the discussion in last meeting, RAN4 need to assume the lower boundary of the commercial aircraft altitude from the candidate range 3km~7km. Maybe the median value 5km can be a compromise.

Proposal 4: it’s proposed to assume 5km as the lower boundary of the commercial aircraft altitude.

For coverage range, the candidate values can be 100km, 150km, 200km and 300km. If we plan to assume multiple ATG BS sites for inter-cell interference, we can assume 100km coverage range for simulation to achieve higher inter-cell interference. And the narrower coverage range is, the better capability network can achieve. Given the assumption above, we can get the one ATG site with three sectors as below.

Figure 2 Illustration on one ATG site with three sectors

For ATG network, 7 sites / 3 sectors for each site with wrap around can be assumed as below.

Figure 3 Illustration on ATG network with 7 sites wrap around

As the scenario of ATG BS non-co-located with TN BS is included in last meeting, considering uncoordinated scenario, the heterogeneous scenario with both ATG network and Macro terrestrial network is shown as below.

Figure 4 Illustration on the heterogeneous scenario with both ATG network and Macro terrestrial network

The terrestrial network can be 19 macro sites / 3 sectors per site with wrap around. It’s proposed to drop 19 macro sites only near the ATG BS site or nadir of aircraft. There is no need to fill in the ATG sector by putting all the macro sites.

Proposal 5: it’s proposed to assume the heterogeneous scenario with both ATG network and Macro terrestrial network as table 1, table 2 and figure 4.

Table 1 Terrestrial macro network

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.5 km (3.5GHz) 0.9 km (2GHz) |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 10% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Note: There is no need to fill in the ATG sector by putting all the macro sites. 19 macro sites can be dropped only near the ATG BS site or nadir of aircraft as shown in figure 4. |  |  |  |

Table 2 ATG network

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | hexagonal grid, 7 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance | 150km |  |
| BS antenna height | 25 m (suburban) |  |
| Aircraft distribution (horizontal) | Uniform |  |
| Aircraft altitude | lower boundary, i.e. 5km |  |
| Minimum BS - UE distance (2D) | TBD |  |
| Note 1: ATG network is un-coordinated operation (100% Grid Shift) with TN. The ATG site can be set in one position from A, B, C and D randomly as in figure 4. Note 2: The aircraft can be dropped in the first macro site with 3 sectors randomly as in figure 4. |  |  |

For the blind area, one alternative is to set one more sector where BS orientation is perpendicular to sky. However, other solution is not precluded. More studies are needed.

Discussion on antenna model of TN BS

Referring to clause 6.2.3.2 from NTN TR 38.863, the AAS antenna parameters for TN BS can be baseline.

Table 3 AAS antenna parameters for Macro BS

|  |  | Rural |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921[14] |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |
| 1.7 | Number of supported polarizations, P | 2 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) | 90-100 |
| 1.13 | Mechanical downtilt (degrees) | 3 |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

Proposal 6: it’s proposed to use the AAS antenna parameters for Macro BS in table 3.

Discussion on antenna model for CPE mounted in the aircraft

Since UE orientation aims to the ground, omni antenna assumption for CPE mounted in the aircraft can be excluded. It’s unnecessary to consider the direction which is opposite to the ground.

Proposal 7: Omni antenna assumption for CPE mounted in the aircraft can be excluded.

It’s proposed to adopt the following array antenna model in Table 1. It’s highlighted that the antenna array configuration can be further discussed based on the expected maximum output power and antenna gain of CPE mounted in the aircraft. Based on the discussion in last meeting, there are no conclusion on whether CPE mounted in the aircraft has beam steering capability. RAN4 never assume FR1 UE has the capability of beam steering. Thus, if we assume that FR1 UE has the capability of beam steering, it’s very challenging for RAN4 to specify the UE requirements based on beam steering assumption. Given the objective of this WI is to reuse the existing requirements as soon as possible, the TU budget is limited. Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress.

Proposal 8: Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress as all RF requirements related to the beamforming should be developed under the limited TU. Based on the concerns above, it’s proposed to focus on the CPE mounted in the aircraft without beam steering capability in Rel-18 ATG WI.

Table 4 Array antenna pattern and parameters for CPE mounted in the aircraft

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ,φ=-min12θ-90θ3dB2,SLAv, θ3dB=90°, SLAv=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hθ,φ=-min12φφ3dB2,Am, φ3dB=90°, Am=25dB |
| Combining method for 3D antenna element pattern (dB) | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi GE,max=DE,max-LE |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| (Mg, Ng, M, N, P) | FFS |
| (dv, dh) | (0.5λ, 0.5λ) |
| LE (dB) | 2.0 dB |
| UE orientation | UE orientation is perpendicular to the ground θetilt=90° and φescan=0° |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max. |  |

Proposal 9: It’s recommended to adopt Table 4 UE array antenna pattern and parameters for ATG scenario.

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: For maximum coverage angle in the horizontal plane, +/- 60º can be reused for ATG BS.

Proposal 2: In order to reuse the existing requirements and industry eco-system, 25º maximum coverage angle range in the vertical plane can be reused for ATG BS.

Proposal 3: For ATG BS mechanical up-tilt angle, we can further choose one value from 17.5º~ 13.5º range considering the lower boundary of commercial aircraft altitude and the cell range.

Proposal 4: it’s proposed to assume 5km as the lower boundary of the commercial aircraft altitude.

Proposal 5: it’s proposed to assume the heterogeneous scenario with both ATG network and Macro terrestrial network as table 1, table 2 and figure 4.

Table 1 Terrestrial macro network

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.5 km (3.5GHz) 0.9 km (2GHz) |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 10% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Note: There is no need to fill in the ATG sector by putting all the macro sites. 19 macro sites can be dropped only near the ATG BS site or nadir of aircraft as shown in figure 4. |  |  |  |

Table 2 ATG network

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | hexagonal grid, 7 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance | 150km |  |
| BS antenna height | 25 m (suburban) |  |
| Aircraft distribution (horizontal) | Uniform |  |
| Aircraft altitude | lower boundary, i.e. 5km |  |
| Minimum BS - UE distance (2D) | TBD |  |
| Note 1: ATG network is un-coordinated operation (100% Grid Shift) with TN. The ATG site can be set in one position from A, B, C and D randomly as in figure 4. Note 2: The aircraft can be dropped in the first macro site with 3 sectors randomly as in figure 4. |  |  |

Figure 4 Illustration on the heterogeneous scenario with both ATG network and Macro terrestrial network

Table 3 AAS antenna parameters for Macro BS

|  |  | Rural |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern | TR 38.921[14] |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements |
| 1.7 | Number of supported polarizations, P | 2 |
| 1.8 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V |
| 1.9 | Array Ohmic loss (dB) (Note 2) | 2 |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) | 90-100 |
| 1.13 | Mechanical downtilt (degrees) | 3 |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

Proposal 6: it’s proposed to use the AAS antenna parameters for Macro BS in table 3.

Proposal 7: Omni antenna assumption for CPE mounted in the aircraft can be excluded.

Proposal 8: Assuming beam steering capability for CPE mounted in the aircraft will enlarge the scope of ATG WI and may cause some risks of progress as all RF requirements related to the beamforming should be developed under the limited TU. Based on the concerns above, it’s proposed to focus on the CPE mounted in the aircraft without beam steering capability in Rel-18 ATG WI.

Table 4 Array antenna pattern and parameters for CPE mounted in the aircraft

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ,φ=-min12θ-90θ3dB2,SLAv, θ3dB=90°, SLAv=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hθ,φ=-min12φφ3dB2,Am, φ3dB=90°, Am=25dB |
| Combining method for 3D antenna element pattern (dB) | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi GE,max=DE,max-LE |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| (Mg, Ng, M, N, P) | FFS |
| (dv, dh) | (0.5λ, 0.5λ) |
| LE (dB) | 2.0 dB |
| UE orientation | UE orientation is perpendicular to the ground θetilt=90° and φescan=0° |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max. |  |

Proposal 9: It’s recommended to adopt Table 4 UE array antenna pattern and parameters for ATG scenario.

- References

[1] R4-2215147, WF on UE requirements for ATG, Apple

[2] RP-220962, New WID: Air-to-ground network for NR, CMCC

[3] R4-2214459, WF on co-existence evaluation for ATG, Huawei, HiSilicon