3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216074

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on UE implementation for band plan

Agenda Item:	7.4.2

Document for:	Approval

Introduction

In last meeting, WF [1] on band definition for LTE-based 5G terrestrial broadcast in UHF was approved with the following background information and way forwards.

<Background>

The following band definitions are presented as candidate options by the moderator after the 1st round of email discussion [104-e][128].

- Option 1: Define one large single band 470 – [694/698/702] MHz in addition to multiple regional (smaller) bands

- Option 2: Define one large single band 470 – [694/698/702] MHz, but the UE is only required to support a subset of this band

- Option 3: Define sub-bands corresponding to TV channels (i.e., CH34, 35, 36, etc) but the UE would not be required to support all of them

- Option 4: Define sub-bands of UHF, for example, lower UHF, middle UHF, upper UHF

- Option 5: Some hybrid of these options, or other options entirely

<Way forward/Agreement>:

- The entire 470-[694/698/702] MHz range is covered by one single band or multiple sub-bands.

  - i.e., at least one band starts from 470 MHz and one band ends at 694/698/702 MHz. (The higher band edge is TBD)

- For RAN4#104-bis meeting, companies are encouraged to study the following issues.

  - The feasibility of one large single band in terms of UE implementation in Rx filter, antenna or other components.

  - If one large band is not feasible in UE, what is the maximum singe band bandwidth and minimum number of sub-bands.

    - Discuss also tradeoff between smaller and larger bands.

  - The feasibility of defining a harmonized band/sub-band up to 694/698/702 MHz if the higher band edge 694/698/702 MHz is different among regions.

  - Band(s) plan proposal and motivation

It seems that companies have no common understanding about the UE implementation of RF front-end. In this paper, we’d like to provide our views on this.

Discussion

Based on the discussion in last meeting, companies are encouraged to study the UE implementation in Rx filter, antenna or other components. In addition, companies suggested that it is very mature for DVB-H tuner to handle such wide band. So we checked and learned some UE implementation of DVB-H tuner. The typical RF block diagram for DVB-H tuner is shown below referring to this link [2].

Figure 1 the typical RF block diagram for DVB-H tuner

It’s noted that this direct-conversion DVB-H tuner can cover an input frequency range of 470MHz to 780MHz. It’s observed that there are some difference between DVB-H tuner and RF front end and RFIC from IMT industry. The first one is that the RF front end analog filter is optional. In my understanding, it’s very important to implement the RF front end filter in order to meet the out-of-band blocking requirements. However, it seems that the RF front end filter can be optional for DVB-H receiver.

Proposal 1: RAN4 can discuss whether we need to assume the implementation of RF front end filter or when we consider the band plan for LTE based 5G terrestrial broadcast.

Proposal 2: If the implementation of RF front end filter is assumed, which type of the filter can be assumed, e.g. low-pass filter or band-pass filter?

The second one is that after the LNA, the signal is filtered by an adjustable bandpass tracking filter to reduce interferers, which may be different from the IMT receiver design.

Proposal 3: RAN4 can discuss whether we can assume an adjustable bandpass tracking filter after LNA or we just assume to reuse the current IMT receiver design framework.

As it’s very crowded in smartphone with lots of filters and chips, we can consider whether the filters designed for 600MHz IMT bands can be reused for LTE based 5G terrestrial broadcast. If it’s allowed, it’s very helpful for UE vendors to reduce the cost and space. In addition, it’s very good to reuse the IMT industry for broadcasting service.

Proposal 4: RAN4 can discuss whether the filters designed for 600MHz IMT bands can be reused for LTE based 5G terrestrial broadcast.

Summary

Proposal 1: RAN4 can discuss whether we need to assume the implementation of RF front end filter or when we consider the band plan for LTE based 5G terrestrial broadcast.

Proposal 2: If the implementation of RF front end filter is assumed, which type of the filter can be assumed, e.g. low-pass filter or band-pass filter?

Proposal 3: RAN4 can discuss whether we can assume an adjustable bandpass tracking filter after LNA or we just assume to reuse the current IMT receiver design framework.

Proposal 4: RAN4 can discuss whether the filters designed for 600MHz IMT bands can be reused for LTE based 5G terrestrial broadcast.

- References

[1] R4-2214442, WF on band definition for 5G broadcast in UHF, Nokia, Nokia Shanghai Bell

[2] https://www.maximintegrated.com/en/design/technical-documents/app-notes/3/3847.html