3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216093

Electronic Meeting, 10 October – 19 October 2022

Source:	Ericsson, ZTE

Title:	TP for TR 37.718-21-11: Including band combinations DC_20-41_n78

Agenda item:			5.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_20-41_n78 configurations.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_20-41_n78

### 5.x.1	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-41A_n78A | DC_20A_n78A DC_41A_n78A | CA_20A-41A | n78A |
| DC_20A-41C_n78A | DC_20A_n78A DC_41A_n78A DC_41C_n78A | CA_20A-41C | n78A |

### 5.x.2	Co-existence studies

There are IMD2 impact from UL 20_n78 affecting DL band 41.

There are IMD2 and IMD5 impact from UL 41_n78 affecting DL band 20.

### 5.x.3		∆TIB and ∆RIB values

For DC_20-41_n78, the TIB,c and RIB,c values are reused from DC_20_n41-n78 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-41_n78 | 20 | 0.5 |
|  | 41 | 0.3 |
|  | n78 | 0.8 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-41_n78 | 20 | 0 |
|  | 41 | 0 |
|  | n78 | 0.5 |

### 5.x.4	REFSENS requirements

MSD value band 41 is reused from DC_20A_n41A-n78A.

MSD value band 20 is reused from CA_n28A-n41A-n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-41A_n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
| DC_20A-41C_n78A | 41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 839 | 5 | 25 | 798 | 30.8 | IMD24 |
|  | 41 | 2642 | 10 | 50 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |

---End of changes---

# Reference

[1]		RP-222505, New WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon