3GPP TSG-RAN WG4 Meeting # 104bis-e										R4-2216127

Electronic Meeting, October 10 – October 19, 2022

Source: 	vivo

Title: 	Discussion on UE requirement for multi-Rx DL reception

Agenda Item:	6.8.2.3

Document for:	Discussion

- Introduction

In the last meeting, several agreements on multi-Rx were reached [1]:

Agreement:

- Proposal: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

- Proposal: UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- Proposal: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

- FFS whether the concept of panel should not be explicitly used in core requirements and test configurations.

- FFS whether the single panel should be excluded.

- Further discuss on the on the candidate AoA pairs for setting the UE RF requirement

  - One Fixed AoA1 (e.g. Peak) + Full set AoA2.

  - Multiple AoA1 + Full set AoA2.

  - Fixed offset between the two AoAs, both probes swept simultaneously.

  - Full set AoA1 + Full set AoA2

  - Other solutions are not precluded. Companies are also encouraged to bring the analysis on how to quantify the Refsens value when receiving multiple signals.

In this contribution, we provide our views on how to define the UE requirement to ensure the performance gain of multi-Rx DL reception.

- Discussion

In the last meeting, several baseline of RF requirement of multi-Rx DL reception were achieved, but how to define the requirement under such situation is still not clear. Before we discuss the RF requirement itself, there are several basic issues should be figured out clearly.

- Power imbalance

In FR2 inter-band CA discussion, considering the non-collocated deployment, the impact of power imblance was considered with 1 dB relaxation and verified by setting the EIS of untested band at the 50% spherical coverage. Similarly, the power imbalance also exist in MTRP deployment, should we reflect its impact on RF requirement and verification setting? From our perspective, the answer is NO. It is noted that we have agreed that the RF requirement will be defined based on 2-layer MIMO case, and if large power imbalance exists, the 2-layer MIMO cannot be maintained and NW also not expect UE to perform MIMO operation, in other words, the baseline for RF requirement has excluded the power imbalace case.

Observation 1: UE cannot maintain 2-layer MIMO under large power imbalance and NW also not expect the UE work with MIMO under such situation.

Proposal 1: The impact of power imbalance does not need to be considered in RF requirement and verification for multi-Rx DL reception.

- Reference measurement channel

Different from the Tx requirement, most Rx requirements are defined with reference measurement channel (RMC). The multi-Rx is based on the 2-layer MIMO but there is no MIMO RMC exist in current TS 38.101-2. One way is to define a 2-layer MIMO RMC, then we can get a battery of EIS pair from 2 AoAs that joint throughput meet the MIMO RMC, which also means that the 2 AoA is bonded closer, and Another way is to apply the single carrier RMC to each layer. In the last meeting, we have agreed that:

- For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme isadopted as a baseline.

The single DCI scheme imply that the 2-layer data form 2 TRP comes from one TB, which means that we cannot get the throughput of each layer and only get a combined throughput, so perhaps we have no choice but to define a new RMC

Proposal 2: A new RMC for 2-layer DL MIMO need to be defined for multi-Rx.

- Panel concept

In the last meeting, whether the concept of panel should not be explicitly used was discussed. In our under-standing, until now, there is no clear discription of panel in the spec no matter RAN1 or RAN4 and in fact, the commercial module has different formation, e.g., single aperture for dual polarization or sperate aperture for dual ploarizaiton, etc. The RAN4 spec only need to ensure the UE performance and should not put any extra restriction on UE detailed design.

Another risk to use panel concept in multi-Rx discussion is that we may construct a specific relationship between AoA and panel. This may be ok when only two panel exist, but if the panel numer raises, this relationship also put restrition on UE behavior. For example, when UE equiped with 3 panels but need to map to 2 AoA, one of the panels may be useless. Furthermore, we are hard to identify each panel during the test.

Observation 2: Using panel concept explicitly risks limiting UE design and UE behavior which is not expected.

Proposal 3: Do not use panel concept no matter in RF requirement or test configuration.

Now, we can further discuss how to define the RF requirement. In our understanding, the multi-Rx RF requirement is not to verify “multi-panel” because current commercial UEs are already equiped with more than one antenna module. The key point for this feature is how to verify “simultaneously”. We can first review that whther the traditional method, i.e., min peak EIRP & 50% spherical coverage, can ensure the multi-panel work simultaneously. Unfortunately, as we provided the results in [2], when multiple panels activated simultaneously, the peak and 50% spherical coverage may worse than two panel switching case, whicn means the performance gain can not be reflected or guaranteed by the traditional method.

Figure 1 spherical coverage of 2 panel case in [x]

Figure 2 heat map of 2 panel case in [x]

Observation 3: Define the peak EIS and 50% spherical coverage can not reflect the performance gain that bring from the multi-panel activated simultaneously.

So, we should first figure out what is the performance gain that we should guaranteed when multiple panels activated. From our perspective, the ideal situation is each layer can perform like single carrier and the throughput can be doubled when two panel work simultaneosly, however, considering the mutual impact and non-idel channel characteristic, the performance of each layer will wrose than the single carrier. In other words, the requirement should ensure that when multiple panels activated, the degradation should be with an acceptable range comparing to single carrier case. Based on the analysis above, we propose to define a tolerance to guarantee the performance.

Step-1: Perform the single carrier spherical coverage test, get the EIS_0 which is the top 50% EIS of the whole sphere.

Step-2: Perform multi-Rx spherical coverage test and only verify the top 50% regien of single carrier, then get EIS_1 from AoA1 and EIS_2 from AoA2.

  - Detail of the test setup can be further discussed

Step-3: Calculate the difference in same direction ∆EIS_1 = EIS_0AoA1 - EIS_1 and ∆EIS_2 = EIS_0AoA2 - EIS_2

Step-4: Reselect the AoA1 and AoA2, get a set of ∆EIS_1 and ∆EIS_2.

Step-5: EIS tolerance = max(∆EIS_1, ∆EIS_2) ≤ [TBD] dB

The tolerance depends on at least two perspectives: one is hardware impairment, e.g., mutaul impact between panels, polarization gain loss in single panel behavior, etc. Another is system performance, and the tolerance should ensure the throughput degradation of each AoA is within an acceptable range.

Proposal 4: Define a EIS tolerance = max(∆EIS_1, ∆EIS_2) ≤ [TBD] dB to ensure the performance gain of the multi-Rx DL reception.

- Conclusion

In this contribution, we provide our views on multi-Rx requirement:

Observation 1: UE cannot maintain 2-layer MIMO under large power imbalance and NW also not expect the UE work with MIMO under such situation.

Observation 2: Using panel concept explicitly risks limiting UE design and UE behavior which is not expected.

Observation 3: Define the peak EIS and 50% spherical coverage can not reflect the performance gain that bring from the multi-panel activated simultaneously.

Proposal 1: The impact of power imbalance does not need to be considered in RF requirement and verification for multi-Rx DL reception.

Proposal 2: A new RMC for 2-layer DL MIMO need to be defined for multi-Rx.

Proposal 3: Do not use panel concept no matter in RF requirement or test configuration.

Proposal 4: Define a EIS tolerance = max(∆EIS_1, ∆EIS_2) ≤ [TBD] dB to ensure the performance gain of the multi-Rx DL reception.

- References

- R4-2214457, WF on FR2 multi-Rx chain DL reception, Sony, RAN4#104e

- R4-2216125, Discussion on UE implementation assumption of multi-Rx DL reception, vivo, RAN4#104e-bis