3GPP TSG-RAN WG4 Meeting # 104bis-e										R4-2216128

Electronic Meeting, October 10 – October 19, 2022

Source: 	vivo

Title: 	Feasibility evaluation of FR2 UL 256 QAM

Agenda Item:	6.7.2

Document for:	Discussion

- Introduction

In the last meeting, the link level simulation assumption was approved [1]:

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 3%, 3.5%, 4%, rxEVM: 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

In this contribution, we further provide our link level simulation results.

- Discussion

2.1 link level simulation

- 29 GHz

- TDL-A

- TDL-D

- AWGN

The simulation results above are summarized as follows.

Observation 1：The UL 256 QAM under 29 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.0 dB | 22.8 dB | 20 dB | 21 dB | 18 dB | 19.2 dB |
| CP-OFDM | 22.5 dB | 23.8 dB | 20.3 dB | 21.5 dB | 19.2 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.2 dB | 23.1 dB | 20.3 dB | 21.2 dB | 18.5 dB | 19.5 dB |
| CP-OFDM | 22.7 dB | 24 dB | 20.8 dB | 21.8 dB | 19.5 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.9 dB | 23.5 dB | 20.6 dB | 21.5 dB | 19.2 dB | 19.7 dB |
| CP-OFDM | 23.2 dB | 24.7 dB | 21.2 dB | 22 dB | 19.8 dB | 19.9 dB |

- 39 GHz

- TDL-A

- TDL-D

- AWGN

Observation 2：The UL 256 QAM under 39 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24 dB | 27 dB | 22.3 dB | 25.8 dB | 19.9 dB | 22 dB |
| CP-OFDM | 28 dB | N/A | 24 dB | N.A. | 21.5 dB | 26 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.4 dB | 27.5 dB | 23 dB | 26 dB | 20 dB | 23 dB |
| CP-OFDM | 29.2 dB | N/A | 25 dB | N.A. | 21.8 dB | 28 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.8 dB | 28.2 dB | 23.3 dB | 28 dB | 20.3 dB | 23 dB |
| CP-OFDM | 34 dB | N/A | 25.8 dB | N.A. | 22 dB | 30 dB |

- 48 GHz

- TDL-A

- TDL-D

- AWGN

Observation 3：The UL 256 QAM is hard to bring performance gain under 48 GHz

2.2 System level simulation

In [2], we provide an initial system level evaluation for UL 256QAM and the results shows only few UEs can achieve 26 dB target SNR at BS side. Unlike DL scenario, the output power of UE is limited and although UL 256 QAM can be proved to bring performance gain in link level simulation, it is hard for UE to perform a high SNR at BS side. In addition, when UE try to achieve a high SNR at BS side, the output power of UE is larger and the adjacent channel interference can not be ignored during the simulation. The ACIR in TR 38.803 can be the baseline for further evaluation.

Observation 4: The system level simulation is needed to show whether the UE can achieve target SNR at BS side.

Proposal: Taking the following system level simulation assumption as starting point for further evaluation:

Table I system level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 | 1 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | UMa LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH - Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | PC1/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| ACIR |  | 15 dB |  |
| Target SNR at BS side |  | [25] dB |  |
| UE max output power |  | PC1: 35 dBm/PC3: 23 dBm/PC5: 23 dBm |  |

- Conclusion

In this contribution, we share our simulation results of

Observation 1：The UL 256 QAM under 29 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.0 dB | 22.8 dB | 20 dB | 21 dB | 18 dB | 19.2 dB |
| CP-OFDM | 22.5 dB | 23.8 dB | 20.3 dB | 21.5 dB | 19.2 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.2 dB | 23.1 dB | 20.3 dB | 21.2 dB | 18.5 dB | 19.5 dB |
| CP-OFDM | 22.7 dB | 24 dB | 20.8 dB | 21.8 dB | 19.5 dB | 19.7 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 22.9 dB | 23.5 dB | 20.6 dB | 21.5 dB | 19.2 dB | 19.7 dB |
| CP-OFDM | 23.2 dB | 24.7 dB | 21.2 dB | 22 dB | 19.8 dB | 19.9 dB |

Observation 2：The UL 256 QAM under 39 GHz can achieve performance gain at:

| Tx_EVM = Rx_EVM =3% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24 dB | 27 dB | 22.3 dB | 25.8 dB | 19.9 dB | 22 dB |
| CP-OFDM | 28 dB | N/A | 24 dB | N.A. | 21.5 dB | 26 dB |

| Tx_EVM = Rx_EVM =3.5% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.4 dB | 27.5 dB | 23 dB | 26 dB | 20 dB | 23 dB |
| CP-OFDM | 29.2 dB | N/A | 25 dB | N.A. | 21.8 dB | 28 dB |

| Tx_EVM = Rx_EVM =4% | TDL-A |  | TDL-D |  | AWGN |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100MHz | 50 MHz | 100MHz | 50 MHz | 100MHz |
| DFT-s-OFDM | 24.8 dB | 28.2 dB | 23.3 dB | 28 dB | 20.3 dB | 23 dB |
| CP-OFDM | 34 dB | N/A | 25.8 dB | N.A. | 22 dB | 30 dB |

Observation 3：The UL 256 QAM is hard to bring performance gain under 48 GHz

Observation 4: The system level simulation is needed to show whether the UE can achieve target SNR at BS side.

Proposal: Taking the following system level simulation assumption as starting point for further evaluation:

Table I system level simulation assumptions

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 | 1 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | UMa LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH - Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | PC1/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| ACIR |  | 15 dB |  |
| Target SNR at BS side |  | [25] dB |  |
| UE max output power |  | PC1: 35 dBm/PC3: 23 dBm/PC5: 23 dBm |  |

- References

- R4-2214453	WF on UL 256QAM	Xiaomi ,  RAN4#104e