# 3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216133

# Electronic Meeting, 10th - 19th October, 2022

Source: 	vivo

Title: 	Discussion on adjacent channel co-existence simulation assumptions for full duplex

Agenda Item:	6.17.2.1

Document for:	Approval

Introduction

In the last meeting, it was agreed that adjacent channel co-existence study in RAN4 for SBFD is needed [1]. The scenarios for adjacent channel co-existence study in RAN4 was captured in [1] and the adjacent channel co-existence simulation parameters was captured in [2]. In the meantime, RAN1 was also studying adjacent channel co-existence with different study targets. In the discussion for adjacent co-existence simulation assumptions, some of the discussion is overlapping in RAN1 and RAN4. In order to avoid large dispute for the adjacent co-existence study, it is better to align the simulation assumption as much as possible. In this contribution, comparison of adjacent channel co-existence simulation assumptions in RAN1 and RAN4 is summarized and our views on adjacent channel simulation are provided.

Discussion

It was agreed in the last meeting adjacent channel co-existence simulations in RAN1 and RAN4 are both needed for different study purposes. However, most of simulation parameters discussed in RAN4 were also discussed in RAN1 in more detailed ways. To avoid duplicate work in RAN4 and align the simulation assumptions with RAN1 as much as possible, comparison of adjacent channel co-existence simulation assumptions in RAN1 and RAN4 is summarized in Table 1.

Table 1. Summary of adjacent channel co-existence simulation assumptions

| Simulation parameters | RAN4 [4] | RAN1 [3] |
| --- | --- | --- |
| Frequency | - FR1: 4GHz as exemplary frequency, FR2: 30GHz as exemplary frequency. - Not consider FR 2-2. | - FR1: 4GHz as exemplary frequency, FR2: 30GHz as exemplary frequency. - Not consider FR 2-2. |
| SBFD configurations | To consider both {DUD} and {DU} configurations in this stage | To consider both {DUD} and {DU} configurations in this stage |
| Inter-site distance (ISD) | FR1 UMa with 500m ISD, FR2 UMa with 200m ISD. | FR1 UMa with 500m ISD, FR 1 and 2 Dense Urban Macro Layer with 200m ISD. |
| Channel model | Option 1: Pathloss re-use TR 38.828 as starting point | For LOS probability of gNB-gNB channel, - For Macro-gNB-to-Macro-gNB case - Option 3: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. o X = 0.75 - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. The channel model used in the evaluation are detailed in Table 2 in the Annex. |
| Grid shift considerations | Further study following candidate options for grid shift in SLS and analysis.  Option 1: 100%  Option 2: 0%  Option 3: 10%  Option 4: TBA | For evaluation of adjacent-channel coexistence between two networks for Urban Macro and Dense Urban Macro layer scenarios in RAN1, consider grid shifts between two networks of 0% and 100%. |
| UE distribution in macro scenario | 7.1 For Indoor/Outdoor UE ratio: Further study with following candidate options: Option 1-1: Re-use TR 38.828, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor; Option 1-2: For Macro-to-Macro cases of both FR1 and FR2, assume 80% indoor and 20% outdoor ratio. 7.2 For UE distribution mechanism: Further study with following candidate options: Option 2-1: Evenly random dropping in service area; Option 2-2: Consider clusters in UE dropping. - Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum distance between macro TRP to cluster centre, e.g., 100m , where the size of each cluster is 120 x 50 (m); - Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster. | For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform (1, Nfl) where Nfl ~ uniform (4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details |
| For number of UE considered in study: | 2 UEs for {DU} subband config, 3 UEs for {DUD} config. | N/A |
| Noise figures | For Macro case, Re-use TR 38.828 assumptions -- FR1 BS: 5dB, UE: 9dB; FR2 BS: 10dB, UE: 10dB. | FR1 BS: 5dB，UE: 9dB; FR2 BS: 7dB, UE: 13dB. |
| BS antenna and TRP considerations |  Option 1: Re-use TR 38.828 for legacy TDD BS, and consider two options for SBFD BS antenna and TRP power  Option 1-1: Utilize half of its original panel for SBFD UL and DL each. In this case, the TRP and elements number for DL and UL in SBFD BS will be half of the TDD BS configuration.  Option 1-2: Utilize an extra panel for subband UL operation. In this case, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration.  Option 2: Use extended AAS model defined in TR 38.803, Table 5.2.3.2.4-2 as the sub-array is essential to be able to provide TX/RX isolation required for SBFD. For FR1 parameters in TR 38.803, Table 5.2.3.2.4-3 is proposed as starting point. Parameters for FR2 is proposed as Table 4 in R4-2212620. Detailed parameters are copied in annex 1  Option 3: SBFD needs to be able to function with realistic power levels per carrier in today’s deployments. E.g. 55 dBm as second power level to be studied. | For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption. - Companies can report the separation of the Tx panel and Rx panel assumed in their simulation. - Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception. For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used. - Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. - Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. - Companies report which option is assumed in their simulation. For UMa: - Option 1: [53] dBm for 100MHz - Option 2: [49] dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1] For Dense Urban Macro layer : - For FR 1: - Option 1: [53] dBm for 100MHz - Option 3: [44] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1] - For FR 2 - Option 1: [43] dBm for 200MHz [refer to TR 38.828 Table 5.2.2.4-1] |
| UE antenna and Tx power | For FR1 Re-use TR 38.828 assumptions. i.e. FR1 max Tx 23dBm, min Tx -40 dBm with 0dBi omni directional antenna. For FR2 Further study with following candidate options: Option 1: Re-use TR 38.828 assumptions. i.e. FR2 max Tx 13.4dBm (peak eirp 22.4dBm), min Tx -40dBm, with antenna configuration in the table of annex 2 Option 2: FR2 max Tx 23dBm (peak eirp 43dBm) with (M, N, P)=(1, 4, 2), 2 panels antennas and element gain as 1.5 dBi | For FR 1: 23dBm For FR 2-1: 23 dBm. EIRP should not exceed 43 dBm |
| BS mechanical down-tilt angles | Further study with following candidate options:  Option 1: Use 6 degrees for Macro BS for FR1 and FR2 as provided in TR 38.803, and 90-deg (point to ground) for indoor.  Option 2: Mechanical tilt should be determined together with array parameters as a package per considered deployment scenario. | N/A |
| Uplink power control model | For downlink scenario, no power control scheme is applied. For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [9] is applied with following parameters. - CLx-ile = –SNR_target + UE_maxpower – ThermalNoise – BS_NoiseFigure + 10*log10(BW) - γ = 1 Where, SNR_target for FR1 and FR2 are 15 dB. | Companies to report power control parameters. For calibration: - P0= -60 dBm, alpha = 0.6 for InH [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -86 dBm, alpha = 0.9 for Dense Urban [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] P0= -80 dBm, alpha = 0.8 for Urban Macro |
| BS ACLR/ACS considerations | Re-use TR 38.828 assumptions -- ACLR/ACS for FR1 BS: 45/46 dBc, FR2 BS: 28/23.5 dBc. | N/A Pending on RAN4’s reply LS on interference modelling |
| SBFD self-interference consideration in SLS | Proposed to use {N = noise floor + XdB} for the SBFD system to simulate the self-interference impact as a simplified method to evaluate its SINR/throughput. For the value of X: - Taking 1dB sensitivity degradation due to self-interference of DL transmission as starting point for system level evaluation and feasibility study - Other values lower than 1dB e.g. 0.1dB/0.8dB not precluded pending on the feasibility study - Final values used in co-existence evaluation shall be aligned with feasibility analysis conclusion. | N/A Pending on RAN4’s reply LS on interference modelling |
| Co-ex study steps | In reference [2]. | In reference [3]. |
| Evaluation metrics | It is proposed to follow the TR 38.828’s evaluation criteria to check the 50% and 5% throughput loss compared to the baseline scenario defined. - FFS for traffic model; | In reference [3]. |
| Other assumptions | Take option 1 as basis for those SLS assumptions that were not discussed. Option 1: Propose to adopt the system characteristics, deployment parameters and other assumptions from TR 38.828 as the starting point for SBFD co-ex study. But the assumptions should be updated accordingly to fulfill the SBFD co-ex purpose. | In reference [3]. |

Since RAN1 started the simulation study one meeting earlier than RAN4, a set of detailed simulation assumptions were already established in RAN1. From our perspective, it saves a lot of effort if RAN4 tries to reuse the simulation assumptions in RAN1. It can be observed in table 1 that some critical simulations assumptions for adjacent co-existence in RAN4 are not aligned with RAN1, e.g. pathloss in channel model, UE distribution, Noise Figure for FR2, BS antenna and TRP considerations. Some FFS for the adjacent channel simulation assumptions in RAN4 are already cleared in RAN1, e.g. Grid shift considerations, traffic model. For other assumptions, we suggest to check if RAN1 has made consensus for co-existence study first.

Observation 1: Some critical simulations assumptions for adjacent co-existence in RAN4 are not aligned with RAN1, e.g. pathloss in channel model, UE distribution, Noise Figure for FR2, BS antenna and TRP considerations.

Observation 2: Some FFS for the adjacent channel simulation assumptions are already cleared in RAN1, e.g. Grid shift considerations, traffic model.

Proposal 1: It is suggested to align the adjacent co-existence simulation assumptions with RAN1 as much as possible.

Conclusion

This contribution gives a comparison of adjacent channel co-existence SLS assumptions in RAN1 and RAN4. The following observations and proposals are made:

Observation 1: Some critical simulations assumptions for adjacent co-existence in RAN4 are not aligned with RAN1, e.g. pathloss in channel model, UE distribution, Noise Figure for FR2, BS antenna and TRP considerations.

Observation 2: Some FFS for the adjacent channel simulation assumptions are already cleared in RAN1, e.g. Grid shift considerations, traffic model.

Proposal 1: It is suggested to align the adjacent co-existence simulation assumptions with RAN1 as much as possible.

- References

[1] R4-2214378, WF on adjacent channel co-existence study, Samsung, RAN4#104-e.

[2] R4-2214379, WF on Simulation assumption for adjacent co-existence study, CMCC, RAN4#104-e.

[3] 3GPP RAN1#109-e&110 meeting, chairman’s notes.

- Annex

Table 2. The channel model used in the evaluation

|  | Dense urban, Urban macro | Indoor office |
| --- | --- | --- |
| Large-scale channel parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) |
| Fast fading parameters | FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 | FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD |