3GPP TSG-RAN WG4 Meeting #104-bis-e                                                       R4-2216152

Online, 10th - 19th October, 2022

Source:	Vodafone

Title:	TP for TR 38.718-02-01: CA_n78-n94

Agenda item:			5.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n78-n94 according to the request in [1].

# 2. Reference

- RP-221876 New WID Rel-18 NR Inter-band Carrier Aggregation / Dual Connectivity for 2 bands DL with x bands UL (x=1,2)

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	CA_n78-n94

### 6.x.1				Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1			Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n78 + n94

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n94 | 880 MHz | – | 915 MHz | 1432 MHz | – | 1517 MHz | FDD |

#### 6.x.1.2			Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n78 + n94

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n78A-n94A | - | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n94 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 6.x.1.3			Co-existence studies

Tables 6.X.1.3-1 and 6.X.1.3-2 list the band edge harmonics for the UL and DL respectively to allow for inspection of potential DL band REFSENS impact and harmonic mixing.

Table 6.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 10800 | 11400 | 13200 | 15200 |
| n94 | 880 | 915 | 1432 | 1517 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

Table 6.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 10800 | 11400 | 13200 | 15200 |
| n94 | 880 | 915 | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

Table 6.X.1.3-1 shows there may be a 4th harmonic impact on the n78 DL from the n94 UL.

#### 6.X.1.4			∆TIB and ∆RIB values

As is the case for DC_8-32_n78 in 38.101-3, zero ΔTIB,c or ΔRIB,c has been proposed in Tables 6.X.1.4-1 and 6.x.1.4-2.

Table 6.X.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n78-n94 | n78 | 0 |
|  | n94 | 0 |

Table 6.X.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n78-n94 | n78 | 0 |
|  | n94 | 0 |

#### 6.X.1.5			REFSENS requirements

No additional REFSENS adjustments are required for this band combination.

#### 6.X.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this band combination.

<End of Text Proposal>