3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216175

# Electronic Meeting, October 10 – October 19, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for Redcap UE Demodulation PDSCH

Agenda item:	4.6.5.1.1

Document for:	Information

- Introduction

In RAN4#104-e [3], the Demod Requirements for RedCap UEs [1] were introduced in the spec [2]. In this paper, we share our simulation and impairment results for the remaining PDSCH cases.

# Simulation Results for PDSCH 256QAM with 1 RX UE

## FR1 15KHz (FDD)

| # Rx | CBW / SCS | Rank | Propagation condition | Antenna configuration | Reference (TS38.101-4 v17.6.0 ) | MCS | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | 1 | TDLA30-10 | 2x1 Low | 5.2.1.1.1-3 Test 1-4 | 20 | 22.7 | 24.7 |

## FR1 30kHz (TDD)

| # Rx | CBW / SCS | Rank | Propagation condition | Antenna configuration | Reference (TS38.101-4 v17.6.0 ) | MCS | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | 1 | TDLA30-10 | 2x1 Low | 5.2.2.1.1-3 Test 1-4 | 20 | 23.4 | 25.4 |

- Other Simulation Results

For completeness, we are also resubmitting in this section the simulation results shared in the last meeting for the other Redcap test cases

## FR1 15KHz (FDD)

Based on the agreed simulation assumptions, we provide here the PDSCH SNR Alignment and Impairment results the new RedCap UE requirements to be introduced for FR1 15KHz FDD test cases at 70% of peak rate.

| # Rx | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference (TS38.101-4 ) | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.1.1-3 Test 1-1 | 1 | 3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.1.1-3 Test 1-4 | 9.6 | 11.6 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.1.1-4 Test 2-1 | 13.8 | 15.8 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.1.1-3 Test 1-4 | 5.6 | 7.6 |

## FR1 30kHz (TDD)

Based on the agreed simulation assumptions, we provide here the PDSCH SNR Alignment and Impairment results the new RedCap UE requirements to be introduced for FR1 30kHz TDD test cases at 70% of peak rate.

| # Rx | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Reference (TS38.101-4 ) | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 5.2.2.2.1-3 Test 1-1 | 1.2 | 3.2 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 5.2.2.2.1-3 Test 1-4 | 9.9 | 11.9 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 5.2.2.2.1-4 Test 2-1 | 14.1 | 16.1 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-1 | -2.3 | -0.3 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-4 | 5.8 | 7.8 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | TDLA30-10 | 2x2 ULA low | 5.2.2.2.1-4 Test 2-1 | 17.4 | 19.4 |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 5.2.2.2.1-3 Test 1-3 | 22.8 | 24.8 |

- Conclusions

Alignment and Impairment SNR results included in the contribution.

- References

[1] RP-211574, “Revised WID on support of reduced capability NR devices”, 3GPP TSG RAN Meeting #92-e, Electronic Meeting, June 14-18, 2021

[2] 3GPP TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17.6.0)”;

[3] R4-2214394, “WF on RedCap UE demodulation and CQI reporting requirements”, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic Meeting, 15 August ‒ 26 August 2022