3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216176

# Electronic Meeting, October 10 – October 19, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for Redcap UE Demodulation PDCCH and PBCH

Agenda item:	4.6.5.1.2

Document for:	Information

- Introduction

In RAN4#104-e [3], the Demod Requirements for RedCap UEs [1] were introduced in the spec [2]. In this paper, we submit our simulation and impairment results for PDCCH and PBCH Demodulation Requirements.

# Simulation Results for PDCCH Demodulation

## FR1 15KHz (FDD)

Based on the agreed simulation assumptions, we provide here the PDCCH SNR Alignment and Impairment results for the new RedCap UE requirements, to be introduced for FR1 15KHz FDD test cases at 1% Pm-dsg.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 5.3.2.1.2-1 Test 3 | 2.7 | 4.7 |

## FR1 30kHz (TDD)

Based on the agreed simulation assumptions, we provide here the PDCCH SNR Alignment and Impairment results for the new RedCap UE requirements, to be introduced for FR1 30kHz TDD test cases at 1% Pm-dsg.

| 1Rx UE or 2Rx UE | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Reference from TS38.101-4 | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 6.3 | 8.3 |
| 2Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x2 low | 5.3.2.2.1-1 Test 2 (R.PDCCH.2-1.2 TDD) | 0.9 | 2.9 |
| 2Rx UE | 20 | 48 | 1 | 8 | TDLC300-100 | 2x2 low | 5.3.2.2.2-1 Test 1 (R.PDCCH.2-1.3 TDD) | -3 | -1 |

# Simulation Results for PBCH Demodulation

We provide here the PBCH SNR Alignment and Impairment results for the new RedCap UE requirements to be introduced for FR1 30kHz TDD test cases at 1% Pm-bch.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCK block index | Reference from TS38.101-4 | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | -3.6 | -1.6 |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | -2.2 | -0.2 |

- Conclusions

Alignment and Impairment SNR results included in the contribution.

- References

[1] RP-211574, “Revised WID on support of reduced capability NR devices”, 3GPP TSG RAN Meeting #92-e, Electronic Meeting, June 14-18, 2021

[2] 3GPP TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17.6.0)”;

[3] R4-2214394, “WF on RedCap UE demodulation and CQI reporting requirements”, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic Meeting, 15 August ‒ 26 August 2022