3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216181

# Electronic Meeting, October 10 – October 19, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for FR2-2 UE Demodulation PDCCH and PBCH

Agenda item:	4.3.7.2.2

Document for:	Information

- Introduction

In RAN4#104-e [2], simulation assumptions and requirement tables were preliminarily agreed for PDCCH demodulation and PBCH demodulation for UEs that support FR2-2[1]. In this paper, we share our simulation results for tests based on these assumptions.

# PDCCH 120kHz

Based on the agreed simulation assumptions and the options provided in [3] for the requirements definition, we provide here the PDCCH SNR Alignment results for FR2-2 test cases, at 1% misdetection rate.

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | Alignment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-650 | 1_0 | 40 | 1x2 Low | 3.6 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 |  | 1_1 | 56 | 1x2 Low | 0.8 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 |  | 1_1 | 56 | 2x2 Low | -2.8 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 |  | 1_0 | 40 | 2x2 Low | -6 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 3.7 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 |  | 1_1 | 56 | 1x2 Low | 0.1 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 |  | 1_0 | 40 | 2x2 Low | -4.1 |

# PBCH

Based on the agreed simulation assumptions and the options provided in [3] for the requirements definition, we provide here the PBCH SNR Alignment results for FR2-2 test cases, at 1% misdetection rate.

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index | Alignment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | TDLA30-650 | New | unknown | -7.5 |
| 1-2 | 480kHz/400MHz |  | TDLA10-650 | New | unknown |  |

- Conclusions

Alignment SNR results for PDCCH and PBCH included in this contribution.

- References

[1] RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, 3GPP TSG RAN Meeting #94-e, Electronic Meeting, December 06-17, 2021

[2] R4-2214390, “WF on UE demodulation performance requirements definition for 52.6 - 71 GHz”, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic Meeting, 15 August ‒ 26 August 2022

[3] 3GPP TR 38.884, “Study on enhanced test methods for FR2 NR UEs (Release 18, v18.2.0)”;