3GPP TSG-RAN WG4 Meeting #104bis-e													R4-2216201

Electronic meeting, October 10-19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Assumptions and Initial Simulation results for SBFD coexistence evaluation

Agenda item:	6.17.2.1

Document for:	Approval

- Introduction

The new RAN1/RAN4 study item on evolution of duplex operation for NR TDD systems in unpaired spectrum was adopted [1]. The assumptions are listed as follows:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

While the work item objectives are the following:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

This document focuses on the highlighted objective on studying adjacent channel coexistence aspects, building on the discussion and agreements reached in RAN4#104-e as summarized in [2], [3]. In addition, some preliminary simulation results are presented in Section 3.

# Discussion

## 2.1 Scenarios for adjacent channel co-existence study in RAN4

The following agreement has been made:

- For FR1 SBFD scenarios for co-ex study: - Consider Urban Macro as baseline scenarios with high priority; - Consider Indoor scenario as second priority and not to preclude other scenarios. - For FR2 SBFD scenarios for co-ex study: - Consider Urban Macro as baseline scenario with high priority; - Consider Urban Micro, Indoor scenarios as second priority and not preclude other scenarios. - For scenarios, including aggressor, victim and aggressor baseline, of co-ex study: - For NR TDD DL as victim, assumes NR TDD DL in adjacent channel as aggressor baseline; assumes SBFD in adjacent channel as aggressor. - For NR TDD UL as victim, assumes NR TDD UL in adjacent channel as aggressor baseline; assumes SBFD in adjacent channel as aggressor. Consider the cases, where NR TDD UL as victim, with lower priority. - For SBFD as victim, assumes TDD DL as aggressor with high priority while consider the TDD UL as aggressor and SBFD as aggressor cases with low priority. The aggressor baseline, for SBFD as victim, is FFS with following candidate options: - Option 1: No system in adjacent channel; - Option 2: SBFD in adjacent channel. - The above agreements are summarized in Table 2.2.2-2. And these tables would be updated with new agreements. For better understanding, the above agreements can be summarized into the tables below. Table 2.2.1-1: Scenarios for SBFD co-ex study FR; Scenario No.; Deployment Scenario1 (Aggressor -> Victim); Priority. FR1 (4GHz); 1; Urban Macro -> Urban Macro; High. 2; Indoor -> Indoor; Low. FR2 (30GHz); 3; Urban Macro -> Urban Macro; High. 4; Urban Micro -> Urban Micro; Low. 5; Indoor -> Indoor; Low. Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios.. Table 2.2.1-2: Victim, aggressor and aggressor baseline for SBFD co-ex study Victim; Aggressor; Aggressor baseline; Priority. NR TDD DL; SBFD; NR TDD DL; High priority. NR TDD UL; SBFD; NR TDD UL; Low priority. SBFD; NR TDD DL; FFS; High priority. NR TDD UL; Low priority. SBFD; Low priority.

There is one FFS related to the aggressor baseline for the case of SBFD as victim and NR TDD DL as aggressor. In our view, in this scenario the focus should be on the SBFD UL receiver performance, where the SBFD network is expected to operate decently even when in a fully isolated manner. Therefore, in this case our preference is to adopt Option 1 in the agreement above corresponding to “No system in adjacent channel”.

Proposal 1: For the scenario with SBFD as victim and NR TDD DL as aggressor, adopt as aggressor baseline: “Option 1: No system in adjacent channel”

## 2.2 SBFD subband configurations

The following agreement was reached in RAN4#104-e [3]:

SBFD sub-band configurations in co-ex study (issue 2-2-6): To consider both {DUD} and {DU} configurations in this stage, while not preclude to reduce to one configuration if two configurations are equivalent for study after agreed on other assumptions.

Both {DUD} and {DU} have been agreed for coexistence evaluations but neither the carrier bandwidth nor the the corresponding SBFD UL/DL frequency split have not been agreed yet. For the carrier bandwith, we think we can reuse the assumptions adopted in the Rel-16 CLI study in TR 38.828 which are 100 MHz for FR1 and 200 MHz for FR2. Furthermore, an UL subband occupying approximately 20% of the total carrier bandwidth can also be assumed which is aligned with previous discussions that took place in both RAN1 and RAN4 WGs. Finally, regarding the guardband between UL and DL subbands we think a reasonable assumption is to adopt similar requirement as used for the guardband between adjacent channels as defined in TS 38.101-1; that is, a minimum of 845 kHz on each side for a 100 MHz carrier bandwidth and 30 kHz SCS, which results in a minimum separation of 1690 kHz between two channels. In number of resource blocks, this can be approximated to 5 resource blocks. In summary:

Proposal 2: For the SBFD coexistence evaluation, assume 100 MHz carrier bandwidth for FR1 and 200 MHz carrier bandwidth for FR2.

Proposal 3: For the UL/DL SBFD frequency split, assume a UL subband occupying approximately 20% of the channel bandwidth, while size of the guardband between UL and DL subbands is derived based on the requirements for guardband between adjacent channels as defined in TS 38.101-1. The following RB split is suggested:

- 104:5:55:5:104 (D:G:U:G:D) or 213:5:55 (D:G:U)  for a FR1 100 MHz carrier with 30 kHz SCS (273 RBs in total)

- 49:4:26:4:49 (D:G:U:G:D) or 103:4:26 (D:G:U) for a FR2 200 MHz carrier with 120 kHz SCS (132 RBs in total)

## 2.3 Grid shift considerations

The following candidate options were agreed for further study in [3]:

- Option 1: 100%

- Option 2: 0%

- Option 3: 10%

- Option 4: TBA

For the case with 100% grid shift, we think this scenario can be already considered for simulations to understand the best-case coexistence performance, although it is important to note that in practice 100% grid shift is not possible to achieve especially when 3 operators operate on adjacent carriers.  Co-located base stations (i.e. 0% grid shift) is a common practice in today’s cellular deployments and should be therefore also studied with high priority. In our view, a reasonable way forward would be to study the 0% and 10% grid shift case in an analytical manner by first discussing the minimum required  inter-operator isolation values that allow feasible SBFD operation (e.g. from receiver blockage perspective). Afterwards, system-evel simulations may be performed with such grid shifts depending on the outcome of this analysis..

Proposal 4: For the grid shift considerations in the SBFD coexistence evaluation:

- Adopt 100% grid shift for adjacent-channel coexistence evaluations to understand the upper-bound of SBFD performance.

- Determine the minimum inter-operator isolation requirements for feasible SBFD operation and, as a next step, determine for which values of grid shift such inter-operator inter-cell isolation values may be achieved. Simulations may or may not be run with grid shift <100% depending on the outcome of the evaluation.

## 2.4 BS antenna and TRP considerations

The following way forward was agreed in [3]:

Further study with following options: - Option 1: Re-use TR 38.828 for legacy TDD BS, and consider two options for SBFD BS antenna and TRP power - Option 1-1: Utilize half of its original panel for SBFD UL and DL each. In this case, the TRP and elements number for DL and UL in SBFD BS will be half of the TDD BS configuration. - Option 1-2: Utilize an extra panel for subband UL operation. In this case, the TRP and element number for DL and UL in SBFD BS will be the same as TDD BS configuration. FR1 Macro Urban; FR2 Macro Urban. BS antenna configurations; For Legacy TDD: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ For SBFD: Option 1: (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.8)λ Option 2: (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ; For 30 GHz legacy TDD: (1, 1, 8, 16, 2) For SBFD: Option 1 (1, 1, 8, 8, 2) Option 2 (1, 1, 8, 16, 2). BS Tx power; For Legacy TDD: 49 dBm For SBFD: Option 1: 46 dBm Option 2: 49 dBm; For legacy TDD: 43dBm For SBFD: Option 1: 40 dBm Option 2: 43 dBm. - Option 2: Use extended AAS model defined in TR 38.803, Table 5.2.3.2.4-2 as the sub-array is essential to be able to provide TX/RX isolation required for SBFD. For FR1 parameters in TR 38.803, Table 5.2.3.2.4-3 is proposed as starting point. Parameters for FR2 is proposed as Table 4 in R4-2212620. Detailed parameters are copied in annex 1 - Option 3: SBFD needs to be able to function with realistic power levels per carrier in today’s deployments. E.g. 55 dBm as second power level to be studied.

Our preference regarding MIMO and BS antenna configuration is to align as much as possible with the RAN1 assumptions and agreements, as quoted in the following:

Agreement For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. Agreement For evaluation and comparison between SBFD and legacy TDD, the two options for the SBFD antenna configuration agreed in RAN1#109 are further clarified as below: - SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2 (same as Opt 2 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3 (new): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD. Agreement For evaluation of SBFD operation, it is up to companies to report the BS antenna configurations used in their simulations. The BS antenna configurations in the following table can be considered for calibration purpose Scenarios; FR; Legacy TDD; SBFD. BS antenna configuration for Indoor office; FR1; M,N,P,Mg,Ng;Mp,Np?= (4,4,2,1,1; 4,4) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (2,4,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; M,N,P,Mg,Ng;Mp,Np=(16,8,2,1,1; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV= (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ. BS antenna configuration for Urban Macro/ Dense Urban Macro layer/ Dense Urban Micro layer; FR1; M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; M,N,P,Mg,Ng;Mp,Np= (4,16,2,2,2; 1,1) dH,dV= (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ.

In short, high transmission power of up to 53 dBm per 100 MHz carrier is assumed for FR1 Urban Macro and Dense Urban scenarios, while the antenna panel size is M,N,P,Mg,Ng;Mp,Np=(8,8,2,1,1;2,8) for legacy TDD and SBFD (Option 2 as per RAN1 agreement), while an antenna configuration with half of the antenna elements (per transmit direction) can also be assumed for SBFD (i.e. M,N,P,Mg,Ng;Mp,Np=(4,8,2,1,1;2,8)) according Option 1 of the RAN1 agreement. It is also important to note that sub-arrays are used consisting of grouping 4 vertical antenna elements in each column of the antenna panel. Based on this, our proposal is the following:

Proposal 5: For the BS antenna and MIMO configuration in FR1 Urban macro scenario, adopt the following:

- BS transmit power of 53 dBm per 100 MHz carrier

- Two options for antenna panel for SBFD: Option 1: (Mg,Ng,M,N,P)=(1,1,4,8,2), (dH,dV)=(0.5,0.8)λ, Option 2: (Mg,Ng,M,N,P)=(1,1,8,8,2), (dH,dV)=(0.5,0.8)λ

- Modeling of sub-arrays consisting of groups of 4 vertical antenna elements in each column of the antenna panel that are connected to the same RF chain.

## 2.5 UE Distribution

The following relevant way forward was agreed in [3]:

For Indoor/Outdoor UE ratio: Further study with following candidate options: - Option 1-1: Re-use TR 38.828, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor; - Option 1-2: For Macro-to-Macro cases of both FR1 and FR2, assume 80% indoor and 20% outdoor ratio. For UE distribution mechanism: Further study with following candidate options: - Option 2-1: Evenly random dropping in service area; - Option 2-2: Consider clusters in UE dropping. - Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum distance between macro TRP to cluster centre, e.g., 100m , where the size of each cluster is 120 x 50 (m); - Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster. For number of UE considered in study: user numbers per transmission reception point should equal to the number of sub-bands, i.e. 2 UEs for {DU} subband config, 3 UEs for {DUD} config.

For the Indoor/Outdoor UE ratio, we dislike Option 1-2 as we expect FR2 to provide coverage mainly for outdoor UEs, while indoor UEs would be typically served by the FR1 layer. Therefore, our preference is to go for Option 1-1.

Proposal 6: For the indoor/outdoor UE ratio, adopt Option 1-1: Re-use TR 38.828, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor.

For the UE distribution mechanism, we think Option 2-1 would result in overly optimistic results especially for macro deployments where the probability of having meaningful UE-UE cross-link interference will be close to zero given the sparse UE positioning. For more realistic evaluation and to avoid reaching misleading conclusions, we think Option 2-2 should be adopted as it better reflects the facts that users may be close to each other e.g. in a meeting room or in the same vehicle.

Besides, we think a few more things require clarification which we list below:

Dropping of the cluster in each macro area for grid shift higher than 0%:

For the case of non-colocated base stations from different operators, the ‘macro cell area’ does not fully overlap between the two simulated operators. As the intention is to assume that UEs belonging to different operators can be positioned close to each other, we think each cluster should be associated to a macro cell from each of the operators. This ensures that UEs from different operators are placed in the same cluster. To achieve this in the simulation, we can simply take one of the two operators as the reference for dropping the clusters.

Indoor/outdoor assumption for UEs inside the cluster:

Regarding the UE distribution in Option 2-2, we understand that the intention of the 120m x 50 m cluster is to resemble a building. Therefore, for ‘Step 2’, it needs to be clarified that UEs dropped inside the cluster are considered indoor.

Proportion of UEs inside/outside the cluster:

It is also unclear how the 80% indoor (or inside the cluster) - 20% outdoor UE distribution is achieved considering the other agreement that the number of UEs per TRP is equal to the number of subbands (i.e. 2 and 3 for DU and DUD,  respectively). Regardless of whether 2 or 3 UEs are deployed per macro cell, it is not possible to have exactly 80% of those UEs placed in the cluster. To address this, we think two alternatives can be considered: i) Option 2-2 discussed above is amended to clarify that the proportion of 80% of the UEs inside the cluster is achieved over the entire network, or ii) more than 1 UE per subband (e.g. 5 or 10) are deployed per macro cell area, where 80% of those are deployed inside the cluster. For simplicity, Round Robin scheduler can be used with the scheduling restriction of 1 UE per subband direction. Both options are fine, although we have slight preference for the first alternative.

These three points are summarized in the following proposal:

Proposal 7: For the UE distribution in UMa simulations, adopt Option 2-2: Consider clusters in UE dropping with the following clarifications in red.

-	Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum distance between macro TRP to cluster centre, e.g., 100m , where the size of each cluster is 120 x 50 (m);

- Note 1: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro TRP to cluster centre should be respected also for TRPs belonging to the other operator.

-	Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster.

- Note 2: UEs dropped within the cluster are indoor and UEs dropped outside the cluster are outdoor.

- Note 3: The percentage of the UEs inside the cluster is achieved over the entire network and not in each individual macro cell area.

## 2.7 Traffic model

For the traffic model, for simplicity, we could adopt the same assumption as in TR 38.828, i.e. low load (RU 10%) and Full buffer.

Proposal 8: For the traffic model, re-use the assumptions from TR 38.828: low load (RU 10%) and Full buffer.

## 2.8 Co-channel inter-site inter-subband interference

In RAN4#104-e, assumptions for self-interference as well as adjacent channel interference were discussed and a way forward was agreed; however, it was not discussed whether to consider co-channel inter-cell inter-subband interference or not in the simulations, nor the corresponding modeling of such interference. Based on the discussions and preliminary simulation results presented in RAN1 (see for example R1-2210041), the co-channel inter-cell inter-subband interference may become the dominant source of degradation especially when high level of self-interference suppression is assumed (+ 100 dB). For this reason, we think it is important to understand and quantify the impact of this source of interference in RAN4 as it may be one of the main obstacles when it comes to SBFD feasibility. The following is therefore proposed:

Proposal 9: RAN4 to include the modelling of co-channel inter-site inter-subband interference for the studies on adjacent-channel coexistence and feasibility.

# Initial Simulation Results

Initial SBFD coexistence simulations results for the Scenario no. 1 are included in this section. This scenario models a FR1 urban macro deployment in which an operator is configured with NR TDD DL (victim) and either NR TDD DL or SBFD are adopted at the adjacent channel operator (aggressor). The target is to quantify the effects on the DL throughput on the victim operator when using SBFD as compared to using conventional TDD. Simulations follow the guidelines provided in [3] and the detailed assumptions are collected in Annex A. Table 1 shows the downlink throughput performance of the victim operator for the different aggressor configurations.

Table 1. DL throughput performance of victim operator

| Observation point | DL throughput with NR TDD aggressor | DL throughput with SBFD aggressor | DL throughput degradation |
| --- | --- | --- | --- |
| 5% | 5.3 Mbps | 5.3 Mbps | 0.0 % |
| 50% | 33.7 Mbps | 36.9 Mbps | -9.5% |
| 95% | 251.7 Mbps | 248.1 Mbps | 1.4% |
| Average | 68.2 Mbps | 71.2 Mbps | -4.3% |

The main learning of this study is that SBFD behaves similar as TDD and shows very similar inflicted DL throughput degradation. In fact, at some observation points, the DL throughput at the victim operator is higher if the aggressor adopts SBFD. This is due to the following:

- As compared to NR DL TDD, SBFD is expected to generate UE-to-UE CLI that can further degrade the DL performance of the victim. However, due to the sparse distribution of the UEs in the macro cell area (3 UEs per cell), the generated UE-to-UE CLI is marginal and has no effect on the DL SINR and/or the DL throughput.

- Additionally, the maximum DL transmit power of SBFD is lower than NR DL TDD since only 80% of the RBs are available for transmissions. This means that for a given value of ACLR, the aggressor NR TDD DL is expected to leak more power into the adjacent channel.

Observation 1: For scenario no. 1, initial simulation results show that SBFD shows marginal DL throughput degradation as compared to NR DL TDD.

Observation 2: The placement of the UE is expected to play a important role in the coexistence between NR DL TDD and SBFD. UE clustering might be required to fully understand the impact of SBFD.

# Conclusion

In this contribution the following observations and proposals were made:

Proposal 1: For the scenario with SBFD as victim and NR TDD DL as aggressor, adopt as aggressor baseline: “Option 1: No system in adjacent channel”

Proposal 2: For the SBFD coexistence evaluation, assume 100 MHz carrier bandwidth for FR1 and 200 MHz carrier bandwidth for FR2.

Proposal 3: For the UL/DL SBFD frequency split, assume a UL subband occupying approximately 20% of the channel bandwidth, while size of the guardband between UL and DL subbands is derived based on the requirements for guardband between adjacent channels as defined in TS 38.101-1. The following RB split is suggested:

- 104:5:55:5:104 (D:G:U:G:D) or 213:5:55 (D:G:U)  for a FR1 100 MHz carrier with 30 kHz SCS (273 RBs in total)

- 49:4:26:4:49 (D:G:U:G:D) or 103:4:26 (D:G:U) for a FR2 200 MHz carrier with 120 kHz SCS (132 RBs in total)

Proposal 4: For the grid shift considerations in the SBFD coexistence evaluation:

- Adopt 100% grid shift for adjacent-channel coexistence evaluations to understand the upper-bound of SBFD performance.

- Determine the minimum inter-operator isolation requirements for feasible SBFD operation and, as a next step, determine for which values of grid shift such inter-operator inter-cell isolation values may be achieved. Simulations may or may not be run with grid shift <100% depending on the outcome of the evaluation.

Proposal 5: For the BS antenna and MIMO configuration in FR1 Urban macro scenario, adopt the following:

- BS transmit power of 53 dBm per 100 MHz carrier

- Two options for antenna panel for SBFD: Option 1: (Mg,Ng,M,N,P)=(1,1,4,8,2), (dH,dV)=(0.5,0.8)λ, Option 2: (Mg,Ng,M,N,P)=(1,1,8,8,2), (dH,dV)=(0.5,0.8)λ

- Modeling of sub-arrays consisting of groups of 4 vertical antenna elements in each column of the antenna panel that are connected to the same RF chain.

Proposal 6: For the indoor/outdoor UE ratio, adopt Option 1-1: Re-use TR 38.828, which means FR1 Macro-to-Macro uses 20% indoor and 80% outdoor; FR2 Macro-to-Macro uses 0% indoor, Micro-to-Micro uses 80% indoor and 20% outdoor.

Proposal 7: For the UE distribution in UMa simulations, adopt Option 2-2: Consider clusters in UE dropping with the following clarifications in red.

-	Step 1: Randomly drop a cluster within a macro cell geographical area considering the minimum distance between macro TRP to cluster centre, e.g., 100m , where the size of each cluster is 120 x 50 (m);

- Note 1: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro TRP to cluster centre should be respected also for TRPs belonging to the other operator.

-	Step 2: 80% UEs are randomly and uniformly dropped within the cluster, and 20% UEs are randomly and uniformly dropped outside the cluster.

- Note 2: UEs dropped within the cluster are indoor and UEs dropped outside the cluster are outdoor.

- Note 3: The percentage of the UEs inside the cluster is achieved over the entire network and not in each individual macro cell area.

Proposal 8: For the traffic model, re-use the assumptions from TR 38.828: low load (RU 10%) and Full buffer.

Proposal 9: RAN4 to include the modelling of co-channel inter-site inter-subband interference for the studies on adjacent-channel coexistence and feasibility.

Observation 1: For scenario no. 1, initial simulation results show that SBFD shows marginal DL throughput degradation as compared to NR DL TDD.

Observation 2: The placement of the UE is expected to play a important role in the coexistence between NR DL TDD and SBFD. UE clustering might be required to fully understand the impact of SBFD.

# References

[1] RP-221352, “Study on Evolution of NR Duplex Operation”, CMCC

[2] R4-2214378, “WF on adjacent channel co-existence study”, Samsung

[3] R4-2214379, “WF on Simulation assumption for adjacent co-existence study”, CMCC

# Annex A: Simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
|  | Operator A (victim) | Operator B (aggressor) |
| Layout | Hexagonal grid, 3 sectors per site and 7 Macro sites. ISD = 500m | Hexagonal grid, 3 sectors per site and 7 Macro sites. ISD = 500m |
| Grid shift | 100% |  |
| System bandwidth per carrier | 100 MHz | 100 MHz |
| Duplexing mode | TDD | TDD/SBFD |
| SCS and TTI size | 30 kHz and 14 OFDM symbols | 30 kHz and 14 OFDM symbols |
| Carrier frequency | 3.95 GHz | 4.05 GHz |
| Total BS TX power | 49 dBm | 49 dBm |
| Total UE Tx power | 23 dBm | 23 dBm |
| Channel model | Macro gNB – outdoor UE: TR 38.901 UMa Macro gNB – indoor UE: TR 38.901 UMa + penetration losses Macro gNB – macro gNB: TR 38.901 UMa with replacement of the UE’s antenna height with gNB’s antenna height UE-UE: TR 36.843 with penetration losses between UEs from TR 38.802 Minimum coupling loss between macro gNBs: 50 dB Minimum coupling loss between UEs: 40 dB |  |
| Antenna Height | 25 m | 25 m |
| UE antenna Height | 1.5 m |  |
| Antenna gain + connector loss | 8 dBi | 8 dBi |
| Antenna gain of UE | 0 dBi |  |
| Antenna configuration | {M, N, P, Mg, Ng} = {8, 4, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ | TDD: {M, N, P, Mg, Ng} = {8, 4, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ |
|  |  | SBFD: {M, N, P, Mg, Ng} = {8, 4, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ |
| Panel mechanical tilt | -6 degrees | -6 degrees |
| Antenna configuration of UE | {M, N, P, Mg, Ng} = {1, 2, 2, 1, 1}; (dH, dV) = (0.5, 0.5)λ |  |
| Antenna element radiation pattern | According to 38.901 | According to 38.901 |
| Antenna element radiation pattern of UE | Isotropic |  |
| Number of UEs | 3 UEs per cell |  |
| UE dropping | Random |  |
| Traffic model | Full buffer (100% RU) |  |
| TDD/SBFD frame configuration | {DDDDU} | TDD: {DDDDU} |
|  |  | SBFD: {XXXXX}, with X = SBFD slot |
| BS/UE receiver | MMSE-IRC |  |
| BS noise figure | 5 dB | 5 dB + X dB, where X = 0 and 1 dB for TDD and SBFD, respectively |
| UE noise figure | 9 dB |  |
| UE speed | Static with fast fading speed = 3km/h |  |
| BS ACLR | - | 45 dBc |
| BS ACS | - | 46 dBc |
| UE ACLR | - | 30 dBc |
| UE ACS | - | 33 dBc |