3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216219

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM relaxations

Agenda item:	4.6.3.3   RRM measurement relaxations   [NR_redcap-Core]

Document for:	Discussion

Introduction

The discussion on RedCap core requirements related to RRM relaxations was continued at RAN4 #104-e [1]. The following agreement were achieved [2].

Agreements: Issue 2-1-1: Whether Scenario 8 should be allowed or not No; Rel-16 relaxation criterion; Rel-17 relaxation criterion; Applicability. 8; Rel-16 not-at-cell-edge; Rel-17 stationary. - Scenario 8 is supported Issue 2-1-1-1: Requirements for scenario 8 if scenario 8 is allowed For scenario 8, if UE can meet both Rel-16 not-at-cell-edge and Rel-17 stationary conditions, the UE is allowed to meet the requirements that are the most relaxed out of Rel-16 not-at-cell-edge and Rel-17 stationary RRM relaxation requirements. The most relaxed requirement is the Rel-17 stationary RRM relaxation requirements. Issue 2-1-2 Update the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED (question from RAN2 LS R2-2206418) Following is used as the baseline for replying LS: - From RAN4 perspective, it is reasonable to change the “Srxlev” for stationary criterion to “SS-RSRP” in RRC_CONNECTED Issue 2-2-1: On scaling factor based RRM relaxation under eDRX with PTW The scaling factor applies only when the relaxed evaluation/measurement time with such scaling factor on one carrier is not greater than single PTW window length. Issue 2-2-2: Clarification on the “4 hours” applying conditions of RRM relaxation under eDRX When UE fulfils both stationary and not at cell edge criterion, UE is allowed to relaxed measurement per 4 hours regardless of the eDRX cycle length. Issue 2-2-3: Higher priority inter-frequency measurement Relaxation - When only Rel-17 stationarity criterion is satisfied and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE performs measurement on high priority layer per 4 hour *Nlayer. - When both R17 criteria are satisfied, - When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, UE performs the measurement relaxation for lower, equal and higher priority frequency layers are the same, i.e., 4 hours. - When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE performs measurement on high priority layer per 4 hour *Nlayer.

The following issues were treated but not agreed [2].

Open issues: Issue 2-1-3 Clarification on RRM relaxation applying conditions - Proposals - Option 1: The RedCap UE shall not relax measurements on any of the neighbour cells when it has failed to meet the S criterion; In detail add the phrase: “In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”, for the cases configured eDRX and non-configured eDRX in clause 4.2B.2.2 in TS 38.133 (Nokia Ericsson CMCC Intel) - Option 2: RAN4 not to capture the additional highlighted text from the WF in the RAN4 specifications (Apple Qualcomm vivo xiaomi MTK) - Option 3: “If the UE is configured with and has fulfilled the stationary and not-at-cell-edge criteria in sections 4.2B.2.10.3 and 4.2B.2.11.3 and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells. (Ericsson) Agreements: No Issue 2-2-4: RRM measurement relaxation in SDT at inactive state - Proposals: - Option 1: RAN4 can define TA validation requirement as a single value = 640ms for SDT in RedCap with RRM relaxation. (Nokia) - Option 2: For FR2 640ms is not be sufficient and existing requirements shall be applied (Apple Huawei) - Option 3: FFS (Ericsson) - Option 4: Reuse the requirements specified for the configuration without eDRX (Huawei) - Option 5: Issue needs more clarification. Cannot catch the issues with SDT procedure in conjunction with neighbor cell measurement relaxation in RRC INACTIVE (Intel) - Option 6: Option 1 is used for FR1 and existing TA validation period is used for FR2 (MTK) Agreements: Follow conclusion of issue 1-1-2 of email thread [223]

In this contribution, we discuss the above open issues from last meeting.

Discussion

The above open issues from last meeting are discussed in this section.

Neighbour cell measurement relaxation when UE has failed to 	meet the S criterion

At RAN4#103-e, the discussion on idle mode requirements for RedCap UE was progressed in the endorsed draft CR [3]. The wording in clause 4.2B.2.2 according to agreed changes in [3] that in case the UE evaluates

“in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.”,

is in fact quite generic referring to measurement rules, rather than measurement configuration, and the clarification discussed at RAN4#103-e and RAN4#104-e

“In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”,

is appropriate in our view referring to the case of neighbour cell measurement relaxation introduced in clauses 4.2B.2.9 to 4.2B.2.11. As the serving cell suitability criterion is not met, we prefer that the UE measures, at the earliest, configured neighbour cells in order to identify suitable candidates for cell reselection. Thus, any measurement relaxation based on one or more measurement relaxation criteria being satisfied, is inappropriate in such case.

The following proposal is made.

RAN4 to proceed along option 1 for Issue 2-1-3, i.e. UE shall not relax measurements on any of the neighbour cells in case UE has failed to meet the S criterion.

RRM measurement relaxation for SDT at inactive state

At RAN4#104-e, the discussion on RRM measurement relaxation for SDT at inactive state was started [1]. This was concatenated with the discussion on TA validation requirements for SDT at inactive state. In our view, neighbour cell measurement relaxation, which is triggered by low mobility criterion, not-at-cell-edge criterion and/or stationary criterion, should be considered independent of TA validation requirements which depend on DRX or RAN eDRX cycle in FR1 but not in FR2. Thus, RRM relaxation of neighbour cell measurements, dependent on whether single stationary criterion is met (scaling factor 6 is applied), or single low mobility criterion is met (scaling factor 3), or stationary not-at-cell-edge criteria are met (long measurement period of 4 hours is applied), should also be maintained for SDT at inactive state, since SDT refers to serving cell rather than neighbour cells.

The following proposal is made.

RRM relaxation of neighbour cell measurements is allowed for SDT at inactive state and depends on whether configured relaxation criteria are met, e.g. whether single stationary criterion is met, single low mobility criterion is met, or stationary not-at-cell-edge criteria are met.

Conclusion

The open issues from last meeting on RRM relaxations have been addressed in this contribution.

The following proposals are made.

- RAN4 to proceed along option 1 for Issue 2-1-3, i.e. UE shall not relax measurements on any of the neighbour cells in case UE has failed to meet the S criterion.

RRM relaxation of neighbour cell measurements is allowed for SDT at inactive state and depends on whether configured relaxation criteria are met, e.g. whether single stationary criterion is met, single low mobility criterion is met, or stationary not-at-cell-edge criteria are met.

Both proposals are implemented in the companion CR [4].

- References

- R4-2214275, “Email discussion summary for [104-e][224] NR_redcap_RRM_2”, Moderator (vivo)

- R4-2215162, “WF on eDRX and RRM measurement relaxations requirements for Redcap UE”, vivo

- R4-2211038, "Draft CR: Changes to RedCap IDLE mode requirements for TS 38.133", RAN4#103-e, Ericsson

- R4-2216883, "CR 38.133 RRM relaxations in case of failed S-criterion and SDT for RedCap", RAN4#104bis-e, Nokia, Nokia Shanghai Bell