3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216232

Electronic Meeting, 10 October – 19 October 2022

Agenda Item:	4.6.5.1.1

Source:	Ericsson

Title:	Open issues on UE demodulation requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#104-e agreed with the way forward on RedCap UE demodulation and CQI reporting requirements [1]. This contribution discusses the remaining open issues on the UE demodulation requirements for RedCap.

# 2	Demodulation requirements

## 2.1	PDSCH demodulation requirements

Agreement: Introduce UE demodulation and CSI reporting requirements for 2Rx RedCap UE supporting HD-FDD in FDD bands as well as 1Rx UE - The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements. - Interested companies can evaluate if the same requirements can be applied for both 2Rx HD-FDD and 2Rx FD-FDD

RAN4 has agreed to define UE demodulation and CSI reporting requirements covering both HD-FDD and FD-FDD. It was agreed that the HD-FDD operation reuses TDD 15kHz DL/UL scheduling pattern, DDDSU, and applies the same requirements (SNR level to achieve 70% of the maximum throughput) as the FD-FDD operation. However some companies had concern whether the same requirements can be applied or not. We therefore ran the PDSCH simulations assuming HD-FDD (DDDSU) and compared with the result with FD-FDD.

Table 1	SNR to achieve 70% of the maximum throughput with FD-FDD scheduling and HD-FDD scheduling.

| 1Rx UE or 2Rx UE | MCS and rank | Propagation condition | Antenna configuration | Metric | FD-HDD | HD-FDD (DDDSU) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 1.0 | 1.0 |
| 1Rx UE | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 9.1 | 9.0 |
| 1Rx UE | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 13.6 | 13.6 |
| 1Rx UE | 256QAM 0.67 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 21.8 | 21.7 |
| 2Rx UE | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | -2.6 | -2.6 |
| 2Rx UE | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.0 | 5.0 |
| 2Rx UE | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 15.9 | 16.0 |
| 2Rx UE | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 20.8 | 20.8 |

Table 1 summarizes comparison of the simulation results between FD-FDD scheduling and HD-FDD scheduling. According to the simulation results, the difference between FD-FDD and HD-FDD is within 0.1dB with regard to the SNR to achieve 70% of the maximum throughput. We can conclude the same requirements can be applied for both HD-FDD and FD-FDD.

Observation 1: Difference between FD-FDD scheduling and HD-FDD scheduling about 70% of maximum throughput is negligible.

Proposal 1: For RedCap PDSCH demodulation requirements, the same requirements can be applied for both HD-FDD and FD-FDD.

# 3	Summary

Observation 1: Difference between FD-FDD scheduling and HD-FDD scheduling about 70% of maximum throughput is negligible.

Proposal 1: For RedCap PDSCH demodulation requirements, the same requirements can be applied for both HD-FDD and FD-FDD.

# References

- R4-2214394 “WF on RedCap UE demodulation and CQI reporting requirements”, Ericsson.