3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2216239

Electronic meeting, October 10 – October 19, 2022

Source: 	Huawei, HiSilicon

Title: 	Evolution of receiver blocking and AGC

Agenda Item:	6.17.2.2.1

Document for:	Discussion

- Introduction

At last RAN4#104-e meeting, self-interference modelling for gNB capable of SBFD operation has been discussed and the following agreements were made on Rx blocking and AGC.

Agreement on RSI dependency on Blocking and AGC（Question 1-4）:

- Proposal 1: The in-band blocking is suggested to applied as starting point to ensure the receiver of UL sub-band is not blocked due to DL sub-band transmission

  - Besides blocking, LNA and dynamic range can be FFS for receiver side

- Proposal 2: AGC may be applied to adjust the receiver gain to avoid ADC saturation if spatial isolation and analog IC, if found feasible, don’t provide enough reduction to self-interference. This may result in cost of an impact on sensitivity and potentially reduced coverage. However, it seems infeasible to model this in SLS.

Note: above proposal will not preclude other study regarding this issue. The size of any impact of coverage associated with AGC should in particular be elaborated.

In this contribution, we provide further analysis on these aspects.

- Discussion

Non-linearity

The in-band blocking is suggested to be applied as starting point for gNB capable of SBFD operation [2]. The in-band blocking in TS 38.104 for NR is -43 dBm, -38 dBm and -35 dBm for Wide Area BS, Medium Range BS and Local Area BS respectively.

And it was agreed to take 1 dB sensitivity degradation due to self-interference of DL transmission as starting point for system level evaluation and feasibility study. 1 dB is overall degradation, so we assume ~0.5 dB contribution from nonlinear IM3. The required IIP3 can be estimated as shown in following table:

Table 2-1: Rx IIP3 estimation

| IIP3 | WA BS | MR BS | LA BS | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- | --- | --- |
| In-band Blocking（dBm） | -43 | -38 | -35 | -43 | -38 | -35 |
| Noise floor （dBm） | -96 | -91 | -88 | -96 | -91 | -88 |
| REFSENS degradation （dB） | 6 | 6 | 6 | 0.5 | 0.5 | 0.5 |
| IM3 interference product （dBm） | -91.3 | -86.3 | -83.3 | -105 | -100 | -97 |
| Required IIP3 （dBm） | -18.85 | -13.85 | -10.85 | -12 | -7 | -4 |

Table 7.4.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + x dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE 1: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. For a BS capable of multi-band operation, "x" is equal to 6 dB in case of interfering signals that are in the in-band blocking frequency range of the operating band where the wanted signal is present or in the in-band blocking frequency range of an adjacent or overlapping operating band. For other in-band blocking frequency ranges of the interfering signal for the supported operating bands, "x" is equal to 1.4 dB. |  |  |  |  |

From Table 2-1, we can find the required IIP3 is to be improved about 7 dB, e.g. -12 dBm IIP3 is needed for WA BS. Rx IIP3 of -10 dBm can be considered as a representative nonlinear capability for current WA BS. And for Medium Range BS and Local Area BS, due to the reduced coverage range requirements, the receiver gain of front end is lower than that of WA BS hence the IIP3 can be higher. We think the needed nonlinear requirements can be covered by current BS hardware capability.

Observation1: the needed nonlinear requirements can be covered by current BS hardware capability.

ADC dynamic

As discussed in last meeting, the receiver gain (AGC) and ADC dynamic range need to be considered for RSI. AGC is used to handle large blocking interference, i.e. the receiver gain can be lower to avoid ADC saturation. According to the cascaded noise figure calculation, it will contribute more to the total noise figure when the gain of prior stage(s) is reduced.

NF=NF1 + (NF2-1)/G1 + (NF3-1)/(G1* G2)

Assuming ADC noise contributes ~ 0.5 dB degradation, the equivalent ADC noise is 9 dB below the noise floor at antenna connector. The tolerant blocking level can be calculated as in Table 2-2. It is found that there is 6 dB margin if assuming 0.5dB degradation of noise figure due to ADC dynamic.

Table 2-2:  blocking level estimation

| Noise floor（dBm） | -89 |
| --- | --- |
| ADC noise (100 MHz) | -98 |
| ADC SFDR | 74 |
| PAPR | 10 |
| max RMS input@antenna connector | -37 |

Observation 2: there is ~6 dB margin if assuming 0.5dB degradation of noise figure due to ADC dynamic.

Reciprocal mixing of phase noise

Due to the DL transmission and UL receive are operating at adjacent frequencies, the reciprocal mixing need to take into account for down conversion architecture. There is no reciprocal mixing issue for RF direct sampling architecture. At RF output 5GHz, the phase noise at 10 MHz offset can be better than -150 dBc/Hz [3]. To be conservative, we use -142 dBc in the estimation. The reciprocal mixing contribution can be calculated as -43-142 + 10*log(20)+60 ~=-112 dBm/20 MHz, which is 16 dB below the noise floor. The impact due to reciprocal mixing is negligible at -43 dBm blocking.

Observation 3: The impact due to reciprocal mixing is negligible when the blocking level is -43 dBm.

If we put them together, using representative parameters such as -10 dBm IIP3, 5 dB noise figure, -142 dBc phase noise@10 MHz offset, the estimated overall noise is about -95.5 dBm, which is 0.5 dB higher than the Rx reference sensitivity in normal operation without SBFD. The receiver selectivity is -43-(-105)=62 dB.

Table 2-2:  Rx estimation

| Rx IIP3 (dBm) | -10 |
| --- | --- |
| Blocker level | -43 |
| IM3 product | -109 |
| ADC noise | -109 |
| Phase noise | -140 |
| reciprocal mixing | -112 |
| Sum of noise caused by IM3, ADC and reciprocal mixing | -105 |
| Receiver noise floor (20MHz) | -96 |
| overall noise | -95.5 |

Observation 4: using the representative parameters, the overall noise caused by -43 dBm blocking is -95.5 dBm, which contributes ~0.5 dB degradation.

Observation 5: 62 dB receiver selectivity can be assumed for BS capable of SBFD operation.

- Conclusion

In the contribution, we provide discussion on the Rx side of evolution of NR duplex operation.

Observation1: the improved nonlinear requirements can be covered by current BS hardware capability.

Observation 2: there is ~5 dB margin if assuming 0.5dB degradation of noise figure due to ADC dynamic.

Observation 3: The impact due to reciprocal mixing is negligible when the blocking level is -43 dBm.

Observation 4: using the representative parameters, the overall noise caused by -43 dBm blocking is -95.5 dBm which contributes ~0.5 dB degradation.

Observation 5: 62 dB receiver selectivity can be assumed for BS capable of SBFD operation.

- Reference

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC

- “RAN1 Chair’s Notes,” 3GPP TSG RAN WG1#109-e, May 9th – May 20th, 2022.

[3]  https://www.analog.com/media/en/technical-documentation/data-sheets/adf5356.pdf