3GPP TSG-RAN WG4 Meeting # 104bis-e                                                       R4-2216253

Electronic Meeting, October 10 – October 19, 2022

Source:	Sony, Ericsson

Title:	Views on multi-Rx chain DL reception in FR2

Agenda item:	6.8.2

Document for:	Approval

# Background

The requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception was discussed in RAN4#104-e [1], and the following agreements were achieved from the system and the scope of the RF core requirements aspect [2]:

- Proposal: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

- Proposal: UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- Proposal: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

To further progress with the RF core requirements, it is necessary to agree on the UE assumption and how to specify the spherical coverage of multi-RX chains from different directions. In this contribution, we further analyze the two issues mentioned above.

# UE implementation assumptions

During the RAN4#104-e meeting, whether to use the term “antenna panel” in the RAN4 core requirement was discussed. The concept of antenna panels has been used in various places in RAN4 discussions, including setting the requirement for single AoA spherical coverage since Rel-15. While it is usually used to refer to a physical antenna array in FR2, it has never been used in the core requirements since the panel concept is highly up to the UE implementation. Meanwhile, it also comes to our notice that the term "antenna panel" is not used in the WI [3], and the requirement that is targeted to be set should be feasible to any UEs with multiple RX chains under two AoAs. Therefore, as a general conclusion, the concept of antenna panel should not be explicitly used in core requirements.

Proposal 1: The concept of antenna panel should not be explicitly used in core requirements

On the other hand, to progress on the core requirement, UE implementation assumptions are needed, where the term "antenna panel" can still be used during the discussion for convenience.

The work-item scope does not limit the angular separation or the spatial separation between the TRPs. Therefore, assuming at least two Rx chains per antenna panel or antenna array is reasonable. The UE can receive two streams from two different directions even though both arrived within the spatial coverage of the same antenna panel.

RF requirements should be defined for UEs with simultaneous reception from different directions with different QCL TypeD RSs. This means that the UE must at least indicate support of the capability simultaneousReceptionDiffTypeD-r16

| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11]. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

as a basic prerequisite. Then the UE shall be able to receive a single-layer transmission from two different TRP (different Type D states) at least with the same information transmitted on both. If the UE supports singleDCI-SDM-scheme-r16 in addition, then different information (layers) can received from the two TRPs. This suggest the following test configuration

According to the agreement in last meeting “UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions”, the UE is mandated to support both capabilities if it needs to pass the RF test. However, it has also come to our attention that it is possible to also verify the mTRP receiving performance by by carried out with an RMC with one layer  if the UE does not support  singleDCI-SDM-scheme-r16. Considering we are still at early stage of the WI, and the scenario is within the scope of the WI (up to 4 layers), we think it is worth that RAN4 also discuss this case.

Observation 1: if the UE supports simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16 the test can be carried our with an RMC with different layers on the two probes, if the latter is not supported, the test is also possible by carried out with an RMC with one layer, the same on the two probes.

Proposal 2: RAN4 can discuss how to treat the UE which does not support singleDCI-SDM-scheme-r16 in UE RF test.

# Consideration of spherical coverage test for devices with simultaneous reception from different directions

Two questions need to be answered to test the spherical coverage of devices with simultaneous reception from different directions: 1. How to set the two AoAs to derive the minimum requirement, and 2. How to define the EIS level with simultaneous reception from different directions transmission.

## AoA setup

### Full set AoA1 + Full set AoA2

Multiple candidate solutions can be proposed in RAN4#104-e regarding the AoA setups [2]. Ideally, one shall test all the possible pairs of AoAs to fully assess the spherical coverage performance of the device with simultaneous reception from different directions. However, this is not feasible considering the extreme test burden. On the other hand, this method may still be considered since it can indicate the UE performance under arbitrary UE orientation and angle of incoming signals. In addition, such a core requirement setting is stand alone from the test method discussion, while other methods that have been proposed so far (discussed later in the paper) may depend on the testability of the proposed AoA settings.

On the other hand, further study may need to be carried out later to identify the performance gap between the minimum requirement derived based on full set of AoA1 + full set of AoA2 and the actual value obtained based on the test method that is selected since it is clear that a full set of AoA1 + a full set of AoA2 is not testable.

Observation 2: full set of AoA1+ full set of AoA2 is not feasible from the testability aspect. However, it may still be used to derive the core requirement as it provides an overall assessment of device performance under arbitrary UE orientation and angle of incoming signals. However, a performance gap between the derived requirement and the actual test may appear.

Proposal 3: If full set AoA1 + full set AoA2 is selected, the performance difference between the derived requirement and the actual test setup defined in the end needs further study.

### Fixed AoA1(s) + Full set AoA2

Another possible way is to fix the direction of one DL (AoA 1) while sweeping the other DL directions (AoA 2), as shown in Fig. 1 (a). In practice, the AoA 1 can be fixed at a given direction towards the device under test (e.g., the beam peak direction). Then, the test procedure would be similar to the single AoA spherical coverage test since only one AoA or probe needs to be swept around the device. On the other hand, the testability of having a fixed relative position between the device and AoA1, in which the device is rotated during the measurement, needs to be studied and confirmed. Moreover, if this approach is adopted, more than one AoA1 may need to be considered to ensure the device can operate adequately in the field.

Observation 3: More than one AoA1 may need to be selected to ensure the device's performance in real life. In addition, testability issue may appear if one of the AoAs needs to have a fixed relative orientation towards the device while the device needs to be rotated.

Proposal 4: the testability of having one of the AoA fixed relative to the DUT must be confirmed before selecting this method.

(a)                                                                                                    (b)

Figure. 1. Possible test setup for two AoAs DL test to verify the UE spherical coverage of simultaneous reception from different directions. (a) one AoA fixed while the other AoA sweep. (b) both AoAs are swept with a fixed offset in between the probes.

### Fixed offset(s) between AoA1 and AoA2

An alternative way to sweep AoA 1 and AoA 2 simultaneously but with a fixed offset between the probes as shown in Fig. 1(b) and Figure 2. This appears to be the most feasible test setup. We understand that this method may face less testability issues compared to the one above. However, if this approach is adopted, how to choose the offset needs to be further discussed. One possibility is to measure with a narrow and wide angular probe separation to cover the cases of collocation and non-collocated TRP. Example angular separations are 30 degrees for a “narrow” separation and 135 degrees for “wide” separation, both with the probes in the same plane if feasible from a test setup standpoint. One layer is transmitted per probe, the same layer on both if the UE does not support singleDCI-SDM-scheme-r16.

Figure 2: fixed offset between the probles.

## EIS metric

From the system aspect, it has been agreed UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration. In this case, the sensitivity level on each direction/layer can be measured, and to correctly demodulate the signal, the sensitivity level on both AoAs must be above the minimum requirements that we will define for each given pair of AoAs. Therefore, the sensitivity level of each AoA pair can be defined as the worst sensitivity level (or highest sensitivity value) between the two sensitivity values.

We are considering a test with two probes from different directions with am EUIS level from each probe set at the DUT, then the requirement can be the sum of the EIS. The EIS in each direction is lowered such that the BLER requirement is met for the RMC.

The total EIS would be a relevant metric for both cases considered in Proposal 2. According to the 38.101-4 for radiated measurements, the plane of reference is the “input of the antenna array” and considering the definition of the Noc (same reference point) suggest the most relevant metric for the RF requirements for multi-RX is the total EIS for the layers/directions.

Another way to look at this issue is that the test equipment can lower the DL power from two probes simoutnaously. In this case, the one of the two Rx chains which has lower antenna gain become the bottle neck of the EIS performance and we are actually measuring the “worst EIS” among the two Rx chains.

Alternatively, set one of the probes to the 50 percentile EIS value and measure the EIS spherical coverage for the other probe/direction. The the measurement is repeated the with swapped roles of the probes. The common spherical coverage should exceed a specified value with the codewords demodulated at the required BLER. This should be feasible since we assume two separate RX chains, the power levels per direction can be different.

Proposal 5: RAN4 shall define the EIS metric for measuring the DL spherical coverage with simultaneous reception under different AoAs setups. One possibility is to use the total EIS from the two directions. Alternatively, the EIS is fixed from one direction at the 50%-ile for the eisting EIS spherical coverage requirement while the EIS spherical coverage is measured on the other.

# Performance analysis on the spherical coverage with simultaneous reception under different AoAs setups.

The above section discussed the possible way to verify the UE spherical coverage. To provide a general feeling of the spherical coverage of the device with simultaneous reception from two different directions, simulations based on a realistic smartphone prototype are provided. Two antenna panels with two Rx channels per panel are used for each simulated model, and the following cases are compared:

- Two AoAs (AoA1 and AoA2) are generated independently and randomly for each realization, and each of them is uniformly distributed on the sphere around the devices. It can represent the case that the device is located anywhere between the two TRP “uncorrelated”, while considering the random orientation of the device. In addition, it can also be equal to the case for full set of AoA1 + full set of AoA2 when the number of realizations is large enough.

- AoA1 is fixed at the beam peak direction, and AoA2 is swept through the sphere around the device

- AoA 1 and AoA 2 have fixed offsets (45 degrees and 135 degrees) in the horizontal plane and are swept together through the sphere around the device.

A couple of remarks on how we obtain the CDF simulation of the results:

- The two downlink signals are received on different Rx chains, and the gain from the two Rx chains in those two directions (Gain_AoA1 and Gain_AoA2) are simulated.

- For each measurement point or each realization, we only check if the minimum gain between the two directions is below a given threshold gain value, as shown in (1).

CDF = P (min (Gain_AoA1, Gain_AoA2)< Gain)                                                 (1)

Though the CDF simulation doesn't directly indicate the spatial coverage of simultaneous reception, it does mean the probability that the device can support simultaneous reception, which can also represent the UE performance in real life to some extent. As a reference case, we have also added the single AoA spherical coverage as shown .

Figure. 3. CDF of two AoA reception compared with single AoA spherical coverage under two different UE antenna panel configurations

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: if the UE supports simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16 the test can be carried our with an RMC with different layers on the two probes, if the latter is not supported, the test is also possible by carried out with an RMC with one layer, the same on the two probes.

Observation 2: full set of AoA1+ full set of AoA2 is not feasible from the testability aspect. However, it may still be used to derive the core requirement as it provides an overall assessment of device performance under arbitrary UE orientation and angle of incoming signals. However, a performance gap between the derived requirement and the actual test may appear.

Observation 3: More than one AoA1 may need to be selected to ensure the device's performance in real life. In addition, testability issue may appear if one of the AoAs needs to have a fixed relative orientation towards the device while the device needs to be rotated.

Proposal 1: The concept of antenna panel should not be explicitly used in core requirements.

Proposal 2: RAN4 can discuss how to treat the UE which does not support singleDCI-SDM-scheme-r16 in UE RF test.

Proposal 3: If full set AoA1 + full set AoA2 is selected, the performance difference between the derived requirement and the actual test setup defined in the end needs further study.

Proposal 4: the testability of having one of the AoA fixed relative to the DUT must be confirmed before selecting this method.

Proposal 5: RAN4 shall define the EIS metric for measuring the DL spherical coverage with simultaneous reception under different AoAs setups. One possibility is to use the total EIS from the two directions. Alternatively, the EIS is fixed from one direction at the 50%-ile for the eisting EIS spherical coverage requirement while the EIS spherical coverage is measured on the other.

# References

- R4-2214245 Email discussion summary for [104-e][134] FR2_multiRx_UERF Moderator (Qualcomm Incorporated)

- R4-2214457	WF on multi-Rx chain DL reception  Sony

- RP-221753	Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL	 Qualcomm Incorporated