3GPP TSG-RAN WG4 Meeting # 104-bis-e                	R4-2216259

Electronic Meeting, October 10 – 19, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on applicability of tests with different SCS

Agenda item: 	4.3.6.1

Document for: 	Discussion and Decision

Introduction

This paper presents Nokia’s view on RRM performance requirements with CCA for the operation between 52.6 GHz and 71 GHz.

Discussion

One important part of the RRM core requirements discussion is related to timing requirements. With the new sub-carrier spacings new timing requirements were defined for 480 kHz and 960 kHz, the CP has a reduced duration, meaning that the UL transmissions have to be more accurate in order to avoid inter-symbol interference and demodulation degradation.

Timing requirements defined for FR2-2 are related to the subcarrier spacing, with more accurate requirements applied for larger SCS.

As part of the RAN4#104, only one agreement was reached in relation to the applicability of the test configurations. This agreement was reached for RSSI requirements. In this agreement the UE is only required to be tested with one SCS. This is fine in general, since the performance of RSSI requirements not expected to vary significantly depending on the SCS. The agreement can be found in [1].

Agreement: RAN4 to define accuracy requirements for RSSI measurements with 120 kHz, 480 kHz, and 960 kHz - The UE is only required to be tested in one of the supported test configurations

We understand that the agreement that was reached in RAN4#104 cannot be generalised for other test cases. For requirement in which the performance depends on the SCS we need to discuss individually the applicability of the test configurations. This is particularly true for the timing requirements, where the UE needs to be more accurate when supporting a larger SCS.

The same conclusions from RSSI do not apply for other test cases, where the SCS have strong impact on the requirements.

The test configurations in which the UE is required to be tested must be discussed for each test case.

UE is required to be tested with the largest supported SCS for UL transmit timing test cases.

UE is required to be tested with the largest supported SCS for timing advance accuracy test cases.

Conclusion

In this document, Nokia’s views on the RRM performance requirements. The following observations and proposals are discussed:

- Timing requirements defined for FR2-2 are related to the subcarrier spacing, with more accurate requirements applied for larger SCS.

SCS have little impact on the RRSI measurement accuracy. Therefore it was agreed in RAN4 that the UE only needs to be tested for one of the supported test configurations.

The same conclusions from RSSI do not apply for other test cases, where the SCS have strong impact on the requirements.

- The test configurations in which the UE is required to be tested must be discussed for each test case.

UE is required to be tested with the largest supported SCS for UL transmit timing test cases.

UE is required to be tested with the largest supported SCS for timing advance accuracy test cases.

References

- R4-2214478, WF on NR extension to 71 GHz RRM performance requirements, Qualcomm