3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216288

Electronic Meeting, October 10 – 19, 2022

Title: 	Initial discussion on RRM impact due to introduction of APT 600 NR band

Source: 	Huawei, HiSilicon

Agenda item:	5.27.4

Document for:	Discussion

Introduction

In RAN#96-e, a WI [1] for introduction of APT 600MHz NR band was approved, and the related objectives of this WI are described as follows:

- Specify a new NR FDD operating band to include band numbering and core requirements to operate in APT region by considering that: - UE transmit: 663 - 703 MHz, BS transmit: 612 MHz – 652 MHz, - To support the following channel bandwidths and SCS as in the Table 2 below - UL channel bandwidth is limited to 20 MHz to limit the MSD as was the case with n71 - The use of UE power class 3 (23dBm). Table 2: Channel bandwidth and SCS parameters SCS (kHz); channel bandwidth (MHz). 15; 5; 10; 15; 20; 25; 30; 35. 30; 10; 15; 20; 25; 30; 35. - Develop a technical specification for the APT 600 MHz band for option B1 (as defined in table 1) for the APT region 600 MHz band with a view to developing the normative text on: - Operating band, channel bandwidth and system parameters- see Table 1 and Table 2 - BS and UE RF core requirement taking into account potential coexistence issues - RRM requirement

In this contribution, we will provide initial discussion on the RRM impacts of introducing a new NR FDD operating band.

Discussion

According to the WID [1], a new NR FDD operating band in FR1 will be introduced. RAN4 needs to study whether to reuse the existing RRM requirements or to define new RRM requirements for the new NR FDD band in FR1.

For RRM specification, the NR operating bands are divided into several band groups based on UE REFSENS requirements defined in RF specification. In TS 38.133, the NR band groups for FR1 are defined as Table 3.5.2-1 as follows:

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  | NR CCA10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation | Operating bands |
| A | NR_FDD_FR1_A | n1, n18, n24, n70, n744, n91, n92, n93, n94, n100 | NR_TDD_FR1_A | n34, n389, n39, n40, n50, n51, n53, n101 | NR_SDL_FR1_A | n67, n75, n76 | NR_CCA_FR1_A | - |
| B | NR_FDD_FR1_B | n65, n66, n743 | NR_TDD_FR1_B | n387 | NR_SDL_FR1_B | - | NR_CCA_FR1_B | - |
| C | NR_FDD_FR1_C | n30 | NR_TDD_FR1_C | n48, n771, n78, n79 | NR_SDL_FR1_C | - | NR_CCA_FR1_C | - |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - | NR_CCA_FR1_D | - |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41, n90 | NR_SDL_FR1_E | - | NR_CCA_FR1_E | - |
| F | NR_FDD_FR1_F | n266 | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - | NR_CCA_FR1_F | - |
| G | NR_FDD_FR1_G | n3, n8, n12, n13, n14, n20, n71, n85 | NR_TDD_FR1_G | n104 | NR_SDL_FR1_G | n29 | NR_CCA_FR1_G | - |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - | NR_CCA_FR1_H | - |
| I | NR_FDD_FR1_I | - | NR_TDD_FR1_I |  | NR_SDL_FR1_I | - | NR_CCA_FR1_I | n46 |
| J | NR_FDD_FR1_J | - | NR_TDD_FR1_J | n478, n96 | NR_SDL_FR1_J | - | NR_CCA_FR1_J | n96 |
| NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is only used for V2X SL service, the band is exclusively used for NR V2X in particular regions. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. |  |  |  |  |  |  |  |  |

Based on the UE REFSENS requirements for the new NR FDD band in FR1, RAN4 needs to determine which NR band group the new NR FDD band belongs to. Accordingly, the NR bands grouping for FR1 in Table 3.5.2-1 shall be updated.

Proposal 1: For APT 600MHz NR band, the NR bands grouping for FR1 defined in Table in TS 38.133 shall be updated according to the UE REFSENS requirements for the NR FDD band.

In TS 38.133, most RRM requirements are non-band specific defined except NR measurement accuracy requirements. For example, the absolute accuracy requirements for intra-frequency RSRP in FR1 are defined in Table 10.1.2.1.1-1 as follows:

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

It can be observed that the minimum Io value for some cases is defined for each NR band group. If the new NR FDD band belongs to an existing band group, then there is no need to update the NR measurement accuracy requirements. If the new NR FDD band belongs to a new band group, then minimum Io value for new band group needs to be added for NR measurement accuracy requirements.

The FR1 side condition in terms of minimum RP (received power) are dependent on NR band group. The minimum RP is derived from the minimum Io defined for NR measurement accuracy requirements. When the new NR FDD band belongs to a new band group, the minimum RP value for new band group also needs to be added for FR1 side condition requirements.

Proposal 1: RAN4 to study the RRM impacts due to introduction of APT 600MHz NR band including at least the following aspects:

- NR frequency bands grouping for FR1

- NR measurement accuracy requirements in FR1

- Side condition for FR1 measurements

Conclusions

In this paper we provided our initial analysis on RRM impact due to introduction of APT 600MHz NR band.

Proposal 1: RAN4 to study the RRM impacts due to introduction of APT 600MHz NR band including at least the following aspects:

- NR frequency bands grouping for FR1

- NR measurement accuracy requirements in FR1

- Side condition for FR1 measurements

Reference

- RP-221778,  “New WID on APT 600 MHz NR band”, Spark NZ, Qualcomm Inc