3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216312

Electronic Meeting, 10 – 19 October, 2022

Title: 	Discussion on other requirements for NTN RRM

Source: 	Huawei, HiSilicon

Agenda item:	4.2.5.2

Document for:	Discussion

Introduction

RRM requirements for NTN are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- RRC re-establishment

- UL spatial relation switch

In this paper we will provide our views on remaining issues for NTN requirements.

Discussion

RRC re-establishment

### Issue 6. Re-establishment Agreement: - Define NTN re-establishment requirements as in the tables below. Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); Cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; [6400]Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: Kmulti_SMTC is defined in clause 9.2C.5.1.. Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, I [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; [6400]Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: K_satellite is defined in clause 9.3C.4..

In [1] it was agreed to define NTN re-establishment requirements as in the tables above.

In particular, the re-establishment delay for one intra- or inter-frequency layers when the serving cell Es/Iot is < -8dB is defined as 6400ms. Based on our discussion paper [2], this number 6400ms is based on the assumption that UE would search for at most [8] satellites per carrier, and the time required is [8] times of the TN requirements (800ms).

We think the scaling factor of 8 can be confirmed. In the latest 38.331, NW can broadcast ephemeris for up to 8 satellites with ntn-NeighCellConfigList and ntn-NeighCellConfigListExt in SIB19. However, the scaling factor should be applied only when LEO are searched on the target frequency. When GEO are searched on the target frequency, the UE should be able to search all the cells as in TN, and no scaling factor is required. To not over-relax the requirements, we suggest to update the re-establishment requirements for the case with serving cell Es/Iot is < -8dB:

- 6400ms when LEO are searched on the target frequency, and

- 800ms when GEO are searched on the target frequency

Proposal 1: Update the re-establishment requirements for the case with serving cell Es/Iot is < -8dB:

- 6400ms when LEO are searched on the target frequency, and

- 800ms when GEO are searched on the target frequency

UL spatial relation switch

Delay requirements for UL spatial relation switch in NTN is defined in clause 8.12C.

It is noted that same as TN, the requirements are applicable for the case when the UE is requested to switch to a spatial relation with the higher layer parameter spatialRelationInfo associated to a DL RS. As such, the requirements apply when beamCorrespondenceWithoutUL-BeamSweeping is set to 1.

| beamCorrespondenceWithoutUL-BeamSweeping Indicates how UE supports FR2 beam correspondence as specified in TS 38.101-2 [3], clause 6.6. The UE that fulfils the beam correspondence requirement without the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall set the field to supported. The UE that fulfils the beam correspondence requirement with the uplink beam sweeping (as specified in TS 38.101-2 [3], clause 6.6) shall not report this field. | Band | Yes | N/A | FR2 only |
| --- | --- | --- | --- | --- |

Based on 38.306, the UE capability beamCorrespondenceWithoutUL-BeamSweeping is FR2 only. Therefore, we understand the requirements for UL spatial relation switch are for FR2 only. This is also aligned with the general RRM assumption that Rx is Omni-directional in FR1, so no Tx beam can be derived from an association to a DL RS. As NTN is only in FR1 in Rel-17, we suggest to remove the requirements for UL spatial relation for NTN to avoid any possible confusion.

Proposal 2: Remove the requirements for UL spatial relation switch for NTN.

Conclusions

In this paper we provided our views on remaining issues for NTN RRM requirements.

Proposal 1: Update the re-establishment requirements for the case with serving cell Es/Iot is < -8dB:

- 6400ms when LEO are searched on the target frequency, and

- 800ms when GEO are searched on the target frequency

Proposal 2: Remove the requirements for UL spatial relation switch for NTN.

Reference

- R4-2214471, WF on NR NTN RRM requirements, Moderator (Qualcomm Incorporated)

- R4-2213518, Discussion on other issues for NTN RRM, Huawei, HiSilicon