3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216330

# Electronic Meeting, 10 – 19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCs for PDC measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Perf |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Test cases for UE Rx-Tx measurement for PDC need to be introduced to verify the delay and accuray performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Based on the WF in R4-2210618, this CR introduces the following TC for UE Rx-Tx. - #4: UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR2 SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delay and accuray performance for UE Rx-Tx measurement for PDC cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.6.x1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

A.7.6.X1.2	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR2 SA

A.7.6.X1.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.X.2 for measurement accuracy in AWGN propagation condition in FR2 in standalone scenario.

The supported test configurations in listed in Table A.7.6.X1.2.1-1.

Table A.7.6.X1.2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

There is a single cell in the test: PCell (Cell 1) on RF channel 1 in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in TS 38.331 [2], shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.7.6.X1.2.1-2 and Table A.7.6.X1.2.1-3 respectively.

Table A.7.6.X1.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 | Cell 1 is the PCell. |
| RF Channel Number |  | 1 | 1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.2 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 20 |  |

Table A.7.6.X1.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | PDC-SRS.3 |  |
| Note 2 | dBm/SCS | 1 | -89 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
| TRS | dB | 1 | -Infinity | -3 |
| TRS | dB | 1 | -Infinity | -3 |
| TRS-RSRP Note 3 | dBm/SCS | 1 | -Infinity | -92 |
| Io | dBm/95.04 MHz | 1 | N/A | -58.25 |
| Propagation Condition |  | 1 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: TRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: TRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

A.7.6.X1.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.12.4.2. The UE shall be able to report the first Rx-Tx time difference measurement results no later than 40 ms from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.X.2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%.

<End of Change 1>