3GPP TSG-RAN WG4 #104bis-e	R4-2216340

October 10th ‒ 19th, 2022

Electronic Meeting

Agenda item:	6.2.2

Source: 	Qualcomm Incorporated

Title: 	Considerations on CA_n5-n8

Document for:	Discussion

# Introduction

Considerations on CA_n5-n8 are provided in this contribution.

# Discussion

The objectives of SI [1] are as follows:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands and simultaneous reception on two or three bands for the band combination of 700, 800 and 900MHz spectrum for smart phone form factor

  - The following band combinations will be considered. And the feasibility study on three band combination will start after the completion of feasibility study of all the fallback band combinations.

    - CA_n8-n20-n28 with uplink configurations of CA_n8-n20, CA_n8-n28, CA_n20-n28, and the fallback modes

    - CA_n5-n8 with uplink configuration of CA_n5-n8, and the fallback modes

Note1: Spectrum restrictions should be studied to solve overlap of band n5 downlink and band n8 uplink

Note2: The current filter is used as the baseline. Further study whether or not to have new solutions.

    - CA_n5-n28 (full range) with uplink configuration of CA_n5-n28.

Summary of band combinations considered in the SI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |

  - The following aspects need be studied

    - UE architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna

    - Performance due to impacts including inter-modulation products

    - Method to manage the inter-modulation product impacts

Note: Revisit in RAN#98 whether additional aspects need to be added.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

## Considerations on feasibility

By the end of this SI RAN4 should be able to conclude whether the proposed NR CA combos are feasible for smartphone form factor, and if yes, which are feasible RF architectures. It is worth to discuss under which conditions an NR UL/DL CA combination can be concluded to be feasible for smartphone form factor.

In our view, both the conducted domain and OTA domain must be feasible to call a combination feasible. Even 3GPP would be able to specify all REFSENS and all other requirements for a LB-LB-(LB) combination using certain RF architecture, but if the OTA domain using same RF architecture assumptions is not feasible due to for instance too poor antenna efficiency, then RAN4 should not conclude a combination feasible. Same thing works the other way around; if both conducted domain and OTA domain using the same RF architecture assumptions are feasible, then the combination should be concluded as feasible.

In our view, at least the following aspects together define the feasibility of an LB NR CA band combination.

-Antenna aspects (# of LB antennas, bandwidth ratio, etc)

-Conducted RF performance (MSD, etc)

-RF FE architecture (complexity, size, etc)

Observation: Antenna aspects, Conducted RF performance, and RF FE architecture together define the feasibility of an LB NR CA band combination

One of the important parameters is antenna isolation. 10dB has been the baseline for a very long time, recognizing 3GPP specifies minimum requirements. In the analysis below we provide results for both 10dB and 20dB to illustrate the impact in case the antenna isolation in certain UE design is larger than 10dB.

In the analysis below we calculate the antenna bandwidth ratios. Comments from as many OEM’s as possible on feasible antenna bandwidths/antenna interfaces would greatly help the discussion. Perhaps one way to tackle the feasibility aspects on antenna bandwidth would be to consider if there would be a sort of threshold value up to which a LB-LB CA could be considered feasible from antenna point of view. That is not at all a straightforward discussion but could be worth the effort.

Observation: Discussing and agreeing an antenna bandwidth threshold value for LB-LB CA could help in feasibility discussions

## CA_n5-n8

The frequency domain illustration of this combination is shown below. Green color depicts UL, and red color depicts DL for each of the bands.

Figure 1 CA_n5-n8 frequencies

In RAN4#104-e, the following agreements were done to address the overlap between bands:

- UE should support the full range of spectrum for single band operations on both n5 and n8

- Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

Table 1 Frequency restriction ranges for n5-n8

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

Using frequency restrictions shown above, the combination has no IMD’s. Thus the MSD studies are done for Cross-band cases only.

For this combination, we provide analysis and considerations for three RF architectures:

- Full n5/n8 RF filters with 3 antennas

- Dedicated RF filters for restricted frequencies with 2 antennas

- Dedicated RF filters for restricted frequencies with 3 antennas

In the analysis 10MHz UL is centred at 909MHz and 5MHz DL is centred at 877.5MHz. In this case the UL and DL are as close to each other as possible and the DL has the narrowest possible BW.

### Full n5/n8 RF filters with 3 antennas

The spectrum arrangements and the RFFE filter line-up are shown below. Black box with no will depict the frequencies for RF filters, and green/red coloured parts depict the frequency range restrictions.

Figure 2 Spectrum arrangements using full n5/n8 RF filters

Figure 3 RF FE filter line-up with 3 antennas

The bandwidth ratios are obviously normal for Ant 1 and Ant 2 as they cover only one normal LB each. Even the bandwidth ratio for Ant 3 is comparable to many other low bands.

Figure 4 Antenna bandwidth ratios for full band n5/n8 RF filters with 3 antennas

With full band RF filters, even when there is no overlap between the UL and DL spectrum, there are aspects that significantly impact the RF performance;

-n8 PA/Transceiver RX band noise is not attenuated by n8 Duplexer

-n8 TX leakage is not attenuated by n5 Duplexer/RX filter

Either of these two would prevent the usage of concurrent reception within 869...880 DL and 904…915 UL. Usually, the duplexer TX filter attenuates the RX band noise and PRX/DRX filter attenuate TX leakage by 40dB or so. When the filters can’t provide any attenuation, the only way would be to increase the antenna isolation respectively, i.e. to go from 10dB (40dB filter attenuation) to 50dB (0dB filter attenuation) which is not feasible.

Observation: CA_n5-n8 is not feasible using full band n5/n8 RF filters and concurrent reception within 869...880 DL and 904…915 UL

An alternative to concurrent reception within 869...880 DL and 904…915 UL could be to use scheduling restrictions, i.e. not to schedule concurrent reception within 869...880 DL and transmission within 904…915 UL. RF FE would not be the bottleneck, however both the UL and DL throughput would be degraded making the benefit of DL/UL CA very questionable as shown in the exemplary figure below.

Figure 5 Example illustration of TDM for n5 DL-n8 UL

Observation: RF FE would not be the bottleneck for CA_n5-n8 using full band n5/n8 RF filters and non-concurrent reception within 869...880 DL and 904…915 UL but both the UL and DL throughput would be degraded making the benefit of DL/UL CA very questionable

### Dedicated RF filters for restricted frequencies

The spectrum arrangements and the RFFE filter line-up are shown below.

Figure 6 Spectrum arrangements for restricted frequencies

In this case dedicated RF filters opens a big discussion of how to handle this kind of scenarios in RAN4. On high level, two alternative cases can be envisioned:

-Create new bands supporting just the restricted frequencies

-Not to create new bands but manage it in other means in RAN4

We need to recognize in RAN4 that in either case if CA between these restricted frequency ranges using concurrent reception within 869…880 and transmission within 904…915 is supported, UE needs both full n5 and n8 RF filters and dedicated filters for the restricted frequency ranges.

Observation: If CA between the restricted frequency ranges of n5 and n8 using concurrent reception within 869…880 and transmission within 904…915 is supported, UE needs both full n5 and n8 RF filters and dedicated filters for the restricted frequency ranges

When the feasibility analysis for different CA_n5-n8 architectures is done, RAN4 must do a very important decision on how to handle the non-overlapping frequency ranges within overlapping bands. Proponent operator’s input as well as inputs from other companies would be very valuable for further discussions.

In our view, at least the scenario where a 3GPP specified CA combination would need filters covering only parts of the specified bands should be avoided.

Observation: When the feasibility analysis for different CA_n5-n8 architectures is done, RAN4 must do a very important decision on how to handle the non-overlapping frequency ranges within overlapping bands. Proponent operator’s input as well as inputs from other companies would be very valuable for further discussions

### Dedicated RF filters for restricted frequencies with 2 antennas

Figure 7 RF FE filter line-up with dedicated RF filters with 2 antennas

In this case, the UL’s are purposedly transmitted from different antennas. This way both antennas can be tuned to optimize single UL and hence the TRP’s of the UL’s, possibly leading to better radiated performance. To limit the number of antennas into two, a triplexer design is used.

Observation: Transmitting UL’s from different antennas would allow better optimization of each TRP

The bandwidth ratio for Ant 1 is a bit larger than the ratio for any LB while the ratio for Ant 2 is moderate compared to some other LB’s (n28, n71)

Figure 8 Antenna bandwidth ratios for dedicated RF filters with 2 antennas

The following parameter values are used in the analysis. Please note that the triplexer characteristics are estimates at this point of time. The PA noise is also based on estimate at this point of time. To illustrate the impact of antenna isolation, we analyzed two different antenna isolations; 10dB and 20dB.

Figure 9 Cross-band MSD using dedicated RF filters and 2 antennas

Using estimated parameters, the REFSENS for 5MHz DL is -91.8dBm using conventional 10dB antenna isolation. Compared to 5MHz n5 REFSENS, the degradation is 5.2dB.

Observation: For NR CA using dedicated RF filters for restricted frequencies with two antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

### Dedicated RF filters for restricted frequencies with 3 antennas

The following RF filter line-up is used.

Figure 10 RF FE filter line-up with dedicated RF filters and 3 antennas

In this case, the UL’s are purposedly transmitted from different antennas. This way both UL antennas can be tuned to optimize single UL and hence the TRP’s of the UL’s, possibly leading to better radiated performance. To provide an alternative to triplexer-based solution (2.2.2), three antennas are used.

The bandwidth ratios for Ant 1 and Ant 2 are low even compared to other low bands while the ratio for Ant 3 is moderate compared to some other LB’s (n28, n71)

Figure 11 Antenna bandwidth ratios for dedicated RF filters with 3 antennas

The following parameter values are used in the analysis. Please note that the RF filter characteristics are estimates at this point of time. The PA noise is also based on estimate at this point of time. To illustrate the impact of antenna isolation, we analyzed two different antenna isolations; 10dB and 20dB.

Figure 12 Cross-band MSD using dedicated RF filters and 2 antennas

Using estimated parameters, the REFSENS for 5MHz DL is -92.3dBm using conventional 10dB antenna isolation. Compared to 5MHz n5 REFSENS, the degradation is 4.7dB.

Observation: For NR CA using dedicated RF filters for restricted frequencies with three antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

# Conclusion

Considerations on general feasibility of a CA combination was provided with the following observation

Observation: Antenna aspects, Conducted RF performance, and RF FE architecture together define the feasibility of an LB NR CA band combination

Observation: Discussing and agreeing an antenna bandwidth threshold value for LB-LB CA could help in feasibility discussions

Observation: Transmitting UL’s from different antennas would allow better optimization of each TRP

MSD analysis using coarse component assumptions was conducted for three different RF FE filter lineups.

Observation: CA_n5-n8 is not feasible using full band n5/n8 RF filters and concurrent reception within 869...880 DL and 904…915 UL

Observation: RF FE would not be the bottleneck for CA_n5-n8 using full band n5/n8 RF filters and non-concurrent reception within 869...880 DL and 904…915 UL but both the UL and DL throughput would be degraded making the benefit of DL/UL CA very questionable

Observation: If CA between the restricted frequency ranges of n5 and n8 using concurrent reception within 869…880 and transmission within 904…915 is supported, UE needs both full n5 and n8 RF filters and dedicated filters for the restricted frequency ranges

Observation: When the feasibility analysis for different CA_n5-n8 architectures is done, RAN4 must do a very important decision on how to handle the non-overlapping frequency ranges within overlapping bands. Proponent operator’s input as well as inputs from other companies would be very valuable for further discussions

Observation: For NR CA using dedicated RF filters for restricted frequencies with two antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

Observation: For NR CA using dedicated RF filters for restricted frequencies with three antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

# Reference

- RP-221464, “Revised WID: Study on enhancement for 700/800/900MHz band combinations for NR”, CATT