3GPP TSG-RAN4 Meeting #104bis-e	R4-2216371

# Electronic Meeting, October 10th – October 19th, 2022

Agenda item:	5.26.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Discussion on CA_n40-n41 synchronous RX_TX

Document for:	Approval

# Introduction

At RAN#96 a new Work Item on Simultaneous Rx/Tx inter-band combinations for NR CA/DC and LTE/NR DC in Rel-18 were approved [1]. The specification of TDD CA operation has been considered recently in RAN4 – 103 [2] with the introduction of CA_n38-n40 into 38.101-1. As flagged by other companies during RAN4 – 104 the aspects of where the operational band of n40 and n41 is in use and how their TDD scheme is deployed we present this paper capturing the information provided in relation to the flag of [2] and presents a correction to 38.101-1 that will prevent misinterpretation of the defined bands and their modes of operation, since new information on the use and practice of CA_n40-n41 has been shared.

# Discussion

As highlighted by other companies the introduction of CA_n38-n40 would require considerations of the mode of operation. 38.101-1 captures the TDD operation in two different notes:

NOTE 1:	Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability.

NOTE 9:	Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.

Due to legacy of low loss TDD filtering for the mode of operation of the combination CA_n38-n40 the NOTE 9 was added in Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) of 38.101-1. Making the captured ΔTIB,c for CA_n38-n40 applicable only for UE supporting Inter-band CA when supporting this without simultaneous Rx/Tx similar to CA_n40-n41.

Another table of 38.101-1 supports the decision surrounding the CA combination in terms of mode of operation. In Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands) of 38.101-1 the ΔTIB,c value of the CA combination CA_n40-n1 has a NOTE 3 at each band.

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n41 | 0.53 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

This NOTE 3 clarifies that ΔTIB,c relaxations for CA_n40-n41 is considered without simultaneous RX/TX, but as highlighted by other companies in RAN4 #104-e [3] the CA combination of n40 and n41 should not preclude the operation of simultaneous Rx/Tx in CA_n40-n41.

TS 38.101-1 section 6.2A.4.2, specifies that: “For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero”.

Observation 1: The current specification 38.101-1 states “Unless otherwise stated, ΔTIB,c is set to zero”

One way to understand this is that the current specification (i.e. TS 38.101-1) only includes ΔTIB,c values for CA_n40-n41 without simultaneous Rx/Tx and that for CA_n40-n41 with simultaneous Rx/Tx ΔTIB,c values are zero.

However, it can be argued that it is not clear if a requirement exists for the case when CA_n40-n41 is considered with simultaneous RX/TX.

Observation 2: In the current specification of 38.101-1 it is not clear if a requirement exists for the case when CA_n40-n41 is considered with simultaneous RX/TX.

One way to clarify this in the standard would be to remove the reference to NOTE 3, since CA_n40-n41 then shall be considered both with and without simultaneous Rx/Tx as also indicated in Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) where there is no NOTE that restricts the CA, while the NOTE 3 will remain for CA_n38-n40, since this is defined in Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands) of 38.101-1 including NOTE 9. This means that CA_n40-n41 adopts the ΔTIB,c test requirements for CA_n40-n41 with simultaneous Rx/Tx. Adopting this further relaxation is however undesired and unjustified.

In our opinion it would be beneficial to ensure Note 3 is interpret as an exception to the general statement that if no ΔTIB,c values are captured in the table then it is zero. This is illustrated in the TP#1 below:

***************************** Start of TP#1 ************************************

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n41 | 0.53 | 0.53 |
| NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. . |  |  |

***************************** End of TP#1 ************************************

Proposal 1a: Note 3 is modified as proposed in TP#1 to ensure it is interpreted as an exception to the otherwise zero ΔTIB,c.

Alternately Table 6.2A.4.2.3-1 may be updated similar to the practise of CA_n39-n41, with NOTE 2 restricting to single UL without simultaneous Rx/Tx. This by introducing a new Note for this type of operation. This is illustrated in the TP#2 below:

***************************** Start of TP#1 ************************************

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n41 | 0.53 / 011 | 0.53 / 011 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE [11]: Applicable for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx. |  |  |

***************************** End of TP#1 ************************************

Proposal 1b: Note [9] is introduced as shown in TP#2 to ensure ΔTIB,c are clear for this type of operation.

In our opinion it would be more beneficial not to introduce further notes to the specification and adopt Proposal 1a.

Regardless of which of the two options in Proposal 1a and 1b this unclarity in the specification is persistent also in Rel-17 specification. It is therefore proposed to not only correct this in Rel-18 but also in Rel-17.

Proposal 2: The clarification of applicable requirements for inter-band carrier aggregation with simultaneous Rx/Tx shall be introduced from Rel-17.

# Conclusion

Observation 1: Observation 1: The current specification 38.101-1 states “Unless otherwise stated, ΔTIB,c is set to zero”

Observation 2: In the current specification of 38.101-1 it is not clear if a requirement exists for the case when CA_n40-n41 is considered with simultaneous RX/TX.

Objective is to choose either proposal 1a or 1b:

Proposal 1a: Note 3 is modified as proposed in TP#1 to ensure it is interpreted as an exception to the otherwise zero ΔTIB,c.

Proposal 1b: Note [11] is introduced as shown in TP#2 to ensure ΔTIB,c are clear for this type of operation.

Proposal 2: The clarification of applicable requirements for inter-band carrier aggregation with simultaneous Rx/Tx shall be introduced from Rel-17.

# Reference

- RP-222648, Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18, TSG-RAN Meeting #96e

- R4-2209929, TP to TR 38.717-02-01: Addition of CA_n38-n40, TSG RAN WG4 Meeting #103-e

- R4-2214220, Email discussion summary for [104-e][102] R17_Maintenance_part, TSG RAN WG4 Meeting #104-e