3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216387

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	4.5.3.1.1

Source:	Ericsson

Title:	Discussion on the left open issue for PDSCH requirement for HST-SFN

Document for:	Discussion

# 1	Introduction

After the RAN4#104-e meeting, a WF [1] was agreed to carry all agreements for PDSCH requirement for HST-SFN.

In this contribution, we are going to give our views on the left open issues and provide our related simulation results. In the end we’ll give proposals

# 2	Discussion

## 2.1 SFN scheme A

As for the PDSCH requirement for HST-SFN scheme A, there is only one open issue left for further discussion:

### Sub-topic 2-2-2: MCS and Rank Agreement: - Further discuss and decide in next Ran4 meeting with following options - MCS 13 with Rank 2 - MCS 17 with Rank 2 - It’s encouraged companies can provide results for both options under FDD and TDD modes. - RAN4 will decide based on the alignment outcome among companies’ results and whether peak TP can be achieved or not.

Companies are encouraged to provide simulation results to see if the max throughput can be reached for MCS17. If so, the SNR value at 70% max throughput should be captured to assist further decision on the MCS value.

Following the simulation assumptions and parameter configurations agreed previously, we have provided our simulations results:

Table 2.1-1: Simulation results summary

| Case Number | Duplex & Antenna | MCS and rank | SNR @ 70% max Tput |
| --- | --- | --- | --- |
| 1 | FDD, 2x2 | 13, rank 2 | 8.6dB |
| 2 | FDD, 2x4 |  | 5.7dB |
| 3 | TDD, 2x2 |  | 10.1dB |
| 4 | TDD, 2x4 |  | 7.6dB |
| 5 | FDD, 2x2 | 17, rank 2 | Max TP can’t be reached |
| 6 | FDD, 2x4 |  |  |
| 7 | TDD, 2x2 |  |  |
| 8 | TDD, 2x4 |  |  |

Figures are showing below:

Results for MCS13

Figure 2.1-1 Performance of MCS13

For MCS13, the max throughput can be reached. The SNR at 70% max throughput for each case is within a reasonable range.

Observation 1: For MCS13, the max throughput can be reached. The SNR at 70% max throughput for each case is within a reasonable range.

Results for MCS17

Figure 2.1-2 Relative throughput of MCS17, FDD

Figure 2.1-3 Relative throughput of MCS17, TDD

From the throughput figure of MCS17, it is observed the max throughput cannot be reached for both FDD and TDD.

As it is also observed from the simulation results of MCS17, still only around 80% of maximum throughput can be achieved even at 50dB SNR, which is also indicating the max throughput cannot be reached.

Observation 2: For MCS17, the max throughput cannot be reached.

Given the observations above, we propose to select MCS13 for defining HST-SFN scheme A requirement.

Proposal 1: Set MCS13 for HST-SFN Scheme A requirement

# 3	Summary

In summary, we have following observations and proposals:

Observation 1: For MCS13, the max throughput can be reached. The SNR at 70% max throughput for each case is within a reasonable range.

Observation 2: For MCS17, the max throughput cannot be reached.

Proposal 1: Set MCS13 for SFN Scheme A requirement

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

## Annex: Simulation configurations

Here we provide our simulation assumptions for information.

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |  |
| CBW | 10 MHz | 40 MHz |  |
| Antenna configuration | 2x2; 2x4 |  |  |
| DMRS type | Type 1 |  |  |
| Number of DMRS symbols | 1+1+1 |  |  |
| TDD pattern | / | 7D1S2U, S: 6D 4G 4U |  |
| TRS configuration | 10ms, 2 slot pattern |  |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |  |
| Test metric | SNR @70% of maximum throughput |  |  |
| Maximum Doppler shift | 870Hz |  | 1667Hz |
| MCS | Scheme A: option 1: MCS13 option 2: MCS 17 Scheme B: MCS17 |  |  |
| Modeling of TRP pre-compensation | Assume perfect modeling of TRP pre-compensation |  |  |