3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216388

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	4.5.3.1.1

Source:	Ericsson

Title:	Simulation results for PDSCH requirement for HST-SFN

Document for:	Information

# 1	Introduction

After the discussion in RAN 104-e, companies agreed to introduce requirement for HST-SFN scheme B.

Agreement: - Introduce PDSCH requirement with HST-SFN scheme B with test applicability rule - [If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B]

Companies were encouraged to bring simulation results in this meeting for alignment.

For HST-SFN scheme A, simulation results are also required to see if the max throughput can be reached by configuring MCS 17.

In this contribution, we are going to share our simulation results for HST-SFN scheme A of two different MCS values and also share results of HST-SFN scheme B for alignment.

# 2	Discussion

## 2.1 SFN scheme A

Following the simulation assumptions and parameter configurations agreed previously, we have provided our simulations results:

Table 2.1-1 Summary for SFN scheme A results

| Case Nunmber | Duplex & Antenna | MCS | SNR @ 70% max Tput |
| --- | --- | --- | --- |
| 1 | FDD, 2x2 | 13 | 8.6dB |
| 2 | FDD, 2x4 |  | 5.7dB |
| 3 | TDD, 2x2 |  | 10.1dB |
| 4 | TDD, 2x4 |  | 7.6dB |
| 5 | FDD, 2x2 | 17 | Max TP can’t be reached |
| 6 | FDD, 2x4 |  |  |
| 7 | TDD, 2x2 |  |  |
| 8 | TDD, 2x4 |  |  |

## 2.2 SFN scheme B

Simulation results summary for SFN scheme B:

Table 2.2-1 Summary for SFN scheme A results

| Case Nunmber |  | MCS | SNR @ 70% max Tput |
| --- | --- | --- | --- |
| 1 | FDD, 2x2 | 17 | 11.2dB |
| 2 | FDD, 2x4 |  | 8.6dB |
| 3 | TDD, 2x2 |  | 12.2dB |
| 4 | TDD, 2x4 |  | 9.3dB |

# 3	Summary

In summary, we shared our simulation results for both scheme A and scheme B including 2Rx and 4Rx for FDD and TDD.

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

## Annex: Simulation configurations

Here we provide our simulation assumptions for information.

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Antenna configuration | 2x2; 2x4 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1+1 |  |
| TDD pattern | / | 7D1S2U, S: 6D 4G 4U |
| TRS configuration | 10ms, 2 slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds =700m; Dmin=150m |  |
| Test metric | SNR @70% of maximum throughput |  |
| Maximum Doppler shift | Scheme A: 870Hz |  |
| MCS | Scheme A: option 1: MCS13 option 2: MCS 17 Scheme B: MCS17 |  |
| Modeling of TRP pre-compensation | Assume perfect modeling of TRP pre-compensation |  |