3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216392

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	4.5.3.1.2

Source:	Ericsson

Title:	Simulation results for PDCCH enhancement for multi-TRP transmission

Document for:	Information

# 1	Introduction

After the last RAN4 #104 meeting’s discussion, companies agreed on introducing PDCCH repetition requirement without specify the receiver assumption, and additional margin will be added to compensate the performance gap (if any).

Besides, the aggregation level was agreed as 2 and the SNR setting for each TRP was agreed to not consider scaling factor during the test. Companies were encouraged to provide the simulation results for further alignment and decide the exact value for the additional margin.

Therefore, in this contribution, we are going to share our simulation results for both FDD and TDD with antenna configuration of 2x2 and 2x4.

# 2	Discussion

Simulation results with implementation of soft-combining are summarized below:

Table 2-1 Performance with soft-combining

| Duplex | Antenna configuration | Aggregation level | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- |
| FDD | 2x2 | 2 | -2.3 |
| FDD | 2x4 | 2 | -6.1 |
| TDD | 2x2 | 2 | -2.3 |
| TDD | 2x4 | 2 | -6.2 |

Simulation results without implementation of soft-combining are summarized below:

Table 2-2 Performance without soft-combining

| Duplex | Antenna configuration | Aggregation level | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- |
| FDD | 2x2 | 2 | -0.8 |
| FDD | 2x4 | 2 | -4.6 |
| TDD | 2x2 | 2 | -0.9 |
| TDD | 2x4 | 2 | -5.0 |

Note: the BLER for without soft-combining is calculated by the way of selective decoding where decoding is successful if any one PDCCH candidate is decoded successfully.

# 3	Summary

In summary, we shared our simulation results of both with and without soft-combining to assist further alignment of the results.

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

# Annex

Simulation assumption for PDCCH repetition

Table 1: Parameter configurations

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30kHz SCS |
| Repetition transmission schemes | FDM |  |
| CBW | 10 MHz | 40MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 |  |
| Aggregation level | AL=2 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | Non-Interleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |
| Operating SNR condition for selection AL | SNR > -4 dB |  |
| SNR setting for each TRP | The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP |  |
| Time offset/Frequency offset for each TRP | timing offset =-0.5us, frequency offset=200Hz | timing offset =-0.25us, frequency offset=300Hz |
| RB allocation | Frequency non-overlapping |  |
| Note: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TP1 and TP2 |  |  |