3GPP TSG-RAN4 Meeting #104bis-e	R4-2216393

# Electronic Meeting, 10 Oct - 19 Oct, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-4: PDCCH requirement with inter-slot repetition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Perf |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for PDCCH with inter-slot repetition |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new sections for requirements for FDD/TDD PDCCH with inter-slot repetition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements will not be defined for FDD/TDD PDCCH with inter-slot repetition |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1.4 (New) 5.3.2.2.4 (New) 5.3.3.1.4 (New) 5.3.3.2.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

##### 5.3.2.1.4	Minimum requirements for PDCCH with inter-slot repetition

The performance requirements are specified in Table 5.3.2.1.4-2, with the addition of test parameters in Table 5.3.2.1.4-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.1.4-1: Tests parameters

| Parameter | Value |
| --- | --- |
| Repetition transmission schemes | FDM |
| CCE to REG mapping type | nonInterleaved |
| REG bundle size | 6 |
| Time offset/Frequency offset of the second TxRP from the first TxRP | - timing offset = -0.5us, frequency offset = 200Hz |
| Frequency domain resource allocation for CORSET | Frequency non-overlapping |
| Note 1: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TRxP1 and TRxP2 |  |

Table 5.3.2.1.4-2: Minimum performance for PDCCH with 15kHz SCS

| Test num. |  | Bandwidth(MHz) |  | CORESET RB (Note 4) |  | CORESET duration |  | Aggregation level |  | Reference Channel |  | Propagation Condition (Note 1) |  | Antenna configuration and correlation Matrix (Note 2) |  | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pm-dsg (%) |  | SNR (dB) (Note 3) |
| 1 | 10 |  | 24 |  | 2 |  | 2 |  | R.PDCCH. 1-2.1 FDD |  | TDLA30-10 |  | 2x2, ULA Low |  | 1 |  | TBA |  |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 1>

<Start of change 2>

##### 5.3.2.2.4	Minimum requirements for PDCCH with inter-slot repetition

The performance requirements are specified in Table 5.3.2.2.4-2, with the addition of test parameters in Table 5.3.2.2.4-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.4-1: Tests parameters

| Parameter | Value |
| --- | --- |
| Repetition transmission schemes | FDM |
| CCE to REG mapping type | nonInterleaved |
| REG bundle size | 6 |
| Time offset/Frequency offset of the second TxRP from the first TxRP | timing offset =-0.25us, frequency offset=300Hz |
| Frequency domain resource allocation for CORSET | Frequency non-overlapping |
| Note: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TxRP1 and TxRP2 |  |

Table 5.3.2.2.4-2: Minimum performance for PDCCH with 30kHz SCS

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.1 FDD | TDLA30-10 | 2x2, ULA Low | 1 | TBA |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 2>

<Start of change 3>

##### 5.3.3.1.4	Minimum requirements for PDCCH with inter-slot repetition

The performance requirements are specified in Table 5.3.3.1.4-2, with the addition of test parameters in Table 5.3.3.1.4-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.3.1.4-1: Tests parameters

| Parameter | Value |
| --- | --- |
| Repetition transmission schemes | FDM |
| CCE to REG mapping type | nonInterleaved |
| REG bundle size | 6 |
| Time offset/Frequency offset of the second TxRP from the first TxRP | timing offset = -0.5us, frequency offset = 200Hz |
| Frequency domain resource allocation for CORSET | Frequency non-overlapping |
| Note 1: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TRxP1 and TRxP2 |  |

Table 5.3.3.1.4-2: Minimum performance for PDCCH with 15kHz SCS

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 2x4, ULA Low | 1 | TBA |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 3>

<Start of change 4>

##### 5.3.3.2.4	Minimum requirements for PDCCH with inter-slot repetition

The performance requirements are specified in Table 5.3.3.2.4-2, with the addition of test parameters in Table 5.3.3.2.4-1. The downlink physical channel setup according to Annex C.3.1.

Table 5.3.2.2.4-1: Tests parameters

| Parameter | Value |
| --- | --- |
| Repetition transmission schemes | FDM |
| CCE to REG mapping type | nonInterleaved |
| REG bundle size | 6 |
| Time offset/Frequency offset of the second TxRP from the first TxRP | timing offset =-0.25us, frequency offset=300Hz |
| Frequency domain resource allocation for CORSET | Frequency non-overlapping |
| Note: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TxRP1 and TxRP2 |  |

Table 5.3.2.2.4-2: Minimum performance for PDCCH with 30kHz SCS

| Test num. | Bandwidth(MHz) | CORESET RB (Note 4) | CORESET duration | Aggregation level | Reference Channel | Propagation Condition (Note 1) | Antenna configuration and correlation Matrix (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) (Note 3) |
| 1 | 40 | 48 | 2 | 2 | R.PDCCH. 2-2.1 FDD | TDLA30-10 | 2x4, ULA Low | 1 | TBA |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 Note 4: CORESETs from TRxP #1 and TRxP #2 should not be overlapped |  |  |  |  |  |  |  |  |  |

<End of change 4>