TSG-RAN Working Group 4 (Radio) meeting #104-bis-E	R4-2216404

Electronic Meeting, 10th – 19th October 2022

Source:	Ericsson

Title:	SBFD gNB Radio and antenna considerations

Agenda item:	6.17.2.2.1

Document for:	Approval

# Introduction

During RAN4#104-e, RAN4 discussions around SBFD were initiated. SBFD is a new and interesting BS technology and needs careful evaluation. The gain mechanisms of SBFD (UL coverage gain due to longer UL transmission, latency reductions due to continuous UL, ability to switch DL and UL capacity) are fairly intuitive. The ability to obtain gain for SBFD depends fully on the ability to mitigate interference mechanisms such as BS TX and RX self-interference, inter-sector interference and inter-site interference (from the same and different operators). The interference mechanisms are complex and, since they directly impact the ability to obtain gain, should be modelled and investigated carefully as part of a feasibility study and gain evaluation. Impacts on transmitter radio, receiver radio, antenna design, interference suppression and cancellation algorithms need proper assessment. It has been shown that without proper modelling of transmitter impairments, receiver impairments and antenna modelling to capture the isolation due to near field effects there is no reasonable approach to evaluate the feasibility and performance of different cancellation schemes and solutions.

In [1], a discussion around transmitter, receiver modelling and high isolation advanced antennas including the impact of beamforming was initiated. As SBFD will affect the BS only, important aspects such as multi-carrier operation, deployment aspects such as isolation on sectorized sites and energy efficiency was further discussed.

In this paper, an updated analysis for FR1 based on discussions in [1] is presented. In addition, a discussion and analysis considering modelling of radio and antennas for FR2 is elaborated in detail.

# Discussion

In this section, the radio and antenna modelling aspects for both FR1 and FR2 are further discussed. The analysis considers a BS implementation that is compliant to current 3GPP BS specifications as the baseline. For some requirements, realistic models for e.g., transmitter, a model capturing net model of PA+DPD as well as CFR + channel filtering are presented.

Detailed studies using advanced simulations to capture the high antenna isolation behaviour (inter and intra gNB) and performance in presence of beam forming is also presented in this section.

## Transmitter aspects and modelling

### FR1

During RAN4#104-e, we presented a discussion and model for the transmitter part of a SBFD BS. It is important to capture the main elements of a transmitter for the following reasons:

- The TX leakage signal to the RX sub-band is a product of distortions from a BS transmitter. In order to characterize the feasibility and complexity of interference cancellation algorithms, it is necessary to have a model of what the signal is that needs to be cancelled. Otherwise, it is not possible to investigate and benchmark differing proposals for interference cancellation suppression.

- Apart from interference cancellation, an alternative attractive means to reduce TX distortion is to reduce the interference from each transmitter by means of either reducing the PA output level or employing a more complex DPD. To benchmark the gain and complexity of such schemes, a realistic PA model is needed.

The proposed transmitter model is depicted in Figure 2.1.1-1 containing PA+DPD and CFR+channel filtering.

Figure 2.1.1-1: Summary of initial example of gNB transmitter modelling.

Observation 1: To properly model the TX interference signal to be supressed or cancelled from a BS, at least the CFR process and PA+DPD combination need to be modelled realistically.

PA and DPD model

High output power is obtained from a BS using a PA. To reduce the emissions from a PA, a DPD is often used. [1] offered a more detailed explanation of PA and DPD modelling.

For FR1, we believe that it is useful to have both a PA model and a combined PA+DPD model. The combined PA+DPD model enables an effective means to provide a realistic signal for evaluation of interference cancellation performance. Combining PA and DPD into one model is a useful means of reducing simulation complexity. We provide the coefficients for a polynomial based PA+DPD model in annex A. The model is based on a DPD suitable for meeting the RAN4 ACLR requirement.

Apart from a PA+DPD model, it is useful to capture a model for the PA only. The reason is that it may be useful to apply a more complex DPD that suppresses interference beyond what is required for the RAN4 requirement. An increased complexity DPD at each transmitter may be preferable to high complexity interference cancellation, since IC complexity in an AAS scales with the product of the number of transmitters and number of receivers, whereas DPD complexity scales only according to the number of transmitters. By settling on a PA model, it is possible to propose and alternative PA+DPD with a more powerful DPD. We encourage such proposals, including the model and an estimation of the complexity of a mode powerful DPD.

For FR2, to achieve similar level of additional interference suppression beyond what is required for the RAN4 requirement, taking into account the number of transmitters and the signal bandwidth, DPD is likely to be more complex.

Proposal 1: Adopt the PA models in annex A for FR1 when considering TX interference suppression or cancellation.

Crest factor reduction

In modern BS, crest factor reduction is a crucial scheme for operating PAs with maximum efficiency. We have described crest factor reduction in more detail in our previous contribution [1]. A model for crest factor reduction of the input signal is proposed as follows:

- Apply hard clipper on the signal:

xn=xn,if x(n)≤LLej∠x(n),otherwise

where, for an example of clipping at 5.7 dB PAPR, the parameter L=105.7/10⋅avgxn2. CFR, regardless of the exact approach used, generates both distortions to the signal and out-of-band emissions. It is therefore necessary to apply filtering after the hard clipper to suppress the out-of-band emissions.

The transmitter characteristics satisfying the ACLR and EVM requirements based on proposed net models of PA+DPD and CFR + Channel filtering is presented in Figure 2.1.1-2.

Figure 2.1.1-2: Example of spectra of OFDM signals with CFR processing

Proposal 2: Adopt the transmitter model containing net PA+DPD and CFR+channel filtering for evaluation of TX interference suppression or cancellation

Interference Cancellation (IC) complexity

Digital interference cancellation has been postulated as a means for reducing several types of interference:

- TX distortion from the gNB transmitter chain via the antenna array

- TX distortion from the gNB transmitter chain due to reflections from objects in the environment

- RX distortions caused by the receiver chain

In order to cancel TX distortion from the gNB transmitter chain, the interference cancellation needs a copy of the signal at each transmitter and a model for the coupling within the array and from environmental reflectors. Furthermore, the IC needs to apply the same filtering as occurs in the receive chain together with a model for any in-band receiver effects.

To estimate the complexity of the IC, it is useful to consider the sampling rate and the lengths of filters needed. For a 100 MHz carrier, the sampling rate is at 122.88 MHz. Hence, one time sample corresponds to 2.44 m. To cover potential strong reflectors within even a small radius, the adaptive filters need to keep track of several taps if no over sampling is used. In reality, delays of the reflected signals are generally not aligned with the sampling grid perfectly. Over-sampling is likely needed to obtain adequate cancellation performance. The filter lengths will scale with the over-sampling rate accordingly. The analog filters also add to the effective lengths of the overall channel responses.

Thus, the complexity of an individual IC is based on the sampling rate increased by the number of filter taps, considering the need to replicate receiver filtering and to capture environmental reflections.

For TX cancellation, the signal from each transmitter is needed. Since the coupling within the array and phase relationships differ, potentially a filter is needed for each transmitter at each receiver. Due to the possibility of some transmitters being dominant, it may be possible to cut the number of filters; this will depend on array geometry and needs further study.

Taking some assumptions for array sizes

- LA BS with (Mg, Ng, M, N, P)  = (1,1,1,2,2)

- MR BS with (Mg, Ng, M, N, P) = (1,1,2,4,2)

- WA BS with (Mg, Ng, M, N, P)  = (1,1,4,8,2)

where M, N indicate numbers of subarrays and each subarray contains 3 antenna elements. That is, the typical numbers of TX chains are NT= 4, 16, and 64 for the LA, MR and WA BS classes, respectively. For SBFD system implemented to achieve “same gain”, then the number of the RX chains equals that of the TX chain: NR=NT. The complexity of the digital cancellation scales with NT2L. The number of TX-RX combinations is 16, 256 and 4096 for LA, MR and WA

That is, the complexity of digital self-interference cancellation for the three BS classes is potential scaling with a factor of 16, 256, and 4096 for the LA, MR and WA BS classes, respectively.

For cancelling of RX interference, the IC needs a model of the transmitted signal from each transmitter, coupling and the LNA non-linearities. At each receiver, a model for the combined, DL sub-band signal is needed, including environmental reflections, which will require a filtering. The complexity of this part of RX cancellation will scale with 16, 256 or 4096 for the LA, MR and WA BS classes similarly as for the TX distortion cancellation. The filtering applied to account for LNA and RX chain non-linearities will scale according to the number of receivers.

Observation 2: Interference cancellation complexity can scale exponentially with the number of transmitters and receivers.

Observation 3: For wide area BS, the complexity of basic interference cancellation is prohibitive.

Proposal 3: Proposals for interference cancellation should be accompanied by estimates of complexity.

## 2.2 Receiver aspects and modelling

### 2.2.1 General receiver considerations

[1] presented a general overview of considerations on BS receiver at RAN4#104-e. The most important aspects of receiver performance in the context of full duplex are as follows:

- Typical BS receivers have an analogue filter covering the whole band, and hence both the wanted signal and interfering signals within the band are processed by the analogue front end of the receiver. In the context of SBFD, this means that the power from the DL sub-band (after spatial suppression in the antenna) is processed in the analogue parts of the receiver.

  - It may be possible to consider a tunable narrowband filter in front of the receiver, however the complexity of such a filter is significant, and more importantly a filter prior to the LNA will directly increase the receiver noise figure.

- Spectral regrowth due to non-linearities in the LNA can cause IM products from the transmit sub-band to fall into the receive sub-band. This spectral re-growth must either be avoided by using a higher linearity LNA than in a conventional BS or by means of interference cancellation of the receiver distortions.

Figure 2.2.2-1 Spectral regrowth

- Reciprocal mixing of phase noise leads to the phase noise spectrum from the local oscillator being superimposed over the DL sub-band, thus creating interference in the UL sub-band.

- The ADC dynamic range needs to be sufficient to process both the residual DL sub-band and the UL signal

The above aspects will be considered in the following sections. Some other aspects that are of secondary importance and not considered in the current analysis include:

- Other ADC effects, such as timing jitter and differential non-linearity

- Mixer related distortion

- LNA compression point

In addition, due to presence of high power in DL sub-bands, the receiver architecture impacts capabilities to employ filtering to mitigate the DL power and protect the ADC from being saturated. In the next section an overview of different receiver architectures is described.

### 2.2.2 Receiver architectures

There are various receiver structures used for BS. Depending on the deployment as well as capabilities such as beam forming for massive MIMO type of BS, the trend in the industry moves towards usage of some specific structures. When studying the SBFD receiver aspect, due to self-interference and depending on the deployment, the need to handle large dynamic range could require filtering to protect the ADC in the receiver chain. The possibility for such analog filtering is different and depends on the receiver structure which is further discussed in this section.

Observation 4: The feasibility and complexity of filtering solutions depends on the receiver architecture.

Super heterodyne

Figure 2.2.2-1 Super Heterodyne receiver

The super heterodyne was for a long time the major receiver architecture for base station applications and it uses one or more stages of frequency down conversion. The sampling of the analog data is in the real domain and occurs at an intermediate frequency. Being at an intermediate frequency makes it more difficult to integrate and usually requires some sharper anti-aliasing filtering compared to the Homodyne. This is due to the mixer output having significant spuriouses at the output that is generated from multiples of the input frequencies times the LO (Local Oscillator) frequency. This also makes the need of quite good filtering at RF to attenuate out-of-band interferes that could be mixed down into the wanted channel in the same way. Note that sufficient filtering at RF level requires significant guard and high Q resonators. The filtering at IF have either been SAW-filters or if ADCs with very high over-sampling is achieved some of the filtering for spuriouses can be moved into digital domain and a more relaxed anti-aliasing filter can be used. Being at an intermediate frequency for the ADC also gives the possibility to do frequency planning around harmonic distortion to make sure that a clean spectrum is achieved and no second or third order harmonics fold back into the wanted spectrum. The super heterodyne is good in performance up to around 200MHz Maximum RF bandwidth and therefore a good choice when single band base-stations at lower frequencies were concerned. Note that for many bands above 3 GHz, the need for maximum RF bandwidth is larger than 200 MHz.

Due to limited guard between DL sub-band and UL sub-band and limitation in SAW filter technology, it would be difficult to mitigate some of DL sub-band filters in IF domain.

Observation 5: The super-heterodyne receiver architecture enables frequency planning to avoid LO distortions. It is not easily compatible with low complexity analog filtering that does not require large guard between UL and DL sub-bands.

Homodyne / Zero-IF

Figure 2.2.2-2 Homodyne receiver

The homodyne architecture has been applied lately in the base stations and been so for a long time in the handsets. The homodyne is using a down conversion to 0 Hz with IQ-demodulation and the spectrum is then sampled by the ADC in the complex domain. This puts stringent requirements on keeping phase and amplitude perfectly matched between the I- and Q-branches to not create quadrature errors. This is usually not a problem for a single carrier receiver, since a low-pass filter can be used to attenuate everything higher in frequency and at the same time work as an anti-aliasing filter. But for a multi-carrier receiver which is the most common requirement for BS, the spectrum sampled is larger and one carrier could end up in the image frequency range of another carrier, since only one carrier could be down-converted to 0Hz. The match between I- and Q- needed by the IQ-branches in the single carrier receiver is much less compared for the multi-carrier receiver. The benefit of the Homodyne is that larger bandwidth is possible to be sampled and being at a low frequency means that fairly good filters can be implemented on the same chip as the converters and mixers, giving a highly integrated component. The Homodyne architecture works well up to a few hundred MHz in RFBW and multi-band receiver applications are more feasible, but also smaller single band radios as well. When going above 400-500MHz RF-BW it starts to become much difficult to have a good compensation for the IQ-errors that might make other solutions more beneficial from an energy consumption point of view. For SBFD, homodyne receiver would possibly allow for some analogue filtering at baseband to protect the ADC at expense of higher complexity. This approach would work for a single carrier receiver and will limit the flexibility of using different UL sub-band sizes unless complex switchable filter banks are incorporated. The filtering solutions discussed here will apply for single carrier while solutions for multi-carrier receiver become much more complex considering that multi-carrier receiver implementation is quite a common requirement for BS.

Observation 6: The homodyne type of receiver architecture can be more suited to pre-ADC filtering to prevent ADC overload; however, the complexity of such analog filtering needs careful consideration in a multi-carrier BS.

Direct RF-sampling

Figure 2.2.2-3 Direct RF sampling receiver

The Direct RF-sampling architecture is starting to be used in AAS/Massive MIMO radios but less frequently in Classic/non-AAS Radios, due to the performance difference needed. The ADC is sampling the spectrum at RF without any down conversions and by that in the real domain. Many of the chipset suppliers are selecting this architecture for their future roadmaps and strategies as direct RF sampling structures provides the possibility to get a very wideband design. In addition, this creates a very flexible architecture for multi-carrier operation. For this, it will require very high sample rates compared to the other structures to ensure that one is not located in a too high Nyquist zone and still have margin to the different alias-zones for filtering of noise and spuriouses before folding into the wanted spectrum. To achieve needed high sample rates, the ADCs are usually interleaved with multiple ADC-devices sampling at a lower rate per ADC but spread out in time so that the total samples are the same as a single ADC with the full sample-rate. This gives a similar problem like the Homodyne that amplitude and timing need to be perfect between each sub-ADC to make sure that the spectrum becomes as clean as possible. If spectrum cannot be kept clean, there will be artefacts from the interleaving creating spuriouses that are replicas of the largest signal, which is usually an interferer.

To protect the ADC, the RF filter would need to have tunable notches to filter out the DL subbands, and the narrow guard band between DL and UL subband would require filters with very high Q. Tunability at RF is challenging; moreover, to cater for multi-carrier operation with flexibility on which carriers(s) are deployed with SBFD leads to an infeasible filtering solution.

Observation 7: Filtering in the analogue domain is complex for advanced, direct sampling receivers as it needs to be done at RF.

### 2.2.3	Receiver linearity

FR1 analysis

A key aspect of receiver performance is the linearity of the LNA and analogue front end. As mentioned above, spectral regrowth due to non-linearity leads to IM parts from the DL sub-band desensitizing the UL sub-band. To examine the impact of analogue linearity and any needed improvement for SBFD, it is useful to consider the IIP3. The IIP3 performance needed to just meet the RAN4 receiver IM requirements is shown in table 2.2.3-1 for each BS class.

Table 2.2.3-1 Minimal LNA IIP3dB parameter estimation based on RAN4 in-band receiver intermodulation requirements.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 noise floor | ~-95.6 | ~-90.6 | ~-87.6 |
| B | RAN4 RX intermodulation interferer power | -52 | -47 | -44 |
| C | RAN4 allowed desensitization | 6 | 6 | 6 |
| D | Implied DNR = 10*LOG10(10^(C/10)-1) | 4.7 | 4.7 | 4.7 |
| E | Implied gain-normalized distortion = A+D | -90.9 | -85.9 | -82.9 |
| F | Estimated minimum IIP3dB = (3*B-E)/2 | -32.6 | -27.6 | -24.6 |

Typically, BS will be designed with 5-10dB better IIP3 than needed to meet the minimum requirement. In the following analysis, we assume 10dB better IIP3 than needed to meet the minimum RAN4 requirement:

- -22.6 dBm IIP3 for wide area BS

- -27.6 dBm IIP3 for Medium Range BS

- -24.6 dBm IIP3 for LA BS

Note that assuming 10dB better IIP3 is pretty optimistic and may already imply a greater complexity and power consumption than typical for real BS.

The SBFD DL signal will be attenuated by the antenna suppression. The antenna suppression must be obtained over the whole band in order to have a flexible and deployable BS and needs to be obtainable over a range of different combinations of beam directions. Our analysis shows that around 65dB of antenna isolation is achievable. In the following table we take a more optimistic assumption than our own of 80dB isolation.

Table 2.2.3-2 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 80 dB spatial isolation and typical existing BS LNA components.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed antenna isolation | -80 | -80 | -80 |
| F | TX power in DL subbands = B+D | -27 | -42 | -56 |
| G | Assumed IIP3dB | -22.6 | -17.6 | -14.6 |
| H | Gain normalized IM3 power in UL subband =3*F-2*G | -35.9 | -90.9 | -138.9 |

From table 2.2.3-2, we make the following conclusions:

- For LA BS, the spectral regrowth of the DL sub-band is not significant, and no de-sensitization of the receiver would be observed. This would be true even if we had taken a more pessimistic assumption on the antenna suppression of e.g., 65-70dB.

- For MR BS, with existing BS performance the receiver would be substantially desensitized. To avoid desensitization around 5dB improvement of IIP3 would be needed (over and above the already assumed 10dB improvement from 3GPP minimum requirements). This would rise to a much larger level if a more realistic antenna isolation would be assumed.

  - Thus, for MR BS, receiver linearity is a highly challenging, although not necessarily unsolvable problem.

  - Increased receiver linearity or powerful interference cancellation would be needed, which would imply some additional complexity and energy consumption compared to a conventional gNB receiver.

- For WA BS, a catastrophic level of distortion would be observed with a receiver similar to a conventional BS receiver, even with optimistic assumptions on the antenna and IIP3. Even with this optimistic antenna assumption, IIP3 would need to achieve around 0dBm to avoid distortion.

  - Achieving such a high degree of linearity would have significant complexity and energy costs

  - Any kind of receiver interference cancellation would need to achieve at least 60-70dB suppression. Also, a very large dynamic range would be needed in the ADC.

The simplified conclusion is:

Observation 8: For FR1:

- Local area: Receiver linearity easily feasible

- Medium range: Receiver linearity pretty challenging, but not impossible (with cost of complexity, energy)

- Wide Area: Receiver linearity a show-stopper challenge

Proposal 4: RX non-linearity needs to be further studied

It might be postulated that a tunable filter could be placed in front of the LNA in order to suppress the DL signal, or that AGC in front of the LNA could be used to suppress the LNA gain. However, the total noise figure for the receiver is cascaded and the first components have a high impact on the noise figure.

Attenuation of the LNA gain due to AGC or insertion loss of an analogue filter would cause a substantial impact on the receiver noise figure, which would reduce UL coverage. Due to the small size of the UL sub—band and low guards, a filter would need to have an extremely high Q factor. Since the purpose of SBFD is increased coverage, reduction of first stage gain would not seem to be an appropriate design strategy.

Section 2.2.2 describes in more detail the impact of receiver architecture on filtering possibilities prior to the ADC.

Proposal 5: Proposals for filtering solutions to avoid ADC overload should clarify the assumptions on RX architecture

FR2 analysis

Table 2.2.3-3 shows a calculation of IIP3 based on the FR2 RX IM requirements. However, for FR2, RX IM is not the dimensioning requirement for IIP3; higher IIP3 is needed in order to achieve, for example, reasonable EVM. In further analysis, we assume an IIP3 performance of -40dBm, which is 25-30dB higher than the level based on examining the minimum requirements.

Two assumptions for FR2 power levels are shown; a 40dBm assumption, which may be viewed as an upper limit of likely FR2 output power and a 30dBm limit. It is assumed that the 40dBm BS has a sensitivity of -105dBm and the 30dBm BS -100dBm.

Table 2.2.3-3 Minimal LNA IIP3dB parameter estimation based on RAN4 in-band receiver intermodulation requirements.

|  |  | 40dBm BS | 30dBm BS |
| --- | --- | --- | --- |
| A | Assumed EIS | -105 | -100 |
| B | RAN4 RX intermodulation interferer power | -80 | -75 |
| C | RAN4 allowed desensitization | 6 | 6 |
| D | Implied DNR = 10*LOG10(10^(C/10)-1) | 4.7 | 4.7 |
| E | Implied gain-normalized distortion = A+D | -100.3 | -95.3 |
| F | Estimated minimum IIP3dB = (3*B-E)/2 | -69.8 | -64.8 |

According to analysis in section 2.4.2.1, it is reasonable to assume an antenna isolation of 80dB between TX and RX for a BS with reasonable form factor and 10cm separation between panels for FR2. Using this figure and an IIP3 of -40dBm, table 2.2.3-4 indicates a first estimate for IM3 distortion in the receiver. As can be seen, the IM3 distortion would potentially be very large.

Table 2.2.3-4 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 80 dB spatial isolation and typical existing BS LNA components.

|  |  | 40dBm BS | 30dBm BS |
| --- | --- | --- | --- |
| A | RAN4 REFSENS | -105 | -100 |
| B | TX power | 40 | 30 |
| D | Assumed antenna isolation | -80 | -80 |
| F | TX power in DL subbands = B+D | -40 | -50 |
| G | Assumed IIP3dB | -40 | -40 |
| H | Gain normalized IM3 power in UL subband =3*F-2*G | -40 | -70 |

In order to achieve an IM3 distortion 10dB lower than the sensitivity level, an IIP3 of around +2.5dB would be needed for the 40dBm BS and -20dB for the 30dBm case. Alternatively, with the -40dBm IIP3 performance assumption, TX-RX antenna isolation would need to be 105 dB for 40dBm or 95dB for 30dBm. A further alternative would be to use IC to remove RX interference. It should be noted that the TX power in the RX sub-bands is significantly above the FR2 blocking level, and so with digital IC, the analogue receiver would need to perform beyond the 3GPP minimum blocking requirement.

Based on the above analysis, we conclude that SBFD may be possible with an antenna design that requires a very large separation or receiver design that exceeds 3GPP minimum requirements for the MR case. For the WA case, achieving sufficient suppression of RX non-linearity is highly challenging.

Observation 9: For FR2:

- For 40 dBm BS, RX non-linearity presents an extreme challenge

- For 30 dBm BS, RX non-linearity presents a challenge that would necessitate a very large separation and/or improving the receiver.

### 2.2.4 Phase noise reciprocal mixing

FR1 analysis

Phase noise reciprocal mixing occurs when a high-power interferer is introduced to a receiver. The phase noise spectrum of the LO is imposed over the high-power signal, which can lead to interference into a lower power wanted signal. In the context of SBFD, this can lead to phase noise spectra imposed on the DL sub-bands causing desensitization of the UL sub-band.

Figure 2.2.4-1 Reciprocal phase noise mixing

For 30 kHz SCS, the interference power from the DL sub-bands to each of the 20 MHz UL sub-carriers is provided in Figure 2.2.4-2, based on the Rel-17 phase noise models discussed in RAN4.

Figure 2.2.4-2 Reciprocal mixing interference to different sub-carriers in the UL subband (at different offset from UL center frequency) from the DL subbands in a 40-20-40 MHz FR1 SBFD carrier.

Summing over all UL sub-carriers, the total interference powers from the DL subbands to the UL subband are given by the following table:

Table 2.2.4-1 Interference levels due to phase noise reciprocal mixing for FR1

| Phase noise model | Interference caused by reciprocal mixing of phase noise [dBc] |
| --- | --- |
| R4-2011494 (UE, low-cost BS) | -60.3 |
| R4-2010186 DM=5dB (UE, medium quality BS) | -64.7 |
| R4-2010186 DM=0dB (high quality BS) | -69.7 |

In table 2.2.4-2, we present estimates of the phase noise impact to the UL sub-band for each BS class based on the Rel-17 phase noise models discussed in RAN4 and assuming a high quality BS for WA, medium quality for MR and low cost BS for LA. Similar to the discussion on linearity, even though we view an antenna isolation of 70dB as realistic based on EM simulations, we take in this example an optimistic assumption of 80dB isolation.

Table 2.2.4-2 Example ICI distortion in the UL subband caused by SBFD DL subbands assuming 80 dB spatial isolation and example phase noise models. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed antenna isolation | -80 | -80 | -80 |
| F | TX power in DL subbands = B+D | -27 | -42 | -56 |
| I | Interference from reciprocal mixing of phase noise | -69.7 | -64.7 | -60.3 |
| J | Gain normalized interference power in UL = F+I | -96.7 | -106.7 | -116.3 |

From table 2.2.4-2, we make the following observations:

- For the LA BS, phase noise reciprocal mixing does not appear to be an issue.

- For the medium range BS, around 0.1dB sensitivity degradation would occur due to phase noise mixing. With a more realistic assumption of 70dB TX-RX isolation, the degradation would increase to around 1dB.

- For a Wide Area BS, a desensitization of >2.5dB would occur even with this optimistic assumption on antenna isolation, increasing to 9.5dB with 70dB antenna isolation.

  - To mitigate phase noise distortion, there are two options; a more expensive and powerful oscillator than used in usual BS or phase noise interference cancellation. Both of these options would have complexity and power implications.

The simplified summary is:

Observation 10: For FR1,

- Local area BS, reciprocal phase noise mixing is not a significant issue

- Medium range BS, reciprocal phase noise mixing may need attention with realistic TX-RX isolation, but does not seem a major issue

- Wide area BS phase noise reciprocal mixing is a challenge that may need a more complex oscillator or IC than in a conventional BS.

FR2 analysis

Assuming 120 kHz SCS and 3 RB of guard band in a 75-50-75 MHz FR2 SBFD carrier, the interference power from the DL subbands to each of the 50 MHz UL sub-carriers is provided in figure 2.2.4-3.

Figure 2.2.4-3: Reciprocal mixing interference to different sub-carriers in the UL subband (at different offset from UL center frequency) from the DL subbands in a 75-50-75 MHz FR2 SBFD carrier.

Summing over all UL sub-carriers, the total interference powers from the DL subbands to the UL subband are given by the following table:

Table 2.2.4-3 Interference due to phase noise reciprocal mixing (FR2)

| Phase noise model | Interference caused by reciprocal mixing of phase noise [dBc] |
| --- | --- |
| R4-2011494 (UE, low-cost BS) | -44.0 |
| R4-2010186 DM=5dB (UE, medium quality BS) | -49.5 |
| R4-2010186 DM=0dB (high quality BS) | -54.5 |

Based on an assumption of 80dB antenna isolation, the ICI power arising from reciprocal phase noise mixing is shown in the table below.

Table 2.2.4-4 Example ICI distortion in the UL subband caused by SBFD DL subbands assuming 80 dB spatial isolation and example phase noise models. (All powers are gain normalized.)

|  |  | 40dBm BS | 30dBm BS | 30dBm BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -105 | -100 | -100 |
| B | TX power | 40 | 30 | 30 |
| D | Assumed antenna isolation | -80 | -80 | -80 |
| F | TX power in DL subbands = B+D | -40 | -50 | -50 |
| I | Phase noise ICI power factor | -54.5 | -54.5 | -49.5 |
| J | Gain normalized ICI power in the UL subband = F+I | -94.5 | -104.5 | -99.5 |

As the table indicates, reciprocal phase noise mixing will cause extensive desensitization in an FR2 receiver for 40dBm and around 1.4 dB desensitization for 30dB assuming high quality BS or 3dB for medium quality under these assumptions. For MR, the phase noise could be mitigated by more expensive components or cancellation.

Observation 11: Phase noise reciprocal mixing is likely to be a very significant challenge for FR2 for 40dBm. For 30dBm, some improvement in the transmitter design may be needed to avoid degradation.

Proposal 6: Phase noise reciprocal mixing should be examined further, in particular for 40dBm power

### 2.2.5 ADC level

FR1 analysis

The dynamic range of the ADC must be sufficient that it can accommodate both the DL sub-band and UL sub-band power levels. An insight into the implications to the ADC dynamic range can be obtained by considering the blocking requirements met by 3GPP BS in relation to the DL power level experienced in the receiver. Table 4 depicts the blocking levels from the 3GPP specifications:

Table 2.2.5-1 RAN4 blocker and adjacent channel selectivity test interference power levels.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
|  | RAN4 in-band narrowband blocker power | -49 | -44 | -41 |
|  | RAN4 adjacent channel interfering signal power | -52 | -47 | -44 |
|  | RAN4 in-band general blocker power | -43 | -38 | -35 |

Table 2.2.5-2 calculates the DL signal levels experienced in the receiver due to the DL sub-bands. As in other sections, although our observations on antenna isolation suggest that around 70dB is feasible, we take a more optimistic assumption of 80dB.

Table 2.2.5-2 Example interfering powers in SBFD DL subbands assuming 80 dB spatial isolation. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| D | Assumed antenna isolation | -80 | -80 | -80 |
| F | TX power in DL subbands = B+D | -27 | -42 | -56 |

From table 5 we make the following observations:

Observation 12: For FR1,

- For LA BS, the DL power is below the blocking requirement and ADC dynamic range is not likely to be an issue

- For MR BS, the DL power is a few dB below the general blocking level. It needs to be borne in mind that 6dB receiver sensitivity degradation is allowed with the -38dBm general blocker level and also that the TX-RX isolation assumption is optimistic, so some steps might possibly be needed to avoid ADC overload for MR.

- For WA BS, the receiver power level is substantially greater than the RX blocking levels, by 16dB.

The ADC dynamic range may be addressed by:

- More bit resolution in the ADC; however this would substantially increase complexity and power consumption in the ADC

- Filtering just before the ADC; see section 2.2 for a discussion on the relation between filtering possibilities and architecture

- AGC just before the ADC

Since the ADC is the last stage of analogue processing and is not a gain stage, AGC or filtering would have a substantially lower impact on the receiver noise figure than if they would be placed before the LNA. Further investigation is needed, but AGC or filtering may be viable means to address the ADC dynamic range depending on the receiver architecture.

FR2 analysis

FR2 blocking requirements are relative to the sensitivity. Some assumptions for blocking levels are shown in the table below. It should be noted that the blocking requirement allows for 6dB sensitivity degradation, so to avoid RX desensitization the TX signal level needs to be somewhat lower than the blocking level if the receiver performs according to 3GPP requirements.

Table 2.2.5-3 RAN4 blocker and adjacent channel selectivity test interference power levels.

|  |  | 40dBm BS | 30dBm BS |
| --- | --- | --- | --- |
| A | RAN4 REFSENS | -105 | -100 |
|  | RAN4 adjacent channel interfering signal power | -77.3 | -72.3 |
|  | RAN4 in-band general blocker power | -72 | -67 |

Based on an assumption of 80dB antenna isolation, the TX power in the RX sub-bands is depicted in the table below.

Table 2.2.5-4 Example interfering powers in SBFD DL subbands assuming 80 dB spatial isolation. (All powers are gain normalized.)

|  |  | 40dBm BS | 30dBm BS |
| --- | --- | --- | --- |
| A | RAN4 REFSENS | -105 | -100 |
| B | TX power | 40 | 30 |
| D | Assumed antenna isolation | -80 | -80 |
| F | TX power in DL sub-bands = B+D | -40 | -50 |

For both BS power levels, the power into the receiver will be greater than the RX blocking requirement under these assumptions. However, it is likely that, in particular for the 30dBm BS case, receivers can be built that perform better than the 3GPP minimum requirement.

Observation 13: For FR2 the SBFD self-interference will be greater than the 3GPP in-band blocking requirement. For FR2 BS, in particular power levels 30dBm or below, it may be possible to improve the blocking performance.

## 2.3 Inter-gNB/Inter-sector interference modelling

In addition to self-interference from the DL sub-bands within the own gNB, in a multi-sector deployment the UL receiver will experience interference from DL sub-bands in other sectors. A typical RAN4 assumption for isolation within a site between gNB is 30dB. Electromagnetic simulations indicate that the isolation between sectors is more realistically 45dB. Furthermore, if vertical separation between TX and RX is considered then the isolation may increase to 60dB. Note that the isolation between sectors will depend on the beamforming. Further increases in inter-sector isolation may be obtained using site solutions such as increased separation between sectors or possibly isolating materials, although the practicability of such solutions on real sites and on far field propagation patterns needs further consideration.

Figure 2.3-1 Example 3 sector deployment

Inter-sector leakage will, similar to gNB TX-RX leakage impact the received signal in two ways. TX distortions will increase the noise level in the receiver sub-band. The RX signal will fall into the receiver and may cause analogue distortions. Detailed antenna modelling relating to inter-sector leakage/inter-gNB isolation is elaborated in detail in section 2.4.

FR1 analysis

Considering the TX part, table 2.3-1 and 2.3-2 calculate the interference power due to TX imperfections in a neighbor sector falling into the receive sub-band for 45dB and 60dB isolation assumptions respectively. It should be borne in mind that this is for a single sector; in a 3 sector site there will be 2 interfering sectors and so the interference level will be 3dB higher.

The LA BS class is added for context, although sectorized deployments are not likely for local area.

Table 2.3-1 Inter-sector direct TX leakage to UL subband assuming 45 dB sector isolation.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| C | DL subband to UL subband leakage ratio | -45 | -45 | -45 |
| K | Assumed inter-sector antenna isolation | -45 | -45 | -45 |
| L | Inter-sector TX power in UL subband = B+C+K+3 | -34 | -49 | -63 |

Table 2.3-2 Inter-sector direct TX leakage to UL subband assuming 60 dB sector isolation.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | TX power | 53 | 38 | 24 |
| C | DL subband to UL subband leakage ratio | -45 | -45 | -45 |
| K | Assumed inter-sector antenna isolation | -60 | -60 | -60 |
| L | Inter-sector TX power in UL subband = B+C+K+3 | -49 | -64 | -78 |

Table 2.3-2 indicates that even for 60dB isolation, the TX power falling into the UL sub-band will be substantial.

The DL sub-band power from other sectors falling into the receiver will cause the same receiver impacts as intra-gNB power; i.e. LNA non-linearity, reciprocal phase noise mixing, ADC dynamic range etc. Table 2.3-3 indicates the TX power falling into the DL sub-bands and table 2.3-4 the receiver distortion arising due to LNA non-linearity. The “” symbol indicates that the input power to the receiver would be so high that the LNA would by physically damaged.

Table 2.3-3 Example interfering powers in SBFD DL subbands assuming 45 dB sector isolation. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| B | TX power | 53 | 38 | 24 |
| M | Assumed antenna isolation | -45 | -45 | -45 |
| N | TX power in DL subbands = B+M+3 | 11 | -4 | -18 |

Table 2.3-4 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 45 dB sector isolation and typical existing BS LNA components.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| N | TX power in DL subbands = B+M+3 | 11 | -4 | -18 |
| G | Assumed IIP3dB | -22.6 | -17.6 | -14.6 |
| O | Gain normalized IM3 in UL subband = 3*N-2*G |  |  | -24.9 |

Tables 2.3-5 and 2.3-6 indicate the same calculations assuming 60dB isolation

Table 2.3-5 Example interfering powers in SBFD DL subbands assuming 60 dB sector isolation. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| B | TX power | 53 | 38 | 24 |
| M | Assumed antenna isolation | -60 | -60 | -60 |
| N | TX power in DL subbands = B+M+3 | -4 | -19 | -33 |

Table 2.3-6 Example IM3 distortion in the UL subband caused by SBFD DL subbands assuming 60 dB sector isolation and typical existing BS LNA components.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| N | TX power in DL subbands = B+M+3 | -4 | -19 | -33 |
| G | Assumed IIP3dB | -22.6 | -17.6 | -14.6 |
| O | Gain normalized IM3 in UL subband = 3*N-2*G |  | -21.9 | -69.9 |

From the tables, we draw the following conclusions:

Observation 14: For FR1,

- For an LA BS, inter-sector isolation would cause major non-linearity issues and also phase noise reciprocal mixing and ADC overload. However, a sectorized deployment is anyhow not likely to LA.

- For a MR BS, with 60dB isolation the distortions from both the transmitter and receiver would be very substantial. Interference cancellation would need to achieve more than 70dB suppression, which is not viable.

- For WA BS, with 60dB isolation TX related inter-sector interference would be very high and the impact of the DL sub-band on the receiver would be to damage the LNA.

## 2.4 High isolation antenna analysis

In RAN4 #104e, the discussion around advanced antennas providing high isolation between transmitter and receiver was initiated. Feasibility and performance of a suitable antenna structure that can achieve significant isolation between transmitted DL and received UL signals in SBFD slots is one of the most important key considerations for assessment of SBFD. It was shown that reasonable physical separation needs to be combined with more advanced structures that in combination can increase the isolation where the achievable isolation for some structures and structure combinations was discussed, and results were presented.

From the point of view of assessing total achievable isolation, a definition based on TX panel to RX sub-array is a reasonable choice, but from a link gain assessment perspective taking into account the cancellation algorithms, sub-array to sub-array definition could be relevant since interference towards RX is created by coupling to a large set of individual TX sub-arrays with different coupling and gain/phase signal characteristics, and in order to properly assess the complexity and gain of interference cancellation algorithms, it is necessary to capture the signal structure accurately (see Figure 2.4-1).

Figure 2.4-1 Example of sub-array-to-sub-array coupling (left) and panel to sub-array (right)

A reference SBFD antenna setup is introduced in Figure 2.4-2 to provide a baseline for simulations of isolation enhancement techniques. The selected sub-array structure and size relate to other requirements for compatibility with other systems.

Figure 2.4-2 Illustration of reference antenna design. Each antenna element is excited for a dual, linear polarization (e.g., -45/+45 deg. or V/H) and the vertical inter-element distance is 0.7𝜆.

In addition, apart from modelling of the intra-gNB antenna interactions, it is also of importance to consider achievable isolation at site level as discussed in previous section. For a sectorized site, back-lobes and side/grating lobes from other sectors can cause interference towards a SBFD receiver. In general, RX/TX isolation in both intra- and inter-gNB scenarios depends on complex electromagnetic simulations. The separation between the RX and TX antennas is typically within only a few wavelengths, hence the self-interference leakage is mostly a near-field phenomenon.

To accurately model near-field interactions, it is necessary to run full-wave simulations that take into account all the coupling mechanisms and structures in detail. Isolation-enhancing structures usually comprise electrically small elements that require a full-wave solution with a fine mesh to provide an accurate isolation result. The electromagnetic interactions between an SBFD antenna and other structures on the same site happen mostly in its near-field region. Self-interference leakage can also reach the RX antennas through propagation mechanisms in the far-field.

The presented results for intra-gNB showed that, SBFD FR1 antenna performance highly depends on polarization and desired bandwidth and complexity of combined structures. In addition, the achievable isolation can degrade ~10 dB depending on the beam forming over steering range of the antennas.

Similarly, it could be observed that for inter-gNB isolation, SBFD FR1 antenna isolation performance highly depends site installation and achievable isolation can vary significantly depending on the beam forming over steering range of the antennas.

The updated analysis results for FR1 from previous meeting is summarized in Section 2.4.1 and FR2 high isolation antenna aspects are described in detail in Section 2.4.2.

### 2.4.1 FR1 antenna analysis

In this section, the detailed analysis of high isolation antenna technologies around 4 GHz considering intra-gNB isolation is summarized. The achievable isolation by deploying different isolation enhancement structure as well as combination of several structures. The analysis considers sub-array to sub-array coupling for individual isolation enhancing structures while panel to sub-array is considered for combined structures. In addition, the analysis around inter-gNB isolation is also discussed in this paper. For intra-gNB and inter-gNB, the impact of beamforming is investigated which is also summarized in subsequent section.

The achievable isolation for different polarization was studied while impact of radome was not considered in this paper and will be further elaborated in coming meetings.

#### 2.4.1.1 Intra gNB

As a baseline, the coupling between TX and RX physical separation of the sub-array to sub-array is depicted in Figure 2.4.1.1.-1 indication that physical isolation alone is not sufficient to obtain enough isolation between TX and RX.

Figure 2.4.1.1-1 Sub-array-to-sub-array coupling magnitude between a pair of dual-polarized antenna sub-arrays.

Observation 15: Physical separation between TX and RX panels does not provide sufficient isolation. Structures and materials to improve isolation are needed.

Using periodic structures, e.g., electromagnetic band gap square mushrooms with sub-array to sub-array separation of 4l provides isolation of ~60-65 dB but is limited in bandwidth as shown in Figure 2.4.1.1-2.

Figure 2.4.1.1-2 RX/TX coupling magnitude for two sub-arrays as function of frequency obtained from the simulation with a mushroom EBG structure.

Using metallic choke walls as another possible solution to reduce the energy leakage between the TX and RX sub-arrays will provide isolation enhancements but still limited in bandwidth and achievable levels as depicted in Figure 2.4.1.1-3.

Figure 2.4.1.1-3 Full-wave averaged E-field magnitude on an XZ plane cut (left) and RX/TX coupling magnitude for two sub-arrays as function of frequency (right)

An example of combined structures was investigated where achievable sub-array-sub-array isolation was better than 65 dB over large enough bandwidth for FR1.

Figure 2.4.1.1-4 RX/TX coupling magnitude for two sub-arrays as function of frequency corresponding to the using combined isolation improvement techniques

Observation 16: For FR1, with a combination of techniques a sub-array-sub-array isolation of 65dB over reasonable bandwidth is achievable (without radome consideration).

The achievable isolation using combined structures considering TX full panel towards RX sub-arrays (the RX sub-arrays closest to isolation enhancing structures) including impact of beam steering (+/-15 degree in elevation) was investigated and the results are summarized in Figure 2.4.1.1-5.

| - 0 degree (boresight) | - 15 degrees (TX steers main beam toward RX) | - -15 degrees (TX steers main beam away from RX) |
| --- | --- | --- |
| (d) 0 degree (boresight) | (e) 15 degrees (TX steers main beam toward RX) | (f) -15 degrees (TX steers main beam away from RX) |

Figure 2.4.1.1-5 TX panel to RX sub-array coupling magnitude curves considering co-polarized (top) and cross polarized (bottom) ports. Each curve represents the coupling magnitude of the TX panel to a single RX sub-array. Each sub-figure corresponds to a specific elevation angle.

As shown in Figure 2.4.1.1-5, SBFD antenna performance highly depends on polarization, desired bandwidth and complexity of combined structures. In addition, the achievable isolation can degrade ~10 dB depending on the beam forming over steering range of the antennas.

Observation 17: For FR1 (without considering radome), for panel to sub-array isolation around 70 dB over sufficiently large bandwith is achievable as long as the beam is steered in boresight. When the beam is steered elsewhere, the isolation may reduce by ~ 10dB.

For system level analysis, the isolation will depend on the beam steering, but as an approximate assumption it may be the case that an averaging effect when scheduling multiple UEs in different directions leads to an average value. Since the best case is 70dB and there can be 10dB reduction depending on the direction, we assume 65dB average,

Proposal 7: Assume 65dB average panel-sub-array isolation for FR1 (taking into account variation due to beamforming) for system level analysis. For link level analysis, the isolation due to beam-forming need to be modelled properly.

Considering the analysis in this section, it is evident that proper modelling of antenna isolation and beamforming as near field effect is essential for feasibility study of SBFD

#### 2.4.1.1 Inter gNB

Back-lobes as well as side/grating lobes for AAS antennas create a source of interference within a site, which can increase the mutual coupling between antenna systems. This is particularly important when AAS antennas are in each other’s near-field region. Back-lobes, similar to side/grating lobes are dependent on the beam-forming range and applied weights to the array and can vary depending on the beam which can affect the coupling and isolation in the site.

To visualize the impact of beam forming over the scan range of the antennas, a simple structure as in Figure 2.4.1.1-1 was used to assess the inter-gNB/site isolation.

Figure 2.4.1.1-1 Illustration of the considered site and antenna models (top) and coordinate system (bottom).

In addition, two types of beam steering are performed considering steering beam to θ with fixed ϕ=0 degree, and beam steer to ϕ with fixed θ=0 degree, considering the coordinate system shown in Figure 12.

| - (a) Beam steering with varying ϕ and θ=0 degree. | - (b) Beam steering with varying θ and ϕ=0 degree. |
| --- | --- |

| - (c) Beam steering with varying ϕ and θ=0 degree. | - (d) Beam steering with varying θ and ϕ=0 degree. |
| --- | --- |

Figure 2.4.1.1-2 Inter-gNB leakage between antenna modules with inter-pole distance of d=122.32 mm with edge-to-edge distance of 100 mm (top) and Inter-gNB leakage between antenna modules at same height and inter-pole distance of d=400 mm (bottom).

To visualize the mechanisms behind coupling in near field for inter-gNB, various E-field cuts are presented in Figure 2.4.1.1-3.

Figure 2.4.1.1-3 Full-wave averaged E-field magnitude for different cuts, varying azimuth angle ϕ and θ=0 deg.

In addition, the inter-gNB isolation analysis was performed when TX and RX are shifted in elevation since TX and RX sections of the SBFD antenna are separated vertically in order to supress self-interference (intra-gNB). The results for vertically shifted TX and RX are presented in Figure 2.4.1.1-4.

Figure 2.4.1.1-4 Site model (left) Inter-gNB leakage between Sectors 1 and 3 for varying ϕ with fixed θ=0 degree for the site setup (middle) and Inter-gNB leakage between sectors 1 and 3 for varying θ with fixed ϕ=0 degree (right).

For shifted RX/TX site models, various E-field cuts are presented in Figure 2.4.1.1-5 to help visualizing the mechanisms behind coupling in the near-field region for inter-gNB coupling.

- Boresight (ϕ=0 deg and θ=0 deg)

- Azimuth steering (ϕ=30 deg and θ=0 deg)

- Azimuth steering (ϕ=60 deg and θ=0 deg)

Figure 2.4.1.1-5 Full-wave averaged E-field magnitude for different cuts, varying azimuth angle ϕ and θ=0 deg. The shifted RX/TX model is considered here.

As shown in the inter-gNB analysis, SBFD inter-gNB antenna isolation performance (~30-60 dB) highly depends site installation and achievable isolation can vary significantly depending on the beam forming over steering range of the antennas.

Thus, proper modelling of inter-gNB for sectorized site is an essential part when assessing the feasibility and performance for SBFD.

Observation 18: The sector-sector isolation is cell and beam forming dependent. For FR1, assuming vertical separation of the cells, isolations of up to 60dB may be achievable. This can be considered for system level analysis.

Proposal 8: For system level analysis, assume a best-case cell isolation of 60dB for FR1, but analyse further as depending on beamforming and site, the best case may not be achievable.

Additional analysis considering larger RX and TX arrays for FR1 will be presented in the coming meetings.

### 2.4.2 FR2 antenna analysis

#### 2.4.2.1 Intra gNB

Full-electromagnetic simulations were performed to analyse the power leakage from the TX panel to RX branches considering a carrier frequency of 30 GHz. These simulations aim to compare the isolation performance of an antenna design with a specific isolator structure compared to a baseline reference model that uses no isolator structure.

The performed simulations assume a gNB antenna with separate panels for transmission and reception. Each panel consists of an antenna array comprising 8 rows and 24 columns of 2x1 dual-polarized sub-arrays. The vertical separation between elements is 0.7λ and the horizontal separation is 0.5 λ. The radiators of the TX panels are fed with an individual TX branch, such that each radiator has an individual phase shift. Therefore, the TX panel does not face any significant grating lobes within the ±15 degrees range in elevation. This is a rather beneficial scenario, as the presence of grating lobes in the far-field pattern may degrade the isolation between the TX and RX panels. The separation between the TX and the RX panels is set to 10 cm, i.e., approximately 10λ at 30 GHz. The simulation models assume that the antenna panels share a common ground plane, and that no radome is present. The presence of a radome is expected to introduce losses and possibly create an additional coupling medium between the TX and RX.

The main figure of merit is the coupling magnitude between the TX panel and the RX branches. More specifically, this metric is defined as the difference (in decibels) between the total radiated power and the power at some RX branch connected to an antenna sub-array of 2x1 elements. The coupling magnitude is evaluated for three TX beam steering settings: boresight, +15 degrees in elevation (pointing toward RX), -15 degrees (pointing away from RX). For simplicity, the coupling magnitude results are presented for the 48 RX branches (24 branches for each polarization) located in the closest row to the TX panel, i.e., at one of the RX panel’s edges.

The coupling magnitude between the TX panels and RX branches for both co- and cross-polarized ports is presented in Figure 2.4.2.1-1 and the TX radiation patterns are shown in Figure 2.4.2.1-2. The coupling magnitude results indicates that the reference model provides an isolation level of 70 dB over approximately 4 GHz. It is remarkable that the isolation at boresight is much larger than the previously considered FR1 reference model. This is because the high dimensionality of the FR2 array, compared to that of the FR1 antenna, which yields a very sharp far-field beam, thereby reducing the power leakage toward the RX. While in the former there are 384 (8x24x2) antennas in the TX aperture, the latter is formed by only 96 (4x8x3) antennas.

| - Co-polarized ports | - Cross-polarized ports |
| --- | --- |

Figure 2.4.2.1-1 Coupling magnitude of the reference model at boresight

| - Average E-field | - Far-field realized gain (cut at ϕ=0∘) |
| --- | --- |

Figure 2.4.2.1-2 Near- and far-field patterns of the FR2 reference model (cut at ϕ=0 deg)

The antenna isolation enhancing techniques discussed in Section 2.4.1 can be also implemented in FR2. However, some techniques can present themselves more challenging in this frequency range mainly due to implementation and fabrication issues. Isolation structures based on multi-layer choke structures are relatively simple to implement and tend to provide good isolation performance. Therefore, a high-isolation antenna model based on the reference model has been built using choke structures as isolation enhancing structure and evaluated through simulations.

The isolation performance of the choke-based antenna is shown in Figure 2.4.2.1-3 for the boresight case and both polarization ports. The co-polarized port provides an isolation of 80 dB with a bandwidth support of roughly 3 GHz, while the cross-polarized port offers the same isolation level but over a slightly larger bandwidth. In Figure 2.4.2.1-4, the isolation of the reference and choke-based antenna models are compared. This figure indicates that chokes provide enough isolation improvement to keep the isolation below 80 dB over some bandwidth for this FR2 antenna. As shown in Section 2.4.1, the combination of chokes with other structures may provide a better isolation performance over larger bandwidth support. The near- and far-field patterns of the choke-based antenna are shown in Figure 2.4.2.1-5. By comparing the near-field pattern in Figure 2.4.2.1-5 (a) to that of the reference antenna model in Figure 2.4.2.1-2(a), the chokes create a region with low E-field intensity above the RX sub-arrays. Furthermore, the comparison of the far-field realized gain patterns in Figures 2.4.2.1-2(b) and 2.4.2.1-5(b) reveal that the isolation improvement does not have a negative effect on the realized antenna gain of the TX panel. It is important to remark that the large number of radiators in the TX panel, the 10 cm separation, and the isolation structure are the main elements in achieving 80 dB in isolation in the considered setup. The FR1 antenna models did not have as many antenna radiators in the TX panel, nor the relative panel-to-panel distance of 4 λ.

| - Co-polarized ports | - Cross-polarized ports |
| --- | --- |

Figure 2.4.2.1-3 Coupling magnitude performance of the choke-based antenna model at boresight. Each curve represents the coupling between the TX panel and a single RX branch.

| - Co-polarized ports | - Cross-polarized ports |
| --- | --- |

Figure 2.4.2.1-4 Comparison of the coupling magnitude between the choke-based and reference antenna models. Each curve represents the coupling between the TX panel and a single RX branch.

| - Average E field pattern | - Far-field realized gain |
| --- | --- |

Figure 2.4.2.1-5 Near- and far-field patterns of the choke model with main beam steered toward boresight.

An SBFD antenna should satisfy the isolation requirements also within a beam tilting range. Simulations were performed to determine how robust the isolation of the choke-based antenna model is to beam steering. In particular, the coordinate system shown in Figure 2.4.2.1-6 was considered for three different values of steering angle θ: 0 degree (boresight), +15 degrees (toward RX), and -15 degrees (away from RX). Figure 2.4.2.1-7 shows the isolation performance of the choke-based model for co-polarized ports and varying steering angle, and Figure 2.4.2.1-8 presents the same metric for cross-polarized ports. These simulation results suggest that the considered antenna model supports 80 dB within the ±15 degrees tilting range over a bandwidth of approximately 4 GHz. By contrast, the FR1 antenna model suffers a significant loss in isolation performance when steering in the same range, as seen in Figure 2.4.1.1-5. This is because the FR1 antenna array has fewer radiator elements, therefore it produces broader radiation beams as compared to the FR2 antenna model considered in this section. The comparison between Figures 2.4.2.1-7 and 2.4.2.1-8 indicating that the cross-polarized provides a slightly larger isolation than the co-polarized ports. The near-field patterns shown in Figures 2.4.2.1-84 and 2.4.2.1-9 confirm that the considered choke-based antenna is robust to beam tilting, as the RX branches remain in a low E-field intensity region.

Figure 2.4.2.1-6 Illustration of TX elevation beam steering angle θ. The right antenna panel refers to TX while the left one to RX. In this illustration, the azimuth angle ϕ is 0 degrees.

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

Figure 2.4.2.1-7 Coupling magnitude of the choke-based antennas for varying steering angle θ and co-polarized RX ports.

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

Figure 2.4.2.1-8 Coupling magnitude of the choke-based antennas for varying steering angle θ and cross-polarized RX ports.

| - +15 degrees toward the RX | - -15 degrees away from the RX |
| --- | --- |

Figure 2.4.2.1-9 Near-field pattern of the choke model with main beam steered to +15 degrees (a) and -15 degrees (b).

Observation 19: For FR2 (without considering radome), using a structure with RF chokes, 80dB of isolation is achievable over a reasonable bandwidth. Unlike FR1, the isolation does not seem to vary with beam steering.

Proposal 9: Assume 80 dB antenna isolation for FR2 for system level analysis.

#### 2.4.2.2 Inter gNB

As indicated in Section 2.4.1.1, back-lobes as well as side/grating lobes for AAS antennas create a source of interference within a site. Consequently, the TX array in one sector is strongly coupled to the RX in another sector, especially because the latter is within the near-field region of the former. The TX/RX coupling is strongly dependent on the weights applied to the array. To visualize the impact of beamforming on inter-sector leakage, the 3-sector site model depicted in Figure 2.4.2.2-1 is considered. Each sector of the considered site comprises an array of 8x8 dual-polarized FR2 antennas. The horizontal and vertical inter-element distances are 0.5 and 0.6 wavelengths, respectively. The site model is constructed such that the edge-to-edge distance between the antenna panels is 10 cm, as seen in Figure 2.4.2.2-1. Two types of beam steering are performed considering steering beam to θ with fixed ϕ=0 degree, and beam steer to ϕ with fixed θ=0 degree, considering the coordinate system shown in Figure2.4.1.1-1.

Figure 2.4.2.1-1 Illustration of the considered 3-sector site.

| - Beam steering with varying ϕ and θ=0 degree. | - Beam steering with varying θ and ϕ=0 degree. |
| --- | --- |

Figure 2.4.2.2-2 Inter-gNB leakage between Sectors 1 and 2 for different beam steering angles and edge-to-edge distance of 10 cm.

A full electromagnetic simulation was performed to calculate the coupling between Sectors 1 (TX) and 2 (RX). The coupling magnitude is shown in Figure 2.4.2.2-2. The best isolation (60 dB) is observed when the TX steers the main beam 30 degrees downwards, while the worst isolation (20 dB) is seen when it steers the main beam 60 degrees toward the RX. To help the visualization of the near-field coupling between the TX and RX, various E-field cuts are shown in Figure 2.4.2.2-3. These cuts reveal that the, at boresight, the backlobe of the TX array is pointed to the RX arrays. However, it is steered away from the RX arrays when the TX’s main lobe is downtilted. These simulation results show that the inter-gNB leakage is a complex phenomenon that relies on complex electromagnetic simulations.

- Boresight (ϕ=0 degree and θ=0 degree).

- ϕ=0 degree and θ=-30 degree.

- ϕ=30 degrees and θ=0 degree.

- ϕ=60 degrees and θ=0 degree.

Figure 2.4.2.2-3 Full-wave average E field for various cuts.

Observation 20: For FR2 (without considering radome), inter gNB isolation can vary between 20 dB to 60 dB with beamforming.

Additional analysis considering RX and TX shifted panels for FR2 will be presented in the coming meetings.

## 2.5 Far-field antenna modelling

Originally an array antenna model based on pattern multiplication between element factor and array factor was developed for AAS base station in TR 37.840. The element factor considered was Gaussian shaped element factor, even though other element structures are common for base station implementations. This model has been used for many simulation campaigns in RAN1 and RAN4. However, recently this model was questioned in ITU-R WP 5D since the model doesn’t reflect real base station implementations deployed in networks. Evaluating base station in deployed networks gives that sub-array antenna structures is currently used for both FR1 and FR2. The background for having sub-array structures for base station antennas is to optimize the design for maximum performance and coverage. As an addition to the original model an extended array antenna model supporting vertical sub-array structures have been included in TR 38.803. For the frequency range 1710 to 4990 MHz relevant base station antenna parameters are also provided in TR 38.803. An LS [5] has been sent to ITU-R WP 5D and ECC with additional information relevant for sub-array structures used in the frequency band 1710 to 4990 MHz.

The extended antenna model is described by the equations given in Table 2.5-1.

Table 2.5-1: Extended model supporting vertical sub-arrays

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

Observe that if Msub is set equal to 1 and dv,sub equal to dv and qsubtilt equal to 0 degrees, the extended array antenna model collapses into the original antenna model.

High TX-to-RX isolation is crucial for minimizing the impact on uplink performance. Therefore, the antenna panels for transmitter and receiver are separated along the vertical axis with an isolator in between. To understand network performance and coexistence performance, the antenna needs to be modelled based on the assumption made to achieve high isolation between transmitter and receiver. It is essential to consider the same antenna structure used for far-field modelling as also for near-field isolation modelling and vice versa.

RAN4 have communicated relevant sub-array parameters relevant for the frequency range 1710 to 4990 MHz to ITU-R and CEPT. These parameters can be used for urban macro and dense urban macro for the proxy frequency of 4000 MHz assumed for SBFD co-existence simulations. The micro is a scaled down parameter set using sub-arrays. Other deployment scenarios rely on previous parameter assumptions based on single element structures.

In Table 2.5-2 parameter sets for urban macro, dense urban macro, dense urban micro and indoor in presented for SBFD simulations for FR1.

Table 2.5-2: Antenna configuration relevant for FR1 (4000 MHz)

| Parameter |  | Deployment scenario |  |  |
| --- | --- | --- | --- | --- |
|  | Urban macro | Dense urban macro | Dense urban micro | Indoor |
| Horizontal half power beam width | 90 o | 90 o | 90 o | 90 o |
| Vertical half power beam width | 65 o | 65 o | 65 o | 90 o |
| Front-to-back ratio | 30 dB | 30 dB | 30 dB | 30 dB |
| Side lobe suppression | 30 dB | 30 dB | 30 dB | 30 dB |
| Element gain | 6.4 dBi | 6.4 dBi | 6.4 dBi | 5.5 dBi |
| Number of element rows in sub-array | 3 | 3 | 3 | 1 |
| Element vertical separation within sub-array | 0.7l mm | 0.7l mm | 0.7l mm | 0.5l mm |
| Sub-array pre-set down-tilt | 3.0 o | 3.0 o | 3.0 o | 0.0 o |
| Number of columns | 8 | 8 | 8 | 4 |
| Number of rows | 4 | 4 | 2 | 4 |
| Horizontal separation | 0.5l mm | 0.5l mm | 0.5l mm | 0.5l mm |
| Vertical separation | 2.1l mm | 2.1l mm | 2.1l mm | 0.5l mm |
| Array loss | 2.0 dB | 2.0 dB | 2.0 dB | 2.0 dB |
| Conducted power fed to each branch | 35.0 dBm | 35.0 dBm | 32 dBm | 24 dBm |
| Number of supported polarizations | 2 | 2 | 2 | 2 |
| TX/RX antenna vertical separation | 4.0l mm | 4.0l mm | 4.0l mm | 2.0l mm |
| Mechanical down-tilt | 6.0 o | 6.0 o | 0.0 o | 90.0 o |

For FR2-1 RAN4 currently assumes single element structures in TR 38.803. Even though, sub-array antenna geometries are used for many of the deployed base stations is networks in several regions. At last, RAN4 meeting relevant sub-array parameters for FR2-1 AAS BS was presented in [2]. The same information has also been provided to ITU-R WP 5D in [3].

For SBFD the “constant gain” approach is used for urban macro, since the physical size of an antenna twice as large fits the base station enclosure due to shorter wavelength. For dense urban macro and dense urban micro, the panel is divided as “reduced gain” approach suggests. For in-door the antenna is further scaled down. The parameter sets relevant for FR2-1 is described in Table 2.5-3.

Table 2.5-3: Antenna configuration relevant for FR2-1 (30000 MHz)

| Parameter | Deployment scenario |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urban macro | Dense urban macro | Dense urban micro | Indoor |
| Horizontal half power beam width | 90 o | 90 o | 90 o | 90 o |
| Vertical half power beam width | 90 o | 90 o | 90 o | 90 o |
| Front-to-back ratio | 30 dB | 30 dB | 30 dB | 30 dB |
| Side lobe suppression | 30 dB | 30 dB | 30 dB | 30 dB |
| Element gain | 5.5 dBi | 5.5 dBi | 5.5 dBi | 5.5 dBi |
| Number of element rows in sub-array | 2 | 2 | 2 | 2 |
| Element vertical separation within sub-array | 0.6l mm | 0.6l mm | 0.6l mm | 0.6l mm |
| Sub-array pre-set down-tilt | 0.0 o | 0.0 o | 0.0 o | 0.0 o |
| Number of columns | 24 | 24 | 24 | 8 |
| Number of rows | 8 | 4 | 4 | 4 |
| Horizontal separation | 0.5l mm | 0.5l mm | 0.5l mm | 0.5l mm |
| Vertical separation | 1.2l mm | 1.2l mm | 1.2l mm | 1.2l mm |
| Array loss | 2.0 dB | 2.0 dB | 2.0 dB | 2.0 dB |
| Conducted power fed to each branch | 7.0 dBm | 7.0 dBm | 7.0 dBm | 7.0 dBm |
| Number of supported polarizations | 2 | 2 | 2 | 2 |
| TX/RX antenna vertical separation | 4.0l mm | 4.0l mm | 4.0l mm | 2.0l mm |
| Mechanical down-tilt | 15.0 o | 10.0 o | 0.0 o | 90.0 o |

More details related to antenna modelling can be found in a companion contribution [4] on SBFD co-existence simulation results submitted to this meeting.

# Conclusion

In this paper, detailed analysis around transmitter, receiver, interference cancellation considerations and antenna isolation aspects and modelling for SBFD was presented. The analysis covers both FR1 and FR2. In addition, different receiver architectures and their capability for filtering of DL sub-bands was discussed.

The detailed analysis in this paper can be summarized in the following observations and proposals:

Observation 1: To properly model the TX interference signal to be supressed or cancelled from a BS, at least the CFR process and PA+DPD combination need to be modelled realistically.

Proposal 1: Adopt the PA models in annex A for FR1 when considering TX interference suppression or cancellation.

Proposal 2: Adopt the transmitter model containing net PA+DPD and CFR+channel filtering for evaluation of TX interference suppression or cancellation

Observation 2: Interference cancellation complexity can scale exponentially with the number of transmitters and receivers.

Observation 3: For wide area BS, the complexity of basic interference cancellation is prohibitive.

Proposal 3: Proposals for interference cancellation should be accompanied by estimates of complexity.

Observation 4: The feasibility and complexity of filtering solutions depends on the receiver architecture.

Observation 5: The super-heterodyne receiver architecture enables frequency planning to avoid LO distortions. It is not easily compatible with low complexity analog filtering that does not require large guard between UL and DL sub-bands.

Observation 6: The homodyne type of receiver architecture can be more suited to pre-ADC filtering to prevent ADC overload; however, the complexity of such analog filtering needs careful consideration in a multi-carrier BS.

Observation 7: Filtering in the analogue domain is complex for advanced, direct sampling receivers as it needs to be done at RF.

Observation 8: For FR1:

- Local area: Receiver linearity easily feasible

- Medium range: Receiver linearity pretty challenging, but not impossible (with cost of complexity, energy)

- Wide Area: Receiver linearity a show-stopper challenge

Proposal 4: RX non-linearity needs to be further studied

Proposal 5: Proposals for filtering solutions to avoid ADC overload should clarify the assumptions on RX architecture

Observation 9: For FR2:

- For 40 dBm BS, RX non-linearity presents an extreme challenge

- For 30 dBm BS, RX non-linearity presents a challenge that would necessitate a very large separation and/or improving the receiver.

Observation 10: For FR1,

- Local area BS, reciprocal phase noise mixing is not a significant issue

- Medium range BS, reciprocal phase noise mixing may need attention with realistic TX-RX isolation, but does not seem a major issue

- Wide area BS phase noise reciprocal mixing is a challenge that may need a more complex oscillator or IC than in a conventional BS.

Observation 11: Phase noise reciprocal mixing is likely to be a very significant challenge for FR2 for 40dBm. For 30dBm, some improvement in the transmitter design may be needed to avoid degradation.

Proposal 6: Phase noise reciprocal mixing should be examined further, in particular for 40dBm power

Observation 12: For FR1,

- For LA BS, the DL power is below the blocking requirement and ADC dynamic range is not likely to be an issue

- For MR BS, the DL power is a few dB below the general blocking level. It needs to be borne in mind that 6dB receiver sensitivity degradation is allowed with the -38dBm general blocker level and also that the TX-RX isolation assumption is optimistic, so some steps might possibly be needed to avoid ADC overload for MR.

- For WA BS, the receiver power level is substantially greater than the RX blocking levels, by 16dB.

Observation 13: For FR2 the SBFD self-interference will be greater than the 3GPP in-band blocking requirement. For FR2 BS, in particular power levels 30dBm or  below, it may be possible to improve the blocking performance.

Observation  14: For FR1,

- For an LA BS, inter-sector isolation would cause major non-linearity issues and also phase noise reciprocal mixing and ADC overload. However, a sectorized deployment is anyhow not likely to LA.

- For a MR BS, with 60dB isolation the distortions from both the transmitter and receiver would be very substantial. Interference cancellation would need to achieve more than 70dB suppression, which is not viable.

- For WA BS, with 60dB isolation TX related inter-sector interference would be very high and the impact of the DL sub-band on the receiver would be to damage the LNA.

Observation 15: Physical separation between TX and RX panels does not provide sufficient isolation. Structures and materials to improve isolation are needed.

Observation 16: For FR1, with a combination of techniques a sub-array-sub-array isolation of 65dB over reasonable bandwidth is achievable (without radome consideration).

Observation 17: For FR1 (without considering radome), for panel to sub-array isolation around 70 dB over sufficiently large bandwith is achievable as long as the beam is steered in boresight. When the beam is steered elsewhere, the isolation may reduce by ~ 10dB.

Proposal 7: Assume 65dB average panel-sub-array isolation for FR1 (taking into account variation due to beamforming) for system level analysis. For link level analysis, the isolation due to beam-forming need to be modelled properly.

Observation 18: The sector-sector isolation is cell and beam forming dependent. For FR1, assuming vertical separation of the cells, isolations of up to 60dB may be achievable. This can be considered for system level analysis.

Proposal 8: For system level analysis, assume a best-case cell isolation of 60dB for FR1, but analyse further as depending on beamforming and site, the best case may not be achievable.

Observation 19: For FR2 (without considering radome), using a structure with RF chokes, 80dB of isolation is achievable over a reasonable bandwidth. Unlike FR1, the isolation does not seem to vary with beam steering.

Proposal 9: Assume 80 dB antenna isolation for FR2 for system level analysis.

Observation 20: For FR2 (without considering radome), inter gNB isolation can vary between 20 dB to 60 dB with beam forming.

# References

[1]	R4-2212620, “Analysis of issues related to self-interference and co-channel CLI”, Ericsson

[2]	R4-2212457, “On issues related antenna modelling and additional information relevant for FR2 base station”, Ericsson

[3]	Document 5D/1276-E, “BASE STATION PARAMETERS AND MITIGATION TECHNIQUES FOR MMWAVE FREQUENCY BANDS”, Telefon AB LM Ericsson

[4]	R4-2215835, “On initial results for SBFD adjacent channel co-existence evaluation”, Ericsson

[5]	R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Qualcomm

# Annex A

## A.1 GMP coefficients for ~2 GHz GaAs PA at 491.52 MHz sampling rate

aklPA with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

-0.018625 +   0.010388i     0.023333 -  0.0087798i      0.98622 -    0.18951i

0.054357 -   0.088831i    -0.090219 +    0.27003i       1.4755 +    0.65625i

-0.12455 -   0.017389i     -0.21213 -    0.15655i      -17.835 -    0.36305i

0.21638 +    0.66855i       2.8252 -     0.2232i        104.9 -     28.214i

-0.50435 -     2.3545i      -10.033 +     1.1268i      -358.38 +     172.31i

0.62463 +     4.0633i       18.287 -     2.2362i       700.58 -     426.41i

-0.41658 -     3.3421i      -16.322 +     1.9937i      -722.13 +     479.27i

0.12892 +     1.0357i         5.62 -    0.64864i       299.41 -     200.73i

Columns 4 through 5

-0.038271 +   0.057208i      0.05687 -   0.047196i

-0.06683 -    0.23028i    -0.087356 +      0.217i

2.6685 -     1.8173i    -0.055794 -     0.7882i

-18.377 +     11.387i       1.1306 +     3.5817i

52.78 -     30.504i      -2.6675 -     11.211i

-79.138 +     43.326i       2.8947 +     19.236i

60.339 -     31.287i      -1.5249 -     16.556i

-18.351 +     8.9437i      0.30771 +     5.5925i

bkl0PA with rows k∈[0,1,…,7] and columns l∈-1,1:

0.023333 -  0.0087798i    -0.038271 +   0.057208i

-0.60131 +    0.97361i     -0.19392 +    0.52764i

8.3245 -     10.179i      0.18517 -     3.2744i

-66.4 +     51.766i       2.1001 +     27.044i

283.41 -     146.66i       2.6785 -     124.41i

-622.14 +     241.98i       -40.94 +     281.93i

665.85 -     217.24i       85.013 -     304.59i

-273.99 +     80.807i      -52.474 +     125.19i

## A.2 GMP coefficients for DPD

aklDPD with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

0.022471 -  0.0078921i    -0.043712 -   0.016952i      0.98993 +     0.2369i

-0.11504 +   0.094754i      0.30463 -    0.27514i     -0.41542 -      2.164i

0.61892 -    0.20048i       -2.526 +    0.77428i       6.3179 +     15.297i

-2.1807 +    0.41401i       17.271 -     3.5562i      -20.414 -     64.192i

2.4173 -    0.26032i      -72.797 +     12.622i      -13.665 +     157.79i

8.1116 -     2.0655i       171.24 -     25.741i       171.94 -     249.03i

-23.955 +     7.1712i      -209.54 +     24.822i      -286.08 +     262.85i

17.8 -      6.778i       103.15 -     7.9965i       151.32 -     140.44i

Columns 4 through 5

0.089525 -    0.13533i    -0.058866 +   0.041949i

-0.18446 +    0.36701i      0.21464 -    0.21315i

-0.55708 -    0.73642i      -1.2971 +     1.2718i

8.508 +     6.3938i       7.4914 -     8.3733i

-19.38 -     41.588i      -32.321 +     32.588i

1.9616 +     124.32i       81.761 -     67.305i

42.859 -     170.52i       -106.2 +     68.492i

-38.085 +     87.918i       54.323 -     26.754i

## A.3 GMP coefficients for net effect of DPD and PA

aklnet with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

-0.0014656 -  0.0023163i    0.0012144 +  0.0096127i       1.0151 -   0.021398i

0.016921 +    0.02074i    -0.033944 -   0.060851i     -0.13734 +   0.092064i

-0.19421 -   0.090327i      0.70051 +    0.46802i     0.053178 +     1.0772i

1.2719 -    0.37999i      -7.2355 -    0.68417i       7.9188 -     22.487i

-3.6967 +     4.8521i       37.145 -     5.8753i      -66.051 +     153.74i

3.9652 -      16.55i      -99.665 +     25.306i        250.5 -     481.88i

1.2339 +     23.785i       134.36 -      36.61i      -450.95 +     708.65i

-4.4834 -     12.473i      -70.523 +     18.559i        304.9 -      395.4i

Columns 4 through 5

-0.015418 +   0.010133i    0.0069273 -  0.0004492i

0.15998 +  0.0074072i    -0.060129 -   0.060582i

-1.4886 +     0.3234i      0.52252 +    0.51057i

11.254 -     7.8223i      -2.8348 -      1.121i

-62.593 +     48.408i       8.8972 -     1.7227i

198.44 -     135.61i      -15.718 +     9.5199i

-314.86 +     177.91i       13.792 -     9.4002i

189.75 -     88.448i