3GPP TSG-RAN WG4 Meeting # 104bis-e		R4-2216414

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.14.2.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on enhancement of UE TRP and TRS

Document for:	Approval

# 1 Introduction

In RAN#97-e, the new Rel-18 WI on enhancement of UE TRP and TRS requirements and test methodologies was approved [1].

The following objectives regarding the enhancements on anechoic-chamber based test methodology are captured in the WID [1]:

- Specify necessary enhancement of the anechoic-chamber based test methodology (i.e. reference test methodology) to support (test methodology defined in TR 38.834 is the baseline): - UE with NR 2Tx configuration - Case 1: TxD (i.e., TxD capability supported) - Case 2: single layer UL-MIMO (i.e., codebook-based capability supported) - Study proper configuration from UE implementation and test system feasibility perspective - Define test case applicability for case 1 and case 2 - RedCap devices (focused on wearable device only) - Wrist-worn device as 1st priority - Considering UEs with antenna configurations of 1 Tx, 1 Rx, 2 Rx - Forearm phantom should be specified - Consider the coordination with CTIA on this aspect - Other phantoms are not precluded - UE with DL/UL carrier aggregation configuration (2nd priority) - Independent measurements of each CC can be the baseline approach - Limited to Inter-band CA with up to 2DL/2UL cases

In this paper, we provide our views on TRP and TRS test methodology enhancements in the anechoic chamber.

# 2 Discussion

### UE with NR 2Tx configuration

For the test methodology of UE with 2Tx configuration, the following two cases will be considered:

- Case 1: TxD (i.e., TxD capability supported)

- Case 2: single layer UL-MIMO (i.e., codebook-based capability supported)

For both case 1 and case 2, UE is transmitting with 2 antennas simultaneously. In Rel-17 NR TRP and TRS discussion, the following two options were discussed which should apply for both two cases:

- Option 1: Measure the TRP per antenna separately and sum the TRP up

- Option 2: Measure the TRP with two antennas transmitting together

The pros and cons of option 1 and option 2 are analyzed in Table 1.

Table 1: Pros and Cons for two options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | Feasible to test the TPR per branch. | Have to use test mode which could not apply for the legacy UE and increase the implementation complexity. It is not the real UE behaviour for TxD or single layer UL-MIMO. |
| Option 2 | It is the real UE behaviour for TxD or single layer UL-MIMO. | It is not clear what’s the impact of phase difference between two antennas on the TRP measurement. |

For option 1, considering the test mode will limit the applicability of the test method, e.g., test mode introducing for TxD or single layer UL-MIMO in Rel-18 would not apply for the Rel-16/17 UE and it will increase the implementation complexity for the test, we have the following observation and proposal:

Observation 1: Test mode should be avoided for the TRP test method of UE with 2Tx configuration.

Proposal 1: RAN4 to consider the option 2 as the starting point and study the impact of phase difference between two antennas when measuring the TRP.

### RedCap devices

As stated in [1], only wearable device will be considered in this WI. For the wrist-worm device, the antenna can be 1Tx, 1Rx or 2Rx. While from test methodology point of view, the legacy TRP/TRS test method and test system for smartphone should be able to apply for RedCap devices as well since the size of RedCap devices would be smaller than smartphone. In this case, the only difference is the forearm phantom which needs to be coordinated with CTIA.

Proposal 2: The legacy TRP/TRS test method and test system for smartphone can be reused for RedCap devices. The forearm phantom needs to be coordinated with CTIA.

### UE with DL/UL carrier aggregation configuration

For the test methodology for UE with 2DL/2UL CA configuration, the most straightforward way is to reuse the test configurations and test methodology of EN-DC.

- For TRP measurement, configure NR carrier 1 as a constant power of 10dBm, while measuring NR carrier 2 at maximum output power, and vice versa.

- For TRS measurement, the UL power configuration for NR carrier 1 and NR carrier 2 is 50%-50% power splitting and measure the TRP for each NR carrier.

Proposal 3: To reuse the EN-DC test configurations and test methodology for DL/UL CA configuration.

- For TRP measurement, configure NR carrier 1 as a constant power of 10dBm, while measuring NR carrier 2 at maximum output power, and vice versa.

- For TRS measurement, the UL power configuration for NR carrier 1 and NR carrier 2 is 50%-50% power splitting and measure the TRP for each NR carrier.

# 3 Conclusions

This contribution provided our considerations on the enhancement of UE TRP and TRS. The following observation and proposals are made:

Observation 1: Test mode should be avoided for the TRP test method of UE with 2Tx configuration.

Proposal 1: RAN4 to consider the option 2 as the starting point and study the impact of phase difference between two antennas when measuring the TRP.

Proposal 2: The legacy TRP/TRS test method and test system for smartphone can be reused for RedCap devices. The forearm phantom needs to be coordinated with CTIA.

Proposal 3: To reuse the EN-DC test configurations and test methodology for DL/UL CA configuration.

- For TRP measurement, configure NR carrier 1 as a constant power of 10dBm, while measuring NR carrier 2 at maximum output power, and vice versa.

- For TRS measurement, the UL power configuration for NR carrier 1 and NR carrier 2 is 50%-50% power splitting and measure the TRP for each NR carrier.

# 4 References

RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies”, vivo